# 3GPP TSG-RAN WG4 Meeting #115                             R4-2506601

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.18.6

Source: 	CMCC

Title:        Discussion on RRM performance requirements for fast SCell activation for UE  supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN #103 meeting, the WID on RRM enhancements was approved [1]. According to the WID, one of the objectives is fast SCell activation for UE supporting Rel-18 EMR, the details are duplicated as following.

- Objective of core part - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024 - Objective of performance part - Specify RRM performance requirements and test cases for the Core part enhancements.

This contribution provides discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR.

- Discussion

According to existing requirements, upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n + ((THARQ + Tactivation_time +TCSI_Reporting)/ NR slot length). Tactivation_time is the SCell activation delay including delay of AGC, PSS/SSS detection, synchronization. Exisiting SCell activation delay for unknown case is very long. Taking FR1 as example, for unknown SCell, Tactivation_time is 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting. Similar situation exists for FR2.

The motivation for the objective on fast SCell activation for UE supporting Rel-18 EMR is to reduce the unknown SCell activation delay for the case that UE send valid EMR report during a period before the reception of the SCell activation command. The agreements in core part discussion are duplicated as following. It can be seen that compared with legacy requirements, SCell activation delay deactivated SCell based on measurement in IDLE/INACTIVE mode can be reduced a lot. It is necessary to define test cases to verify the performance.

Agreements for fast SCell activation delay based on measurement in IDLE/INACTIVE mode: If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met: - During the period equal to 5s for FR1 before the reception of the SCell activation command: - the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3. - the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting) - If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission. - If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter. - the SSB measured remains detectable according to the IDLE/INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. If the SCell belongs to FR2, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, and if the following conditions are met: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3. - the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e. a valid CQI reporting) - If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission. - If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter. - the SSB measured remains detectable according to the IDLE/ INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode. Then Tactivation_time is: If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

Proposal 1: it is proposed to define test cases for fast SCell activation for UE supporting Rel-18 EMR.

For the scope of fast SCell activation, in addition to Direct SCell Activation at SCell addition, it is alo supported for SCell Activation Delay Requirement for Deactivated PUCCH SCell. From test coverage point of view, it is proposed to define test cases for following cases.

Proposal 2: for fast SCell activation, it is proposed to define test cases for following scenarios

- Direct SCell Activation at SCell addition

- SCell Activation Delay Requirement for Deactivated PUCCH SCell

According to the core part discussion, following cases are considered to apply the fast SCell activation delay requirements based on measurement in in IDLE/INACTIVE mode. Since different cases target for different UE capability and/or network configuration, it is proposed to define test cases for following cases.

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

Proposal 3: for fast SCell activation, it is proposed to define test cases for following cases

- UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- Conclusion

This contribution provides discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR. The proposals are:

Proposal 1: it is proposed to define test cases for fast SCell activation for UE supporting Rel-18 EMR.

Proposal 2: for fast SCell activation, it is proposed to define test cases for following scenarios

- Direct SCell Activation at SCell addition

- SCell Activation Delay Requirement for Deactivated PUCCH SCell

Proposal 3: for fast SCell activation, it is proposed to define test cases for following cases

- UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2504900, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple

- R4-2416853, WF for NR_RRM_Ph5_Part1, Apple

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple

- RP-240817, Moderator's summary for discussion on RRM enhancements