3GPP TSG-RAN WG4 Meeting #115	   R4-2506670

# St.Julian’s, Malta, 12th – 17th , May , 2025

Agenda Item:	7.29.3.3

Source: 	Ericsson

Title:	On general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September 2024, the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For RAN4#114 bis meeting, many open issues are listed in the WF[2]. In this contribution, we will address several aspects which are related to RRM requirements for conditional LTM.

# Discussion

## General aspects and early synchronization

Certain agreements have been reached during the RAN4#114 bis meeting regarding the Rel-19 CLTM scope. One remaining issue is regarding which requirement to be specified for Rel-19 L1 measurement requirement for deactivated Scell or UE based TA estimation.

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement

RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them. RAN4 #114bis < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

The CLTM candidate can be configured on the deactivated SCC based on Rel-18 LTM framework. From the RRC specification TS 38.331, allows the network to configure a deactivated SCC as an SpCell within the CellGroupConfig IE, and then afterwards as an LTM candidate cell in the ltm-CandidateConfig. Following this configuration, the UE is required to perform L1 measurements for the LTM candidate evaluation after receiving the RRC reconfiguration for the LTM candidate cell configuration.

For candidate measurement evaluation, when this frequency being configured to the UE, it is not for Scell, it is for the Spcell candidate for LTM/CLTM purpose. The consequence of no L1 measurement requirement on the deactivated SCC will lead to certain frequency will not be evaluated as CLTM/LTM candidate cells, which limits the network and UE choice for candidate cell selection.

The reason of L1 measurement requirement is not specified in RAN4 specification is tied to TS 38.321 clause 5.9.

The MAC entity shall for each configured SCell:​ 1> if the SCell is deactivated:​ 2> …​ 2> not report CSI for the SCell;​ 2> not transmit on RACH on the SCell;

When UE is not required to report CSI for the specific Scell does not mean UE shall stop measure the deactivated component carrier. There is plenty of measurement that UE perform while not triggering or sending measurement report. This cannot be an appropriate reason for not have the measurement requirement.

Also, this procedure is related to the UE behavior when the Scell is being deactivated, not when UE is being configured with a component carrier that later will be LTM candidate.

- Observation 1: Network may configure deactivated SCC as a spCell and as a LTM candidate for evaluation based on Rel-18 LTM procedure.

- Proposal 1: Define requirements for L1 measurement on deactivated Scell.

Issue 1-2-1: Early UL Synchronization

RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) Do not define RRM requirements for UE based TA measurement. (Apple, MTK) RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.

Besides the PDCCH ordered early TA acquisition, the Rel-18 LTM already supports UE based TA measurement, the issue remains for Rel-19 CLTM is whether to define requirements considering the UE based TA estimation.

We acknowledge a valid TA before the cell switch happens will benefit the performance especially for the RACH-less case, however seting requirements for UE based TA measurement have certain issue in practice.

Firstly, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. Currently there are proposals on how exactly the TA measurement can be performed, still the detail accuracy will be based on how UE implement the measurement.

UE need to estimate the TA value for the target cell based on current serving cell TA value. If sync between serving and target is ideal, the TAtarget =TAserving+2* (Propagation delaytarget-Propagation delayserving)

For the UE estimated TA value error to be acceptable, the error of the TA estimation shall be within half CP length. Assuming the sampling time Tc = 0.509 ns, the TA estimation error from UE point of view shall be considering:

- UL to DL reference timing error Te

- TA resolution error:  which is TA adjustment step size16*64/2u

- TA adjustment accuracy: which defined in TS 38.133 7.3.2.2-1

Considering the Synchronization error between target and serving cell, by comparing half CP length with total error Te+ TA resolution error+ TA adjustment accuracy, we can see how much margin left for Sync error between the target and serving from table 1.

Sync error margin=Half CP length-UL to DL error Te-TA resolution error-TA ajustment accuracy

| SCS of UL siganl (KHz) | Half CP length (us) | UL to DL error Te | TA resolution error | TA adjustment accuracy | Synchronization error margin (us) |
| --- | --- | --- | --- | --- | --- |
| 15 | 2.345 | 12*64*Tc | 16*64*Tc | 256 Tc | 1.303 |
| 30 | 1.15 | 8*64*Tc | 8*64*Tc | 256 Tc | 0.498 |
| 60 | 0.585 | 3.5*64*Tc | 3.5*64*Tc | 32 Tc | 0.34 |

Table 1: Sync error margin between target and serving

Considering the existing sync requirement for the network specified in TS 38.104 for TAE values for intra-band non-contiguous CA is 3 µs, comparing the margin from table 1 with 2*TAE 6us, the margin is too short.  We don’t see proper and accurate UE estimated TA value can be achieved with existing network Synchronization requirement.

- Observation 2: Existing sync requirement between gNB will not be able to guarantee the TA estimation accuracy.

Moreover, existing sync requirements are fundamental for compatibility of existing radio base station site infrastructure and vital for operator flexibility of deployment using existing sites.

- Observation 3: existing sync requirements are fundamental for compatibility of existing radio base station site infrastructure and vital for operator flexibility of deployment using existing sites.

Based on current RAN2 signaling design, the network may also send a TA value in a separate MAC CE. When network indicated and UE estimated TA value both exist, the UE shall follow the value based on network ordered TA acquisition. The UE based TA estimation is only for the case that the network has not indicate TA value.

Since how UE measures the TA value is up to UE implementation, also the accuracy still within concern, we don’t see the motivation to set requirements on this matter.

- Proposal 2: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

## Conditional LTM delay

Issue 1-2-3: The components of conditional LTM delay < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not. Issue 1-2-14: Twait_RS < Way forward > FFS the following proposal: - Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

For Rel-18 LTM, the fine time tracking sample and the sample processing time is included in the interruption time. Due to the reason that UE need to acquire full timing information to another frequency, this can causing interruption for the source cell. When the UE has been configured with intra frequency measurement for the target cell and the target cell TCI state is within the active TCI state, there is no need for one extra sample to acquire full time information.

The difference between CLTM and the LTM is that the network will have no information when UE will execute the cell switch to the target cell. What network can do is at best effort to have the TCI activated and TA value delivered to the UE before UE have decided to switch to the target cell. Based on current RAN2 agreements our understanding is that a MAC CE for delivery of the TA value will be introduced.

The legacy LTM requirement is based on a side condition whether the time gap between TCI state activation MAC CE and the cell switch MAC CE command is longer than 160ms. When this time gap side condition or configured measurement period is longer than 160ms, the UE will still need to perform fine time tracking. However, for CLTM, there will be no cell switch command from the network, hence the time gap side condition will not be applicable for CLTM.

- Observation 3: The LTM side condition for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can not be fully reused.

Whether UE needs to perform fine time tracking have direct relation to whether UE have been perform measurement on the target cell and already have the full time information of the target cell.

Based on the current agreement that the Tmeasure will be defined based on different type of measurement UE have been performed, we think similar logic can be reused here.

If the UE have been configured L1 measurement for the target cell, the same value Tfirst-RS = 0 and TRS-proc= 0 can be reused from LTM due to the UE already have the target cell full timing information. This is also one of the legacy side condition for LTM.

If the UE have been perform L3 intra-frequency measurement in FR1 and FR2, it is reasonable to assume that the UE have full time information. Even though at FR2, the UE may use different RX beam towards the source and the target, the timing information should not have been impact by this.

If UE have been perform L3 inter-frequency measurement, the UE will need extra time to acquire timing information.

- Proposal 3: Tfirst-RS + TRS-proc value can be specified based on the measurement UE have been performed after the TCI activation.

- Proposal 4: Tfirst-RS = 0 and TRS-proc= 0 under the condition that : TCI state of the target cell is within the active TCI state list. The UE is configured with L1 intra/inter frequency measurement for the target cell for both FR1 and FR2. The UE is configured with L3 intra frequency measurement for the target cell for FR1 and FR2.

- Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption +   where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU +Tfirst-RS + TRS-procer

## Subsequent Conditional LTM delay

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay < Way forward > FFS the following options: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. - Proposal 1b: TEvent_DU needs to be updated. - Proposal 1c: Apart from the starting point of the delay. - Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell.

For the subsequent CLTM, we think the RRC delay will not be needed due to the fact that the subsequent CLTM have the RRC configuration for all the potential candidate and execution condition to the UE and it is based on UE evaluation to decide whether trigger execution on another LTM candidate. The RRC delay time shall not be double counted.

- Proposal 6: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

- Proposal 7: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption , where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU +Tfirst-RS + TRS-procer.

# Summary

The following have been observed in the current contribution:

- Observation 1: Network may configure deactivated SCC as a spCell and as a LTM candidate for evaluation based on Rel-18 LTM procedure.

- Observation 2: Existing sync requirement between gNB will not be able to guarantee the TA estimation accuracy.

- Observation 3: existing sync requirements are fundamental for compatibility of existing radio base station site infrastructure and vital for operator flexibility of deployment using existing sites.

The following have been proposed in the current contribution:

	Proposal 1: Define requirements for L1 measurement on deactivated Scell.

	Proposal 2: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

	Proposal 3: Tfirst-RS + TRS-proc value can be specified based on the measurement UE have been performed after the TCI activation.

	Proposal 4: Tfirst-RS = 0 and TRS-proc= 0 under the condition that : TCI state of the target cell is within the active TCI state list. The UE is configured with L1 intra/inter frequency measurement for the target cell for both FR1 and FR2. The UE is configured with L3 intra frequency measurement for the target cell for FR1 and FR2.

	Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interuption +   where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU +Tfirst-RS + TRS-procer

	Proposal 6: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

	Proposal 7: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption , where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU +Tfirst-RS + TRS-procer.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.

- R4-2504910 “WF on RRM requirements for NR_Mob_Ph4_Part2”, China Telecom, April, 2025