3GPP TSG-RAN WG4 Meeting #115	R4-2506711

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.24.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2]. RAN4 discussed the RRM impacts with agreements and issues captured in [3]. In this contribution, we further provide our views on the remaining issues on UE initiated BM.

# Discussion

Based on the discussion in last meeting, the agreements reached are summarized as follows [3]:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

For measurement and evaluation period, most of the components of the delay requirements are agreed. One remaining issue is when timeRestrictionForChannelMeasurement is configured. As commented in last meeting, currently there is no restrictions on whether UE needs to measure all three samples and do filtering or UE can choose to measure one sample every three RSs. To have a reliable event determination, quite a lot of details needs to be defined, e.g. filtering assumption for both new beam and current beam, which samples are used for comparation. But considering time left for this WI, we are also fine to have no requirements for this case in Rel-19.

Proposal 1: Only define RRM requirements when timeRestrictionForChannelMeasurement is configured.

Another issue is how to define the measurement delay considering the delay for current beam and new beam could be different. The status is summarized as follows:

#### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: - The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

Based on the discussion in last meeting, we think option 2 is not that clear from specification perspective. In other words, which measurement triggers the event highly depends on UE implementations and specific cases. We give an example in following figure. For the left fig, the conditions is met over the air at point A. Then, which measurement is the one that trigger the Event? N2 is the first measurement after A, but the previous measurement of current beam is C1, and the offset between N2 and C1 may be still less than the threshold, then UE can only trigger the event after getting the results at C2. Another example is given in the right part. At N2, it is possible that UE can determine the event is triggered by N2 and C1, however, it is also possible that UE’s implementation is to do the comparation as (N1, C1) and (N2, C2), which means UE can only determine the that the event is triggered at C2.

Fig.1 Event2 determination

Observation 1: For different cases (i.e. RSRP change of current beam and new beam), and different UE implementations, it is not clear which measurement will trigger the event.

Observation 2: To define minimum requirements, the measurement delay shall be defined that there is at least one measurement for new beam and one measurement for current beam after the conditions are met over the air.

Based on observation 1 and 2, we proposed to define the delay requirements as option 1.

Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

Another issue is whether to define window-based requirements. RAN4 hasn’t have detailed discussion for this in previous meetings. One reason is that RAN1 is still discussing the detailed design. Considerable progress has been achieved in last RAN1 meeting, and the relevant agreements are shown as follows:

Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications.

From our understanding, there is not much RAN4 efforts on considering this. From UE measurement behaviour/requirements, there is no difference compared with no-time window based. The only additional part is how to determine number of event instance. However, the impact on RAN4 is how to define the evaluation behaviour/requirements, which is also common for non-time window based.

Besides, we think time window-based triggering is also important for this feature, different from L3 event triggering with filtering, L1-RSRP could be impacted by instance channel fading, which may lead to wrong event triggering, and it may mislead NW following action (e.g. wrong TCI state switching).

Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Based on RAN1 agreements in last meeting, UE will start the timer once an event-2 instance is obtained. From RRM requirements perspective, when the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter.

Proposal 4: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter.

For time windowed based case, another issue may need to be discussed is how to count one event instance based on the measurement. Considering that new beam and current beam have same periodicity, similar as no-time-window based case, there is always “pair” of new results from both new beam and current beam. Considering following cases as shown in the figure. For the left part, if there is instance fading happens at point A for current beam, it will only create on event instance, UE will not trigger event to NW, However, if UE only perform measurement on new beam and using history results of current beam, one bad result due to instance change fading will lead to multiple event instance and triggers wrong event to NW work. Another example if as shown in the right part, different comparation will leads to different results of number counting.

Fig.1 Instance counting using different results

Proposal 5: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Another issue is about CSI-RS based L1-RSRP for inter-Cell BM, the status is summarized as follows:

#### Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management - Option 2: - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

From our understanding, the inter-cell/intra-cell is the definition only differentiated w.r.t SSB since UE needs detect SSB with same SSB index from different TRP/Cell with different PCI. However, for CSI-RS there is no such differentiations. Different CSI-RS resources from different TRP/Cells can QCLed to SSB with different PCI.

Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Regarding the two options, technically we think option 2 is reasonable. But regarding the spec impacts, our interpretation is not to have dedicated sections for inter-cell CSI-RS measurement. Instead, CSI-RS based L1-RSRP measurement requirements for UEIBR applies for both intra-cell and inter-cell without restriction/limitation.

Proposal 6: CSI-RS based L1-RSRP measurement requirements for UEIBR are defined which applies to both intra-cell and inter-cell without restriction/limitation.

There are also issues related to TCI state switching, with the status captured as follows:

#### Issue 1-6: Whether to specify new unified TCI switching delay requirements? <Way forward > - For MAC-CE based DL/UL TCI state switch delay - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: (ZTE) - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc - Option 3: (MediaTek) - FFS on whether to remove Tfirst-SSB+TSSB-proc - For DCI based DL/UL TCI state switch delay - Option 1: - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: - Whether to further reduce the delay, wait for further RAN1 progress

From our understanding, there is no impacts on TCI state switching requirements. There is no further agreements in Ran1 about the TCI state switching enhancement so far.

Proposal 7: For UEIBR, No RRM impacts on TCI state switching requirements.

Another issue is about whether to consider event triggering and reporting criteria.

#### Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. - Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

From our understanding, the existing requirements for L3 is implicitly restrict the number of measID could be configured to UE. The MO configuration and measID configuration is quiet flexible from gNB perspective. However, for UEIBR, how many events that UE is required to evaluate needs to considering following two aspects: 1) number of events (event 1/2/7); 2) number of new beams. Based on latest RAN1 feature list, the supported event and supported number of new beams can be explicitly indicated by UE capabilities. Thus, there is no needs to further define such capabilities in RAN4 which may further confuse gNB configurations.

Observation 5: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event.

Proposal 8: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria.

# Conclusions

Proposal 1: Only define RRM requirements when timeRestrictionForChannelMeasurement is configured.

Observation 1: For different cases (i.e. RSRP change of current beam and new beam), and different UE implementations, it is not clear which measurement will trigger the event.

Observation 2: To define minimum requirements, the measurement delay shall be defined that there is at least one measurement for new beam and one measurement for current beam after the conditions are met over the air.

Based on observation 1 and 2, we proposed to define the delay requirements as option 1.

Proposal 2: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

Observation 3: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 3: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Proposal 4: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter.

Proposal 5: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Observation 4: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 6: CSI-RS based L1-RSRP measurement requirements for UEIBR are defined which applies to both intra-cell and inter-cell without restriction/limitation.

Proposal 7: For UEIBR, No RRM impacts on TCI state switching requirements.

Observation 5: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event.

Proposal 8: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2504902 WF on RRM requirements for NR_MIMO_Ph5_Part1