3GPP TSG-RAN4 Meeting #115	R4-2506979

# Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.1.2.1.2 & 7.1.2.1.3

Source:	MediaTek

Title:	Discussion on HPUE intra-band/inter-band CA/DC with 2Tx/3Tx

Document for:	Approval

# Introduction

In RAN4#114bis meeting, the WF[1] has been approved and recommended to further discuss and encourage more companies’ input for each open issues i.e., MPR for intra-band contiguous/non-contiguous CA UE and so on for HP CA/DC UE.

Way forward [1]: ## 1.2 MPR values for intra-band contiguous ULCA with non-contiguous RB allocation (with TxD) - The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference Modulation; PC3 (1Tx); PC2 (2Tx); PC1.5 (2Tx). Handheld; Handheld; Handheld. inner; Outer12; Outer23; inner; Outer12; Outer23; inner; Outer12; Outer23. DFT-s-OFDM; PI/2 BPSK; 2.5; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. QPSK; 2.5; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. 16QAM; 3; 6; 11.5; 41; 8.5; 14.5; [4.5-5]; [9-9.5]; [15.5-17]. 64QAM; 5; 6; 11.5; 6; 8.5; 14.5; [6.5-7]; [9-9.5]; [15.5-17]. 256QAM; 6.5; 6.5; 11.5; 7.5; 8.5; 14.5; [8-8.5]; [9-9.5]; [15.5-17]. CP-OFDM; QPSK; 3.5; 7; 12; 4.51; 9; 15.5; [5-5.5]; [9.5-10]; [16.5-17]. 16QAM; 3.5; 7; 12; 4.51; 9; 15.5; [5-5.5]; [9.5-10]; [16.5-17]. 64QAM; 5; 7; 12; 6; 9; 15.5; [6.5-7]; [9.5-10]; [16.5-17]. 256QAM; 7.5; 7.5; 12; 8.5; 9; 15.5; [9-9.5]; [9.5-10]; [16.5-17]. NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported. Agreement: - Further discussion in upcoming meetings based on the tentative values in the above table. - The MPR contributions from companies is captured in the spreadsheet below: ## 1.3 MPR values for intra-band non-contiguous ULCA (with dualPA) Agreement: - Further discussion in upcoming meetings ## 1.4 A-MPR applicability for intra-band contiguous ULCA with contiguous RB allocation Qualcomm proposal: For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR For all modulations and SCS when Fedge, low - BWChannel_CA < 2490.5 MHz if the RB allocation is an inner allocation as defined in clause 6.2A.2.1, then A-MPR = MPR Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc). if the RB allocation is an outer allocation as defined in clause 6.2A.2.1, then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C for PC3 and PC2 and A-MPR=MPR+1.5dB for BW class C for PC1.5 UE handheld devices and A-MPR=MPR+1.5dB for BW class C for PC1.5 FWA devices. Where - MPR is the MPR as defined in Table 6.2A.2.1-1, Table 6.2A.2.1-1a and Table 6.2A.2.1-1b for PC3 and PC2 respectively and the respective CA bandwidth class - AMPRcc is defined as the PC3_A2, PC2_A4 or PC1.5_A6 AMPR in Table 6.2.3.2-2 for PC3, PC2 and PC1.5 respectively. Agreement: - FFS in next meeting ## 1.5 A-MPR for intra-band non-contiguous ULCA with non-contiguous RB allocation Agreement: - Further discussion in upcoming meetings ## 1.6 A-MPR values for intra-band non-contiguous ULCA (with dualPA) Agreement: - Further discussion in upcoming meetings

In this paper, we share our view on the HP CA/DC UE related to MPR requirements in Rel-19.

# MPR for intra-band contiguous UL CA with non-contiguous RB allocation (with TxD)

The following PC1.5 MPR values for intra-band contiguous UL CA UE with non-contiguous RB allocation are proposed with high-lighted yellow color.

Table 1. MPR values for intra-band contiguous ULCA with non-contiguous RB allocation

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5.0] | [9.5] | [16.5] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5.0] | [9.5] | [16.5] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [5.0] | [9.5] | [16.5] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [7.0] | [9.5] | [16.5] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8.5] | [9.5] | [17] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5.5] | [10] | [17] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5.5] | [10] | [17] |
|  | 64QAM | 5 | 7 | 12 | 6 | 9 | 15.5 | [7.0] | [10] | [17] |
|  | 256QAM | 7.5 | 7.5 | 12 | 8.5 | 9 | 15.5 | [9.5] | [10] | [17] |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |  |  |  |

Proposal 1: RAN4 can consider the above MPR requirements of PC1.5 intra-band contiguous CA UE with non-contiguous RB allocation.

# MPR for intra-band non-contiguous CA UE

For the MPR requirements of PC1.5 intra-band non-contiguous CA UE, RAN4 can estimate the delta MPR values between the MPR requirements of PC2 single carrier with 2Tx UE and the MPR requirements of PC1.5 single carrier with 2Tx UE. For the Edge RB allocations cases, the delta MPR values is 2.5dB~3dB for both DFT-S-OFDM and CP-OFDM waveforms since the MPR simulation assumptions of the dual PA configurations, ACLR, SEM and other parameters are same for those MPR evaluation. Only difference point is 3dB power offset between PC2 and PC1.5 MPR.

TS38.101-1 MPR requirements for PC2 and PC1.5 of single carrier (2Tx)

Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

Table 6.2D.2-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Based on the observation, the following MPR requirements for PC2 with dualPA-architecture to meet the -30dBm/MHz can be adjusted with delta MPR value. In other word, based on the PC2 MPR requirements, RAN4 can estimate the PC1.5 MPR requirements with dualPA-architecture UE

TS38.101-1 MPR requirements for PC2 for intra-band NC-CA UE with -30dBm/MHz emission (2Tx)

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    	2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 	23.04 ≤ B

=========================== TS38.101-1 Clause 6.2A.2.2.1 ===============================

##### 6.2A.2.2.1.5	PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5 + [2.5~3.0]; 	0 ≤ B < 1.44

15.0 + [2.5~3.0]; 	1.44 ≤ B < 2.88

14.0 + [2.5~3.0];   	2.88 ≤ B < 5.76

12.0 + [2.5~3.0]; 	5.76 ≤ B < 10.8

10.5 + [2.5~3.0]; 	10.8 ≤ B < 23.04

9.0 + [2.5~3.0]; 	23.04 ≤ B

##### 6.2A.2.2.1.6	PC1.5 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	19.5 + [2.5~3.0]; 	0 ≤ B < 1.08

19.0 + [2.5~3.0]; 	1.08 ≤ B < 2.16

18.0 + [2.5~3.0]; 	2.16 ≤ B < 5.04

16.5 + [2.5~3.0]; 	5.04≤ B < 10.08

16.0 + [2.5~3.0]; 	10.08 ≤ B < 36

12.0 + [2.5~3.0];  	36 ≤ B < 56.88

10.5 + [2.5~3.0]; 	56.88 ≤ B

=============================================================================

Proposal 2: To derive MPR requirements for PC1.5 intra-band NC CA UE with dualPA-Architecture, RAN4 consider 2.5~3dB delta MPR value from MPR of PC2 intra-band NC CA UE with dualPA-Architecture.

# Re-organization 2Tx/3Tx Note in Clause 5 and 6

For the 3Tx Notes in clause 6 with suffix H (CA+UL-MIMO) and suffix L (CA+Tx Diversity), RAN4 can move detail device type information (FWA/CPE/other device type) or detail Tx/Rx antenna configuration in clause 4 general part to remove detail notes in clause 5 and 6.

Proposal 3: RAN4 add the relevant information of the target device types (FWA/CPE/other device type for 3Tx) and detail Tx Chain configurations can be described into the general part of clause 4 to remove some sprinkled notes in clause 5 & 6.

# Conclusion

In this contribution, we share our view how to define the Pcmax,c and MPR requirements for intra-band non-contiguous CA UE (with Dual PA) and SAR compliance solution for HP inter-band CA with 3Tx UE and so on.

Based on our observation and analysis, we can propose as follow

Proposal 1: RAN4 can consider the above MPR requirements of PC1.5 intra-band contiguous CA UE with non-contiguous RB allocation.

Proposal 2: To derive MPR requirements for PC1.5 intra-band NC CA UE with dualPA-Architecture, RAN4 consider 2.5~3dB delta MPR value from MPR of PC2 intra-band NC CA UE with dualPA-Architecture.

Proposal 3: RAN4 add the relevant information of the target device types (FWA/CPE/other device type for 3Tx) and detail Tx Chain configurations can be described into the general part of clause 4 to remove some sprinkled notes in clause 5 & 6.

## References

- R4-2505100, WF on HPUE RF requirements, Qualcomm, MediaTek, Skyworks, LGE, RAN4 #114bis

- R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113

- R4-2419699, 1UL HPUE NR-CA MSD Simplifications for Rel-19, Skyworks Solutions, Inc. RAN4#113.

- R4-2502865, Draft CR on duty cycle solution for PC1.5 of CA, vivo, LGE, China Telecom, Huawei, Hisilicon #114.

- R4-2502865, Draft CR on duty cycle solution for ENDC, vivo, LGE, China Telecom, Huawei, Hisilicon #114.