3GPP TSG-RAN WG4 Meeting #115	R4-2506998

Malta, Malta, 19th – 23th May, 2025

Agenda item:	7.1.2.3.4

Source:            	Google

Title:                	Discussion on SRS IL imbalance for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the SRS IL (insertion loss) imbalance for 6Rx handheld and FWA UEs.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#114bis, the SRS IL imbalance for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the approved WF [2]. In WF, whether to solve SRS IL imbalance issue has been concluded no further discussion in the future meeting, but it is still not clear whether to introduce the solution for SRS IL imbalance issue in Rel-19. In this paper, we would like to share our view in the following.

# Topic #4: SRS IL imbalance ### Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: - No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions. - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19 Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx Agreement - FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs Issue 4-1-3: SRS IL imbalance solution as optional feature Agreement - If any new requirements or procedures for SRS IL handling are defined in Rel-19, they will be defined as optional UE features ### Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

# Discussion

In Rel-15, SRS Tx switching feature is introduced to obtain more accurate channel state information (CSI) by utilizing channel reciprocity for TDD bands. The motivation to introduce SRS Tx switching feature is that Tx antenna port number is usually less than Rx antenna port number from UE implementation perspective. In other words, in order to obtain more accurate CSI for DL-MIMO transmission at the base station side, the UE can be scheduled to perform antenna switching for SRS transmission in the corresponding Rx antenna port sequentially instead of simultaneous SRS transmission on the corresponding Rx ports.

From UE capability signalling perspective, SRS Tx switching feature can be indicated by the corresponding IE srs-TxSwitch/srs-TxSwitch-v1610 with “xTyR”, e.g., 1T2R/1T4R/ 2T4R/2T2R/4T4R/1T4R-2T4R, where “x” is the capable antenna port for SRS transmission and “y” means the total number of receive antenna where SRS transmission can be performed for the target band or the target band in the band combination. In Rel-17, the xTyR has been extended to support at most “1T8R/2T8R/4T8R” in the corresponding IE srs-AntennaSwitchingBeyond4Rx-r17.

From implementation perspective, if the UE is configured to perform SRS antenna switching, SRS Tx transmission may go through different number of RFFE antenna switch components, e.g., SPXT, DPXT where X=2,3,4,…8, at each associated Rx path on PCB board, which may cause different insertion loss for each SRS Tx path. In addition, the PCB tracing loss may also be different for each SRS Tx associated Rx path. Hence, SRS Tx power among each associated Rx paths would be imbalanced due to RFFE insertion loss and PCB tracing loss, which may cause SRS IL imbalance when the UE performs SRS antenna switching.

In RAN4#114, the SRS IL imbalance issue for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the WF. In WF, the first open issue whether to solve SRS-IL imbalance issue in Rel-19 is still not determined. In our understanding, SRS-IL imbalance issue for SRS Tx power imbalance can be analyzed from two perspective, where the first is from conducted RF antenna connector perspective and the second is from OTA perspective.

From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. In addition, the cost for factory RF calibration would be minor that the UE may need to save all calibration data in the non-volatile (NV) memory. In our understanding, for all SRS Tx power level after well RF calibration, SRS-IL imbalance among each SRS Tx associated Rx paths would be only around 1~2 dB. Hence, SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. Hence, the SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed due to more protection on main Tx/PRx. In addition, if the UE is well calibrated, SRS Tx power imbalance may still often be occurred in the real live network. Which means that the received SRS Tx power at the network side may be varying very fast due to the different channel condition. In our view, even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. Even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

In RAN4#114bis, two general solutions for the SRS-IL imbalance issue on 6Rx handheld and FWA UE are captured in the WF. The first general solution is to introduce UE capability indication on whether UE perform SRS IL self-compensation. Where the self-compensation means SRS IL would be compensated up to its configured maximum output power for SRS transmission. The second solution is to extend Type 3 SRS PHR reporting to with PUSCH subject to RAN1 confirmation on feasibility carrier.

In our view, we think we should firstly identify whether SRS-IL imbalance is a critical issue or not. As above observations, from conducted perspective, it would be a minor issue when the device is well factory RF calibrated at each SRS Tx on the associated Rx paths. It is also agreed for RAN4 assumptions in last meeting that Rel-19 6Rx UE is expected to perform self-compensation of SRS-IL up to its configured maximum output power for SRS transmission. From OTA perspective, it would still not be the critical factor to impact SRS Tx power imbalance at the network side because different antenna radiation pattern and/or channel condition may also impact imbalance a lot even if the device SRS Tx is well calibrated. Therefore, since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective. Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance. Hence, it is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19.

Proposal 1: Since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective. Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance. Hence, it is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. Hence, the SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective.

Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. Even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance.

Proposal 1: Since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective. Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance. Hence, it is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2505101, “WF on 6Rx RF requirements and SRS IL imbalance”, Intel, RAN4#114bis

- R4-2504100, “Discussion on SRS IL imbalance for 6Rx UE”, Google, RAN4#114bis