3GPP TSG-RAN WG4 Meeting #115				R4-2507015

Saint Julian's, MT, 19th – 23rd May, 2025

Agenda item:	7.29.2

Source: 	Ericsson

Title: 	Rel-19 RAN4 UE feature list for NR mobility enhancements Phase 4

Document for:	Discussion

- Introduction

This contribution includes the RAN4 UE feature list for enhancements of NR mobility enhancements Phase 4 in Rel-19. In the below table we propose to define the following capabilities based on RAN4 previous agreements for the work item.

We have agreed to support skipping SSB based L1-RSRP while performing CSI-RS based L1-RSRP. Agreement is listed below

- Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 | Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement. The UE, indicating this capability, supports RX beam sweeping using CSI-RS resources while performing CSI-RS-based L1 RSRP measurements on the neighboring cell, thereby skipping the SSB-based L1-RSRP measurement to acquire a fine RX beam for L1-RSRP computation. | 2-21, 2-22 or 2-23, 2-23a | yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per Band | NO | Yes, only for FR2 | NO |  | Optional with capability signalling |