3GPP TSG-RAN WG4 Meeting #115	R4-2507101

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.3.5

Source:	Ericsson

Title:	UE demodulation requirements for CA including 3MHz CBW

Document for:	Discussion

# 1	Introduction

RAN4#114bis approved the way forward on NR less than 5MHz CA [1]. In the WI, one of the open issues is whether to define PDSCH demodulation requirements for 3MHz CC in the CA demodulation requirements as follows.

Whether to define PDSCH CA demodulation requirements for 3MHz CHBW - Candidate options: - Option 1: Define 3MHz channel bandwidth single carrier requirements in normal PDSCH CA demodulation requirements - Option 2: Do not introduce new PDSCH requirements for <5MHz channels - Way Forward: - Keep open and this issue will be concluded in the next meeting. - In parallel with this discussion, companies are encouraged to bring simulation results in the next meeting.

This contribution discusses the open issues on the UE demodulation performance requirements for CA including 3MHz CBW.

# 2	Simulation results

This section shows the simulation results of PDSCH with 3MHz CBW according to the WF. We have reused the parameters of the existing CA PDSCH demodulation requirements in TS 38.101-4 5.2A.2.1 for 2Rx and 5.2A.3.1 for 4Rx. Table 1 summarizes the ideal simulation results.

Table 1	Simulation assumption for PDSCH demodulation requirements for 3MHz CBW in CA.

| Test number | Duplex/SCS | Bandwidth (MHz) | MCS | TBS (bits) | Propagation condition | Antenna configuration | SNR (dB) to achieve 70% of peak rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz | 3 (15 PRBs) | 16QAM, 0.48 (MCS13) | 7552 | TDLA30-10 | 2x2 ULA Low | 10.2 |
| 2 | FDD 15kHz | 3 (15 PRBs) | 16QAM, 0.48 (MCS13) | 7552 | TDLA30-10 | 2x4 ULA Low | 5.3 |

# 3	Discussion

## 3.1	CA PDSCH demodulation requirements

TS 38.101-4 defines PDSCH CA demodulation requirements to cover all the possible CA combination scenario supported in the UE RF requirements specifications TS 38.101-1/2/3. The CA demodulation requirements specify the PDSCH demodulation requirements per component carrier (CC) because of the assumption that UE receives PDSCH for each CC in CA/DC. For example, for FR1 FDD SCS=15kHz CC, PDSCH demodulation requirements are specified with 5, 10, 15, 20, 25, 30, 40 and 50MHz in Rel-15. In Rel-17, RAN4 added PDSCH demodulation requirements for 35MHz/45MHz CC in CA demodulation requirements because of the WI to introduce new channel bandwidths 35MHz and 45MHz [2].

Since RAN4 RF has defined the CA combination of 3MHz CBW in n100 and 5/10MHz CBW in n101 in TS 38.101-1 V19.1.0 Table 5.5A.3.1-1o, we think RAN4 also needs to ensure the CA PDSCH demodulation performance including 3MHz CC. We should also emphasize it is aligned with the revised WID [3].

Proposal 1:	RAN4 defines the 2Rx/4Rx PDSCH demodulation requirements with 3MHz CBW in FDD SCS=15kHz scenarios as a part of CA demodulation requirements.

As we discussed in RAN4#114bis, the existing CA PDSCH demodulation requirement in TS 38.101-4 Table 5.2-1 configures one PDCCH candidate with AL8 for FDD 15kHz, which requires 48 RBs (= 2 OFDM symbols x 24 PRBs). Since the maximum channel bandwidth of 3MHz CC is 15 RBs, it is not possible to configure AL8 even if 3 OFDM symbols are configured. We therefore propose configuring AL4 for PDCCH with 2 OFDM symbols for 3MHz CC.

Proposal 2:	Configure one PDCCH candidate with AL4 for 3MHz bandwidth CC in Table 5.2A-1.

## 3.2	SDR requirements

Whether to define SDR requirements for 3MHz CHBW - Candidate options: - Option 1: RAN4 to discuss whether there are SDR requirement impact for UE supporting 3MHz channel bandwidth in CA. - Option 2: Do not define new SDR requirements for <5MHz channels.

In TS 38.101-4 5.5A, the SDR requirements are applicable for CA scenario and are also applicable for any CA combination. This means the SDR requirements are also applicable for CA including 3MHz CC.

Observation 1:	The existing SDR requirement specified in TS 38.101-4 5.5A is applicable for any channel bandwidth.

Like the PDSCH CA demodulation requirements, the PDCCH configurations should be adjusted according to 3MHz CBW. Since the SDR requirements configure one OFDM symbol for CORESET, we can schedule PDCCH with up to 2 CCEs in 3MHz FDD 15kHz (15 RBs). We therefore propose configuring 2 PDCCH candidates with AL1 for SDR requirements.

Proposal 3:	Existing SDR requirement specified in TS 38.101-4 5.5A is applicable for CA including 3MHz CC.

Proposal 4:	For the SDR requirements, add the configuration parameters dedicated to 3MHz CC, that is, 2/AL1 for PDCCH in Table 5.5A-1, and 12 RBs for CORESET of SCS=15kHz in Table 5.5A-4.

## 3.3	CQI reporting requirements

Whether to define CA CQI reporting requirements for 3MHz CHBW - Candidate options: - Option 1: The CA CQI test parameters need to be updated for 3MHz CC (Ericsson) - Other options not precluded.

TS 38.101-4 also specifies the CQI reporting requirements in static condition for CA scenario. Since the static CA CQI requirements are applicable for any CA combination, it is applicable for the CA including 3MHz CC. For example, the measurement channel configuration considers any channel bandwidth as shown in Table 6.2A.3.1.1-1.

Observation 2:	The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposal 5:	Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA including 3MHz CC.

Like the PDSCH CA demodulation requirements, the PDCCH configurations should be adjusted according to 3MHz CBW. For example, since the existing CA CQI requirements configure AL8 for PDCCH in TS 38.101-4 Table 6.1.2-1, we need to add the configuration for 3MHz, i.e., AL4. Moreover, the CQI reporting sub-band size in Table 6.2A.3.1.1-2 does not include 3MHz. When the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency granularity according to TS 38.214 5.2.1.4. Since TS 38.214 Table 5.2.1.4-2 does not specify the subband size for 12 PRB, we should not configure subbandSize for 3MHz CC.

Proposal 6:	For CA CQI reporting requirements, add the configuration parameters specific to 3MHz CBW, that is, 
* AL4 for PDCCH in Table 6.2A.3.1.1-1
* No subbandSize configuration in Table 6.2A.3.1.1-2.

# 4	Summary

Observations:

Observation 1:	The existing SDR requirement specified in TS 38.101-4 5.5A is applicable for any channel bandwidth.

Observation 2:	The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposals:

Proposal 1:	RAN4 defines the 2Rx/4Rx PDSCH demodulation requirements with 3MHz CBW in FDD SCS=15kHz scenarios as a part of CA demodulation requirements.

Proposal 2:	Configure one PDCCH candidate with AL4 for 3MHz bandwidth CC in Table 5.2A-1.

Proposal 3:	Existing SDR requirement specified in TS 38.101-4 5.5A is applicable for CA including 3MHz CC.

Proposal 4:	For the SDR requirements, add the configuration parameters dedicated to 3MHz CC, that is, 2/AL1 for PDCCH in Table 5.5A-1, and 12 RBs for CORESET of SCS=15kHz in Table 5.5A-4.

Proposal 5:	Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA including 3MHz CC.

Proposal 6:	For CA CQI reporting requirements, add the configuration parameters specific to 3MHz CBW, that is, 
* AL4 for PDCCH in Table 6.2A.3.1.1-1
* No subbandSize configuration in Table 6.2A.3.1.1-2.

# References

- RP-2504754, “Way Forward for [114bis][326] NR_less_than_5MHz_Ph2_demod”, China Telecom

- R4-2106090, “Way forward on UE demodulation and CQI reporting for channel bandwidths 35MHz and 45MHz for NR FR1”, Ericsson.

- RP-250781, “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel Corporation.