3GPP TSG-RAN WG4 Meeting #115	R4-2507107

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.32.6

Source:	Ericsson

Title:	SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# 1	Introduction

Rel-19 IoT-NTN Phase 3 WI includes three main objectives [1]:

- Support of Store & Forward (S&F) satellite operation with full eNB as regenerative payload

- Support of Capacity enhancements for uplink

- Support broadcast of PWS messages for NB-IoT

From the RAN4 demodulation performance point of view, only the UL capacity enhancement may affect to the SAN demodulation requirements. This contribution shows our view on the SAN demodulation requirements for the UL coverage enhancements.

# 2	Discussion

## 2.1	RAN1 discussion status

As of RAN1#120bis, RAN1 agreed to support to multiplex two UEs for NPUSCH format 1 via orthogonal cover codes (OCC). In the feature, the multiplexing is supported for single tone 3.75kHz and 15kHz only, that is, NPUSCH format 1 multi-tone configuration does not support the multiplexing via OCC.

Observation 1: NPUSCH format 1 single tone 3.75 kHz and single tone 15 kHz configurations support multiplexing via orthogonal cover codes (OCC).

The WID also includes studying the coverage enhancement for NPRACH, but RAN1 has concluded that NPRACH does not support coverage enhancement in Rel-19 IoT-NTN. Therefore, RAN4 does not need to define new NPRACH demodulation requirements in Rel-19 IoT-NTN.

Observation 2: NPRACH does not support coverage enhancement in Rel-19 IoT-NTN.

Proposal 1: No new NPRACH demodulation requirements in Rel-19 IoT-NTN.

## 2.2	Multiplexed NPUSCH format 1 demodulation requirements

Here is a summary of NPUSCH format 1 multiplexing feature:

- Supported channel is single tone NPUSCH format 1 only.

  - 3.75kHz single tone is spread across symbol level, with OCC length 2.

  - 15kHz single tone is spread across slot level, with OCC length 2.

- First UE uses the OCC code [1, 1] and the second UE uses the OCC code [1, -1].

- The total transmission length becomes double due to the OCC spread in time domain.

Since SAN requires new functionality to de-multiplex the NPUSCH symbols, RAN4 may need to define new NPUSCH format 1 in TS36.108/181.

However, RAN1 is still discussing the detailed multiplexing procedure. For example, RAN1 has not decided the DMRS mapping for 3.75kHz and detailed CDM for 15kHz single tone. We think it is early to discuss the SAN demodulation requirements.

Proposal 2: RAN4 wait for RAN1 conclusion on the detailed NPUSCH format 1 multiplexing with OCC-2.

# 3	Summary

Observations:

Observation 1: NPUSCH format 1 single tone 3.75 kHz and single tone 15 kHz configurations support multiplexing via orthogonal cover codes (OCC).

Observation 2: NPRACH does not support coverage enhancement in Rel-19 IoT-NTN.

Proposals:

Proposal 1: No new NPRACH demodulation requirements in Rel-19 IoT-NTN.

Proposal 2: RAN4 wait for RAN1 conclusion on the detailed NPUSCH format 1 multiplexing with OCC-2.

# References

- RP-250472, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek.