3GPP TSG-RAN WG4 Meeting #115	R4-2507114

Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.10.5.3

Source:	MediaTek Inc.

Title:	Discussions on demodulation performance requirements for NGSO channel model

Document for:	Discussion

# Introduction

At RAN4#114-bis meeting, a WF for demodulation performance requirements based on NGSO time varying doppler shift and propagation delay channel models was agreed [1]. In this contribution, we provide our views on remaining issues.

# Discussion

Issue 1-2-3 Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models - Proposals: - Option 1: - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 3: - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with. - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state. - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements.

In previous releases, RAN4 considered only NTN fading channel to define demodulation requirements for NTN, which ignores the time varying Doppler shift and propagation delay introduced by satellite moving. To our understanding, the large Doppler shift and propagation delay will be compensated before the baseband processing at UE side. Therefore, we support to add some fixed frequency offset and time offset during the test to reflect the residual errors with the TE-emulated varying Doppler shift and propagation delay models. RAN4 can determine whether to keep the sane SNR requirements depend on the simulation results.

Proposal 1: Regarding demodulation requirements based on time varying doppler shift and propagation delay channel models, we support Option 2. Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models.

Issue 1-2-5 Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model - agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable

We propose excluding tests 1-3 and 1-4 from consideration, as features like the 32 HARQ process and disabled HARQ are more related to functionality rather than demodulation performance.

Proposal 2: For NR-NTN, do not consider test 1-3 and 1-4 for introducing requirements as features like the 32 HARQ process and disabled HARQ are more related to functionality rather than demodulation performance.

Issue 1-2-8 Test case for eMTC NTN with applying the time-varying Doppler shift and propagation delay model - agreement - FFS to consider Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - If considering Cat-M1 demodulation requirement with applying time varying Doppler shift and propagation delay model - Cover test 1 and test 2 according to TS 36.102 Clause 8.2.1.1 - FFS to cover test 3

There are no conformance tests introduced in RAN5 for IoT-NTN Cat-M1. Additionally, we believe that IoT-NTN NB-IoT is more likely to be commercialized compared to IoT-NTN Cat-M1. Therefore, we prefer not to apply the time-varying Doppler shift and propagation delay model to Cat-M1.

Proposal 3: For IoT-NTN, do not consider applying the time-varying Doppler shift and propagation delay model to Cat-M1 test case.

# Conclusion

In this contribution, we provide our views on demodulation performance requirements for NGSO channel model. Observations and proposals are summarized as below.

Proposal 1: Regarding demodulation requirements based on time varying doppler shift and propagation delay channel models, we support Option 2. Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models.

Proposal 2: For NR-NTN, do not consider test 1-3 and 1-4 for introducing requirements as features like the 32 HARQ process and disabled HARQ are more related to functionality rather than demodulation performance.

Proposal 3: For IoT-NTN, do not consider applying the time-varying Doppler shift and propagation delay model to Cat-M1 test case.

# Reference

- R4-2504700, Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod, Samsung