3GPP TSG-RAN WG4 Meeting #115	R4-2507121

Malta, 19th – 23rd May 2025

Agenda item:	7.30.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM requirements for XR

Document for:	Discussion

- Introduction

The WI was revised at RAN#107[1] and WF was approved at RAN4#114bis [2]. In this contribution, we further discuss the remaining RRM requirements for XR.

- Discussion

According to the latest WF [2], the open issues of RRM requirements are described as follows.

Topic #3: XR skipping requirements impact Issue 3-2: Formulas for measurement delay extension <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, ( 8 + Nskip) Max(MGRP , SMTC period)) CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - FFS: CSSFinter Issue 3-3: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter

For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not. According to the principle of CSSF, it is the factor to present how to divide limited number of measurement occasions among measurement objects. To keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip. Therefore Option 1 should be applied.

Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip.

Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period))

As same as measurement delay extension with DRX, CSSF should be out of effect of Nskip even if the DRX cycle is considered.

Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- DRX cycle ≤ 320 ms

  - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

- DRX cycle > 320 ms

  - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip.

Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period))

Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- DRX cycle ≤ 320 ms

  - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

- DRX cycle > 320 ms

  - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

References

[2] 3GPP, RP-240107, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia.

[2] 3GPP, R4-2504911, “WF on RRM requirements for XR_Ph3”, Nokia.