3GPP TSG-RAN WG4 Meeting #115	R4-2507122

Malta, 19th – 23rd May 2025

Agenda item:	7.26.4

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM requirements for A-IoT

Document for:	Discussion

- Introduction

From the last meeting RAN4#114bis, we started to discuss RRM requirements on A-IoT device 1 based on the WID [1]. Some timing related issues are still open, we contribute the general view.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

Issue 1-1: A-IoT paging Agreement: - RAN4 not to define RRM requirements for A-IoT paging Issue 1-2: A-IoT data transmission Agreement: - Excepting the timing issue discussed in Issue 1-4, no other potential RRM requirement impact is identified for “A-IoT data transmission” Issue 1-3: A-IoT random access procedure Agreement: - RAN4 to include RRM requirements for correct A-IoT device behavior during CBRA and CFRA. FFS the details Issue 1-4: timing Agreement: - Further discuss whether there is RRM impact on transmit timing requirements of device pending on RAN1 conclusion.

As above stated, excepting RAN1 progress, we can discuss random access procedure and transmit timing.

From RAN 1 and RAN2 discussion to date, the procedure of R2D and D2R is illustrated as below.

Figure 1 R2D and D2R procedure.

In terms of RACH procedure, the synchronization between the reader and device is triggered each time the reader transmits R2D and the device receives it. The timing offset can be acquired from R2D message and continuous transmission of D2R after receiving R2D is not supported. Since there is no timing acquisition deference between the Msg1 and other D2R messages, we don’t need to specify additional RRM specification on A-IoT random access procedure.

Observation 1: There is no deference of timing offset acquiring process between the Msg1 and D2R.

Proposal 1: No need to specify additional RRM specification on A-IoT random access procedure in terms of timing requirement.

In general, the timing offset requirements between R2D and D2R is necessary, and sampling frequency offset is one of the factors should be considered, as described in figure 1. The synchronization is triggered each time the reader transmits R2D, and the device receives it, so the longer it takes after R2D is received, the larger the timing deviations due to large SFO. We need to specify how much timing deviations is acceptable and the timing adjustments for deviations over the tolerances for each single or multiple symbols.

Observation 2: The longer it takes after R2D is received, the larger the timing deviations due to large SFO.

Proposal 2: Need to specify how much timing deviations is acceptable and the timing adjustments for deviations over the tolerances for each single or multiple symbols.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: There is no deference of timing offset acquiring process between the Msg1 and D2R.

Observation 2: The longer it takes after R2D is received, the larger the timing deviations due to large SFO.

Proposal 1: No need to specify additional RRM specification on A-IoT random access procedure in terms of timing requirement.

Proposal 2: Need to specify how much timing deviations is acceptable and the timing adjustments for deviations over the tolerances for each single or multiple symbols.

References

[1] 3GPP, RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.

[2] 3GPP, R4-2504905, “WF on RRM requirements for Ambient_IoT_Solutions”, CMCC.