3GPP TSG-RAN WG4 Meeting #115	R4-2507124

Malta, 19th – 23rd May 2025

Agenda item:	7.23.3.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on requirements for measurement event prediction on AIML mobility

Document for:	Discussion

- Introduction

The SI was revised at RAN#105[1] and WF was approved at RAN4#114bis [2]. In this contribution, we further discuss the remaining requirements for RRM measurement event prediction on AIML mobility.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

RAN4 requirements for measurement event prediction Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define the requirement for the predicted RSRPs

As above stated, RAN4 needs to decide whether the option 1 or option 2 for the predicted RSRPs for indirect case. At least the requirement for the predicted RSRPs should be defined but we have been already discussing the RRM requirements on predicted RSRPs, thus, we can reuse the outcomes. For instance, the requirements were discussed in AIML air and in RRM measurement prediction.

Observation 1: The RRM requirements on predicted RSRPs have been discussed in the AIML air and RRM measurement prediction.

Proposal 1: Need to define requirement for the predicted RSRPs by reusing outcomes from other AIML discussions.

RAN4 needs to consider both absolute RSRPs and relative RSRPs, which depends on the event prediction scenario. The event predictions currently agreed upon for RAN2 are only event A3, so we consider the comparison between the serving cell quality and neighbour cell quality and we reuse relative RSRPs in the case.

Proposal 2: RAN4 reuses relative RSRP requirement for the predicted RSRPs.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: The RRM requirements on predicted RSRPs have been discussed in the AIML air and RRM measurement prediction.

Proposal 1: Need to define requirement for the predicted RSRPs by reusing outcomes from other AIML discussions.

Proposal 2: RAN4 reuses relative RSRP requirement for the predicted RSRPs.

References

[1] 3GPP, RP-242393, “Revised SID on AIML for mobility in NR”, OPPO.

[2] 3GPP, R4-2504901, “WF on AI/ML Mobility”, Nokia.