3GPP TSG-RAN WG4 Meeting #115	R4-2507140

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 SCM for demodulation

Agenda item:	7.16.3

Document for:	Information

# Introduction

In RAN4#114bis meeting, companies agreed to run simulations for alignment based on parameters were agreed in [1]. As for comparison among different configurations, cases, or different channel models are also encouraged.

In this contribution, both alignment simulation results and comparison simulation results are delivered.

# Simulation

CDL model name explanation for alignment and comparison (See Annex A)

CDLC A2_3 with 20 clusters: profile is derived from CDLC in TR38.901 including 20 clusters after cluster combination and without cluster truncation.  Both AAV 3 (1, 1) and AAV 1Y (8, 1) will be simulated.

CDLC A4_1 with 12 clusters: profile is derived from CDLC A2_3 with cluster truncation to 12 clusters. The spatial filter is based on AAV 3 (1,1).

CDLC A4_2 with 12 clusters: profile is derived from CDLC A2_3 with cluster truncation to 12 clusters. The spatial filter is based on AAV 1Y (8,1).

SNR normalization factor

- The gain ratio of AAV3 to isotropic AE is 0.7dB.

- The gain ratio of AAV1Y to AAV3 is 8.2dB and to isotropic AE is 8.9dB.

## 2.1	SU PDSCH

Table 2.1.1-1: Common simulation assumptions for PDSCH

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | For CDL channel: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination For TDL channel: Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Table 2.1.1-2: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

- Per CW performance                                                 (b) averaged performance

Figure 2.1-1 8TRx 8 layers PDSCH performance with wideband Type-1 CB, model A2_3 vs A4

- Per CW performance                                                 (b) averaged performance

Figure 2.1-2 8TRx 8 layers PDSCH performance with model A2_3, wideband vs sub-band Type-I CB

- Per CW performance                                                 (b) averaged performance

Figure 2.1-3 8TRx 8 layers PDSCH performance with model A4, wideband vs sub-band Type-I CB

- Wideband Type-I CB, model A2_3 vs A4

(b) model A2_3 Wideband vs sub-band Type-I CB                         (c) model A4 Wideband vs sub-band Type-I CB

Figure 2.1-4 4TRx 4 layers PDSCH performance

The simulation results are summarized in following table.

Table 2.1-1 Simulation results summary for PDSCH with model A2_3

| AAV config. | Rank | PMI granularity | SNR [dB] |  | SNR after norm. [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30% | 70% | 30% | 70% |
| AAV 3 | 8 | Wideband | 7.53 | 21.67 | 8.23 | 22.37 |
|  | 8 | Sub-band | 7.53 | 21.94 | 8.23 | 22.64 |
|  | 4 | Wideband | 5.73 | 15.78 | 6.43 | 16.48 |
|  | 4 | Sub-band | 5.73 | 15.7 | 6.43 | 16.4 |
| AAV 1Y | 8 | Wideband | -0.1 | 14.89 | 8.8 | 23.79 |
|  | 8 | Sub-band | -0.6 | 15.06 | 8.3 | 23.96 |
|  | 4 | Wideband | -2.25 | 8.16 | 5.65 | 17.06 |
|  | 4 | Sub-band | -2.21 | 8.15 | 5.69 | 17.05 |

Table 2.1-2 Simulation results summary for PDSCH with model A4

| AAV config. | Rank | PMI granularity | SNR [dB] |  | SNR after norm. [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 30% | 70% | 30% | 70% |
| AAV 3 | 8 | Wideband | 7.56 | 21.24 | 8.26 | 21.94 |
|  | 8 | Sub-band | 7.55 | 21.34 | 8.25 | 22.04 |
|  | 4 | Wideband | 5.66 | 15.78 | 6.36 | 16.48 |
|  | 4 | Sub-band | 5.69 | 15.72 | 6.39 | 16.42 |
| AAV 1Y | 8 | Wideband | -0.11 | 14.55 | 8.79 | 23.79 |
|  | 8 | Sub-band | -0.04 | 14.77 | 8.86 | 23.96 |
|  | 4 | Wideband | -2.29 | 8.17 | 5.61 | 17.07 |
|  | 4 | Sub-band | -2.27 | 8.14 | 5.63 | 17.04 |

## 2.1	SU PMI

Table 2.1.2-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8TX |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I for alignment Sub-band for Type I for comparison with eType II |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | typeII-r16 typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Test metric |  |  |  | SNR@90% of maximum throughput with follow Type I PMI Throughput ratio of follow Type 1 PMI to random PMI with SNR@90% of maximum throughput with follow Type 1 PMI SNR@90% of maximum throughput with follow eType II PMI Throughput ratio of follow eType II PMI to random PMI with SNR@90% of maximum throughput with follow eType II PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

Other parameters for CDL channel please refer to Table 2.1.1-2.

Figure 2.2-1 8Tx 4 layers and 2 layers PMI with AAV3 wideband Type-I CB, model A2_3 vs model A4

Figure 2.2-2 8Tx 4 layers and 2 layers PMI with AAV3 and model A2_3 and A4,  sub-band Type-I vs eType-II CB

Figure 2.2-3 8Tx 4 layers and 2 layers PMI with AAV1Y wideband Type-I CB, model A2_3 vs model A4

Figure 2.2-4 8Tx 4 layers and 2 layers PMI with AAV1Y and model A2_3 and A4, sub-band Type-I vs eType-II CB

All simulation results are summarized in following table.

Table 2.2-1 Simulation results summary for PMI with model A2_3

| AAV config. | Rank | CB config. | SNR [dB] |  | SNR after norm. [dB] |  | Gamma |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 70% | 90% | 70% | 90% |  |
| AAV 3 | 4 | WB Type-I | 9.74 | 12.51 | 10.44 | 13.21 | 1.76 |
|  | 2 | WB Type-I | 0.21 | 2.59 | 0.91 | 3.29 | 2.5 |
|  | 4 | SB eType-II | 8.5 | 12.52 | 9.2 | 12.22 | 1.76 |
|  | 2 | SB eType-II | -0.73 | 1.59 | 0.03 | 2.29 | 2.5 |
| AAV 1Y | 4 | WB Type-I | 1.57 | 4.59 | 10.47 | 13.49 | 1.8 |
|  | 2 | WB Type-I | -7.89 | -5.61 | 1.01 | 3.29 | 2.57 |
|  | 4 | SB eType-II | 0.43 | 4.21 | 9.33 | 13.11 | 1.83 |
|  | 2 | SB eType-II | -8.65 | -6.52 | 0.25 | 2.38 | 3 |

Table 2.2-2 Simulation results summary for PMI with model A4

| AAV config. | Rank | CB config. | SNR [dB] |  | SNR after norm. [dB] |  | Gamma |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 70% | 90% | 70% | 90% |  |
| AAV 3 | 4 | WB Type-I | 9.68 | 11.45 | 10.38 | 12.15 | 1.83 |
|  | 2 | WB Type-I | 0.11 | 1.44 | 0.81 | 2.14 | 2.72 |
|  | 4 | SB eType-II | 9.59 | 11.62 | 10.29 | 12.32 | 1.8 |
|  | 2 | SB eType-II | -0.69 | 0.65 | 0.01 | 1.35 | 3.1 |
| AAV 1Y | 4 | WB Type-I | 1.84 | 3.62 | 10.74 | 12.52 | 1.87 |
|  | 2 | WB Type-I | -8.04 | 6.57 | 0.86 | 15.47 | 2.81 |
|  | 4 | SB eType-II | 1.50 | 3.66 | 10.4 | 12.56 | 1.83 |
|  | 2 | SB eType-II | -8.58 | -7.33 | 0.32 | 1.57 | 3.1 |

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2504756, Way Forward for [114bis][320] FS_NR_demod_SCM, Nokia

[2]

# Annex A

Table A-1 CDL-C A2_1 and A2_2 with 24 clusters

| Cluster # | Absolute delay [ns] |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.00 |  | -4.4 |  | -38.2 |  | -96.9 |  | 96.8 |  | 98.1 |
| 2 | 76.61 |  | -1.2 |  | -21.8 |  | 118.9 |  | 98.5 |  | 71.1 |
| 3 | 80.99 |  | -3.5 |  | -21.8 |  | 118.9 |  | 98.5 |  | 71.1 |
| 4 | 85.01 |  | -5.2 |  | -21.8 |  | 118.9 |  | 98.5 |  | 71.1 |
| 5 | 79.42 |  | -2.5 |  | -34.2 |  | -124.4 |  | 100.9 |  | 67.6 |
| 6 | 232.36 |  | 0 |  | -5.9 |  | 171.2 |  | 99.2 |  | 76.6 |
| 7 | 235.35 |  | -2.2 |  | -5.9 |  | 171.2 |  | 99.2 |  | 76.6 |
| 8 | 239.44 |  | -3.9 |  | -5.9 |  | 171.2 |  | 99.2 |  | 76.6 |
| 9 | 240.32 |  | -7.4 |  | 44.2 |  | 51.9 |  | 106.4 |  | 62.8 |
| 10 | 289.63 |  | -7.1 |  | -50.6 |  | 63.4 |  | 94.5 |  | 56.6 |
| 11 | 299.77 |  | -10.7 |  | 49.1 |  | -42.1 |  | 107.5 |  | 69.8 |
| 12 | 340.76 |  | -11.1 |  | -73 |  | 43.1 |  | 92.3 |  | 50.8 |
| 13 | 448.40 |  | -5.1 |  | -44.2 |  | 65.1 |  | 104.6 |  | 103.4 |
| 14 | 477.53 |  | -6.8 |  | -50.4 |  | -63.4 |  | 105.2 |  | 50.1 |
| 15 | 792.20 |  | -8.7 |  | -60.2 |  | 79 |  | 91.7 |  | 51.7 |
| 16 | 989.33 |  | -13.2 |  | 64.6 |  | 26.3 |  | 105.2 |  | 121 |
| 17 | 1554.50 |  | -13.9 |  | 62.2 |  | -22.6 |  | 94 |  | 53.9 |
| 18 | 1679.11 |  | -13.9 |  | 55 |  | -1.6 |  | 91.8 |  | 61.6 |
| 19 | 2003.92 |  | -15.8 |  | -76.3 |  | -19.8 |  | 90.7 |  | 37.5 |
| 20 | 2046.81 |  | -17.1 |  | 57.4 |  | 4.5 |  | 108.2 |  | 53.1 |
| 21 | 2301.87 |  | -16 |  | 58.1 |  | 0.2 |  | 91.7 |  | 35.8 |
| 22 | 2422.65 |  | -15.7 |  | 67.3 |  | -4.9 |  | 91.6 |  | 52.9 |
| 23 | 2570.59 |  | -21.6 |  | 76.2 |  | -28.3 |  | 106.4 |  | 46.4 |
| 24 | 3158.09 |  | -22.8 |  | -91.4 |  | 29.3 |  | 109.5 |  | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

Note: In A2_1, all clusters are treated with 20 rays. In A2_2, ray splitting method described in TR38.827 will be applied. “For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). This helps keeping the rms DS of the modified CDL to 1s.”

Table A-2 CDL-C A2_3 with 20 clusters

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 232 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 240 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 290 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 300 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 340 | -14.1 | -73 | 43.1 | 92.3 | 50.8 |
| 9 | 448 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 476 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 11 | 792 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 12 | 988 | -16.2 | 64.6 | 26.3 | 105.2 | 121 |
| 13 | 1552 | -16.9 | 62.2 | -22.6 | 94 | 53.9 |
| 14 | 1678 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |
| 15 | 2002 | -18.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 16 | 2044 | -20.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 17 | 2300 | -19 | 58.1 | 0.2 | 91.7 | 35.8 |
| 18 | 2420 | -18.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 19 | 2568 | -24.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 20 | 3154 | -25.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-3 CDL-C A4_1 based on AAV (1,1)

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-2 CDL-C A4_2 based on AAV (8,1)

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 122 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 126 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 370 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 384 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 462 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 478 | -13.7 | 49.1 | -79.4 | 107.5 | 69.8 |
| 8 | 714 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 762 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1264 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 2478 | -16.9 | 62.2 | 33.4 | 94.0 | 53.9 |
| 12 | 2678 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |