3GPP TSG-RAN WG4 Meeting #115	R4-2507267

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on collision between MUSIM Gap and measurement Gap

Agenda item:	9.3

Document for:	Discussion

Introduction

RAN4 has received an LS from RAN2 related to MUSIM gaps, measurement gaps and collisions in [1]. The overall description is:

On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

Discussion

General issue

The LS seems to raise following two aspects:

- The network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa

- Network can configure the MUSIM gap without priority and the measurement Gap without priority together

Hence, this can be split into following scenarios:

- The network can configure the MUSIM gap without priority and the measurement Gap with priority

- The network can configure the MUSIM gap with priority and the measurement Gap without priority

- The network can configure the MUSIM gap without priority and the measurement Gap without priority

## 2.2	MUSIM gap without priority

Here we need to address two issues for the scenario when network has configured the UE with MUSM gaps without priorities and:

- Configured measurement Gap with priority

- Configured measurement Gap without priority

Hence, the network decides to configure the UE with MUSIM which has no priority. Additionally, the network configures measurement gaps either with (Type-2) or without priorities (Type-1).

During RAN4#106bis meeting RAN4 did discuss this issue:

Sub-topic 2-1 MUSIM gap priority configuration

Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side

- Proposals

  - P1: No need to further discuss how “UE can optionally indicate its preferred priority for all or a subset MUSIM gaps (Qualcomm Apple ZTE oppo)

  - P2: Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps (Huawei Qualcomm Nokia MTK xiaomi Ericsson Chapter vivo ZTE oppo Apple)

  - P3: If a UE has not indicated a priority, then the NW A shall set the priority to the lowest priority based on the gapPriority-r17 IE and the associated priority levels (16 levels defined in Rel-17) (Charter)

  - P4: If the UE requests priority for more than 1 MUSIM gap, priorities levels are different (Nokia Xiaomi MTK Ericsson)

Agreements:

Agree P2 - Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps

Additionally, 38.133 lists following in section 9.1.10.2:

The requirements in clause 9.1.10 apply provided different priorities are allocated to each periodic MUSIM gap and each measurement gap which is configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.

Hence, if the network selects configure the UE with MUSIM gaps without priorities, the UE requirements defined in section 9.1.10 does not apply. We believe this was discussed thoroughly when RAN4 discussed the UE requirements related to MUSIM gaps and hence, no need to re-open the discussion about UE requirements if network selects not to configure priorities for MUSIM gaps.

There is no need to re-open the discussion regarding the UE requirements when network selects to configure MUSIM gaps without priorities.

## 2.3	MUSIM gap with priority

Here we need to address one issue for the scenario when network has configured the UE with MUSM gaps with priorities and:

- Configured measurement Gap without priority

Hence, the network decides to configure the UE with MUSIM which has priority. Additionally, the network configures measurement gaps without priorities (Type-1).

This issue was discussed during the WI as well, and following was agreed in RAN4#109:

Issue 2-3-2: Solutions for collision between MUSIM gap and any measurement gap without assigned priority:

Agreement:

- P2: Collision is handled based on the MGRP of the collided gaps (Ericsson vivo Huawei ZTE Qualcomm MTK)

  - P2-1: In a collision, the gap occasion with longer MGRP will be kept when any measurement gaps in the collision gaps is not assigned a priority; and the gap occasion with shorter MGRP will be dropped.

  - P2-2: No requirements apply if any of the two gaps in a collision have the same MGRP.

RAN4 has captured the agreement in section 9.1.10.5:

Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps:

-	If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4.

-	Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

Hence, the issue has been addressed during the development and RAN4 has agreed UE requirements. We believe also this was discussed thoroughly when RAN4 discussed the UE requirements related to MUSIM gaps and we do not see a need to re-open the discussion about UE requirements for the scenario where the network configure MUSIM gaps with priorities measurement gaps without priorities.

There is no need to re-open the discussion about UE requirements for the scenario where the network configure MUSIM gaps with priorities and measurement gaps without priorities.

Conclusion

In this document we discuss the issues opened in the incoming LS from RAN2 in [1]. Based on the discussion and the RAN4 agreements when developing the UE MUSIM requirements, we conclude that the scenarios addressed in the LS were well addressed during the Rel-18 discussions. Hence, we propose:

- There is no need to re-open the discussion regarding the UE requirements when network selects to configure MUSIM gaps without priorities.

- There is no need to re-open the discussion about UE requirements for the scenario where the network configure MUSIM gaps with priorities and measurement gaps without priorities.

In the appendix we have provided a draft LS reply.

- References

- R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap

Appendix

3GPP TSG RAN WG4 Meeting #115              	                	                          R4-250xxxx

Title:	Reply LS on Collision between MUSIM Gap and Measurement Gap

Response to:	-

Release:	Release 18

Work Item:	NR_DualTxRx_MUSIM-Core

Source:	RAN4

To:	RAN2

Contact person:	Dalsgaard Lars

Lars.dalsgaard@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

RAN4 thanks RAN2 for the LS regarding Collision between MUSIM Gap and Measurement Gap.

Concerning the issue of when the network configures the MUSIM gap without priority and the measurement Gap with priority together and vice versa, RAN4 discussed these scenarios during Rel-18. Current RAN4 UE requirements are clear for these two scenarios, and no UE requirements apply.

Concerning the issue of when the network configures the MUSIM gap with priority and the measurement Gap without priority, RAN4 discussed this scenario during Rel-18. Current RAN4 UE requirements are defined for scenario.

Hence, RAN4 see no reason to define new requirements for the addressed scenarios.

# Action

To RAN2:

RAN4 respectfully ask RAN2 to take the RAN4 into to account.

# Dates of the next TSG RAN WG4 meetings

TSG RAN4 Meeting #116		25th – 29th  Aug, 2025	Bangalore, India

TSG RAN2 Meeting #116bis	13th – 17th Oct, 2025	Prague, CZ