# 3GPP TSG-RAN WG4 Meeting #115	R4-2507319
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.1.2.2.1

Source: 	OPPO

Title: 	Discussion on MPR reduction for single carrier

Document for:	Discussion

- Introduction

In RAN4 #114bis meeting, the power domain enhancements for single carrier were discussed and some agreements were approved in WF [1], and some issues need further discuss in this meeting.

This contribution further discusses power domain enhancement for single carrier based on the open issues.

- Discussion

Issue 1: How to derivation of new inner RB region in extended CBW

Case 1: Only extended X1*NRB at the higher side of the UE CBW

The new inner regions only extended at higher side of UE CBW by X1*NRB will be decided by following functions:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = (1+X1)NRB – floor(LCRB/2) – LCRB

RBStart,High = NRB– LCRB

We can get LCRB1=(1+X1)/2*NRB, LCRB2=2*X1*NRB, and when X1=1/3, LCRB2 =LCRB1, the new inner regions will only be decided by following functions:

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB– LCRB

The new inner regions only extended at higher side of UE CBW 0<X1<1/3 and 1/3≤X1≤1/2 are shown in Figure 2-1 and Figure 2-2:

Figure 2-1 New inner region at the higher side extends by X1*NRB and 0<X1<1/3

Figure 2-1 New inner region at the higher side extends by X1*NRB and 1/3≤X1≤1/2

Based on the figures, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations only extended X1*NRB at the higher side of the UE CBW, X1 ≤1/2 is the extended factor.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

The RB allocation is an Inner RB allocation if the following conditions are met

When 0<X1<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X1)/2 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= (1+X1)*NRB –max(1, floor(LCRB/2))– LCRB,  when ceil(2*X1*NRB) ≤ LCRB ≤  ceil( (1+X1)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(2*X1*NRB)

When 1/3≤X1≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= NRB – LCRB

where ceil(x) is the smallest integer greater than or equal to x.

Case 2: Only extended X2*NRB at the lower side of the UE CBW

The new inner regions only extended at lower side of UE CBW by X2*NRB will be decided by following functions:

RBStart,Low = max(1, floor(LCRB/2))-X* NRB

RBStart,Low =1

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

We can get LCRB1=(1+X2)/2*NRB, LCRB2=2*X2*NRB, and when X2=1/3, LCRB2 =LCRB1, the new inner regions will only be decided by following functions:

RBStart,Low =1

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

The new inner regions only extended at lower side of UE CBW 0<X2<1/3 and 1/3≤X2≤1/2 are are shown in Figure 2-3 and Figure 2-4:

Figure 2-3 New inner region at the lower side extends by X2*NRB and 0<X2<1/3

Figure 2-4 New inner region at the lower side extends by X2*NRB and 1/3≤X2≤1/2

Based on the figures, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations only extended X2*NRB at the lower side of the UE CBW, X2 ≤1/2 is the extended factor.

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

The RB allocation is an Inner RB allocation if the following conditions are met

When 0<X2<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X2)/2 *NRB), and

RBStart,low= max(1, floor(LCRB/2))-X2* NRB,  when ceil(2*X2*NRB) ≤ LCRB ≤  ceil( (1+X2)/2 *NRB)

RBStart, low= 1, when LCRB < ceil(2*X2*NRB), and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

When 1/3≤X2≤1/2

RBStart,Low≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart, low= 1, and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

Case 3: Symmetric extension at the both sides of the UE CBW, extended X*NRB at both side of the UE CBW

The new inner regions only extended at both sides of UE CBW by X*NRB are shown in Figure 2-5 and Figure 2-6:

Figure 2-5 New inner region at the both sides extend by X*NRB and 0<X<1/3

Figure 2-6 New inner region at the both sides extend by X*NRB and 1/3≤X≤1/2

The new inner regions extended at both sides of UE CBW by X *NRB at each side will be decided by following functions:

RBStart,Low = max(1, floor(LCRB/2))-X* NRB

RBStart,Low =1

RBStart,High = (1+2*X)NRB – max(1, floor(LCRB/2)) – LCRB

RBStart,High = NRB  – LCRB

Based on above functions and figures, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations for extended X*NRB at both side of the UE CBW, 0<X ≤1/2, X is the extended factors.

The RB allocation is an Inner RB allocation if the following conditions are met

When 0<X≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart,High= (1+2*X)*NRB – max(1, floor(LCRB/2)) – LCRB, when ceil(4*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(4*X*NRB), and

RBStart,low= max(1, floor(LCRB/2))-X* NRB, when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart, low= 1, when LCRB < ceil(2*X*NRB)

Case 4: Asymmetric extension at the both side of the UE CBW, extended X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW

The new inner regions only extended at both sides of UE CBW by X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW are shown in Figure 2-7 and Figure 2-6:

Figure 2-7 New inner region X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW and 0<X1<1/3 and 0<X2<1/3

Figure 2-8 New inner region X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW and 1/3≤X1≤1/2 and 0<X2<1/3

Figure 2-9 New inner region X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW and 0<X1<1/3 and 1/3≤X2≤1/2

Figure 2-10 New inner region X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW and 1/3≤X1≤1/2 and 1/3≤X2≤1/2

The new inner regions extended at both sides of UE CBW by by X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW will be decided by following functions:

RBStart,Low = max(1, floor(LCRB/2))-X2* NRB

RBStart,Low =1

RBStart,High = (1+X1+X2)NRB – max(1, floor(LCRB/2)) – LCRB

RBStart,High = NRB  – LCRB

Based on above functions and figures, the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations for extended X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW, 0<X1 ≤1/2 and 0<X2 ≤1/2, X1and X2 are the extended factors.

The RB allocation is an Inner RB allocation if the following conditions are met

When 0<X1<1/3 and 0<X2<1/3 or 1/3≤X1≤1/2 and 1/3≤X2≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X1+X2)/2 *NRB) and

RBStart,High= (1+X1+X2)*NRB – max(1, floor(LCRB/2)) – LCRB,  when ceil(2*( X1+X2)*NRB) ≤ LCRB ≤  ceil( (1+X1+X2)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(2*( X1+X2)*NRB), and

RBStart,low= max(1, floor(LCRB/2))-X2* NRB,  when ceil(2*X2*NRB) ≤ LCRB ≤  ceil( (1+X1+X2)/2 *NRB)

RBStart, low= 1, when LCRB < ceil(2*X2*NRB)

When 1/3≤X1≤1/2 and 0<X2<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤ceil( (1+X2)*2/3 *NRB), and

RBStart,High= NRB – LCRB, and

RBStart,low= max(1, floor(LCRB/2))-X2* NRB,  when ceil(2*X2*NRB) ≤ LCRB ≤  ceil( (1+X2)*2/3 *NRB)

RBStart, low= 1, when LCRB < ceil(2*X2*NRB)

When 0<X1<1/3 and 1/3≤X2≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X1+X2)*2/3 *NRB), and

RBStart,High= (1+X1+X2)*NRB – max(1, floor(LCRB/2)) – LCRB,  when ceil(2*(X1+X2)*NRB) ≤ LCRB ≤  ceil( (1+X1+X2)*2/3 *NRB), and

RBStart,High= NRB – LCRB, when LCRB < ceil(2*(X1+X2)*NRB), and

RBStart, low= 1

Based on above analysis, it is complicated for the asymmetric extension at the both side of the UE CBW with extended X1*NRB at the higher side of the UE CBW and extended X2*NRB at the lower side of the UE CBW. Therefore, we propose only consider case 1, case 2 and case 3 in R-19.

Proposal 1: Only define the inner RB allocation for below cases in R-19

Case 1: Only extended X*NRB at the higher side of the UE CBW

Case 2: Only extended X*NRB at the lower side of the UE CBW

Case 3: Symmetric extension at the both sides of the UE CBW by X*NRB at each side of the UE CBW

Proposal 2: The RB allocation is an Inner RB allocation if the following conditions are met

For case 1 that only extended X*NRB at the higher side of the UE CBW

When 0<X<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X)/2 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= (1+X)*NRB –max(1, floor(LCRB/2))– LCRB,  when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+X)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(2*X*NRB)

When 1/3≤X≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= NRB – LCRB

For case 2 that only extended X*NRB at the lower side of the UE CBW

When 0<X<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X)/2 *NRB), and

RBStart,low= max(1, floor(LCRB/2))-X* NRB,  when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+X)/2 *NRB)

RBStart, low= 1, when LCRB < ceil(2*X*NRB), and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

When 1/3≤X≤1/2

RBStart,Low≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart, low= 1, and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

For case 3 that symmetric extension at the both sides of the UE CBW by X*NRB at each side of the UE CBW

When 0<X≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart,High= (1+2*X)*NRB – max(1, floor(LCRB/2)) – LCRB, when ceil(4*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(4*X*NRB), and

RBStart,low= max(1, floor(LCRB/2))-X* NRB, when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart, low= 1, when LCRB < ceil(2*X*NRB)

Issue 2: How to decide the additional fixed extension values X

In last meeting, it agreed RB based extension for MPR reduction for single CC:

  - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB

  - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5

  - The extended RBs are aligned with common RB grid

And how to consider the additional fixed extension values with capability should further study.

In previous meeting, we provided the related simulation results in [2] research the relationship of the MPR reduction and the extended UE CBW. And we got that:

- When extended UE CBW is 0.5*CBW at each side of the UE CBW, all outer RB allocation converts to inner RB allocation and the MPR can be improved up to 2.5dB.

- When extended UE CBW is 0.25*CBW at each side of the UE CBW, the MPR can still be improved up to 2dB although not all outer RB allocation are converted to inner RB allocation.

- When extended UE CBW is 0.125*CBW at each side of the UE CBW, only small part of regions for outer RB allocation can get improvement.

In order to get the extended benefit of MPR reduction as large as possible, the limited number of additional fixed values X should larger than 0.125.

According to the analysis of Issue 1, we can see the new inner  region will be decided by different functions for  0<X<1/3 and 1/3≤X≤1/2

Table 2-1 The extended NRB at each side of the UE CBW with different fixed extended value X

|  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| NRB with 15kHz | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 |
| X=1/4 | 4 | 7 | 13 | 20 | 27 | 34 | 40 | 47 | 54 | 61 | 68 |
| X=1/3 | 5 | 9 | 18 | 27 | 36 | 45 | 54 | 63 | 72 | 81 | 90 |
| X=1/2 | 8 | 13 | 26 | 40 | 53 | 67 | 80 | 94 | 108 | 121 | 135 |

Table 2-1 gives the extended NRB at each side of the UE CBW with different fixed extended value X, we can see:

  - The small step extension for most UE CBW is important, for example, for 20MHz CBW, there is almost 10RBs difference when X increases from 1/4 to 1/3; for 40MHz CBW, there is almost 18 RBs difference X increases from 1/4 to 1/3; for 50MHz CBW, there is almost 22RBs difference X increases from 1/4 to 1/3.

  - The small step extension for a few of small UE CBW is no big meaning, for example, for 3MHz CBW, there is only 1RBs difference X increases from 1/4 to 1/3; for 5MHz CBW, there is 2RBs difference X increases from 1/4 to 1/3; for 10MHz CBW, there is only 5RBs difference X increases from 1/4 to 1/3.

Based on above analysis, to get the extended benefit of MPR reduction as large as possible for most UE CBW, the additional fix extended values X can adopt one of the values 1/4, 1/3.

Proposal 3: The limited number of the fixed values X can adopt the values of 1/4, 1/3, 1/2.

Issue 3: How to derivate extended IBE

In last meeting, there are two options as below excerpt:

  - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below)

  - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

About option 2, some companies observed that it can’t meet the MPR=0 adopting 0.5*NRB/0.5*CBW extended at both sides of UE CBW that equals 50MHz with 15kHz SCS.

To further check this issue, we do the simulation two cases for the symmetric extension at both sides of the UE CBW that one case is extended UE CBW=2* NRB = 540 RBs= 97.2MHz and another case is extended UE CBW= UE CBW+ NRB=98.6MHz, separately. The simulation results are shown in Figure 2-11 and Figure 2-12:

Figure 2-11 the MPR for UE CBW =50Mhz with 15kHz SCS and extended UE CBW=2* NRB = 97.2MHz

Figure 2-12 The MPR for UE CBW =50MHz with 15kHz SCS and extended UE CBW= UE CBW+ NRB=98.6MHz

From Figure 2-11 and Figure 2-12, we can see all MPR=0 for UE CBW =50MHz with 15kHz SCS and UE CBW=2* NRB or extended UE CBW= UE CBW+ NRB.

Proposal 4: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Proposal 5: The extended IBE requirements should be an extension of the IBE requirement of the UE CBW by replacing NRB with NRB,E as below functions:

max-25-10⋅log10(NRB,E/LCRB),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,
-57dBm+10log10SCS/15kHz-PRB

Where NRB,E=NRB,E,L +NRB,E,R+ NRB, NRB,E,L is the extended RBs on left side of UE CBW, NRB,E,R is the extended RBs on right side of UE CBW. And no relaxation is needed.

Issue 4: Capability

How to extend the UE CBW depends on the network not UE, so it doesn’t need define separate UE capabilities for scenario 1 and scenario 2. The UE only need indicate the related capabilities, and UE can indicate more than one extended values.

Proposal 6: it doesn’t need define separate UE capabilities for scenario 1 and scenario 2.

Proposal 7: UE need indicate whether supports R19 MPR reduction. If support, UE can further indicate one or more additional extended values it supports, if not indicate the additional extended value, the UE supports default extended value.

Proposal 8: If Network can configure R19 MPR reduction for the UE, the network need enable this feature and configure the extended value.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band, and proposed:

Proposal 1: Only define the inner RB allocation for below cases in R-19

Case 1: Only extended X*NRB at the higher side of the UE CBW

Case 2: Only extended X*NRB at the lower side of the UE CBW

Case 3: Symmetric extension at the both sides of the UE CBW by X*NRB at each side of the UE CBW

Proposal 2: The RB allocation is an Inner RB allocation if the following conditions are met

For case 1 that only extended X*NRB at the higher side of the UE CBW

When 0<X<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X)/2 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= (1+X)*NRB –max(1, floor(LCRB/2))– LCRB,  when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+X)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(2*X*NRB)

When 1/3≤X≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High= NRB – LCRB

For case 2 that only extended X*NRB at the lower side of the UE CBW

When 0<X<1/3

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+X)/2 *NRB), and

RBStart,low= max(1, floor(LCRB/2))-X* NRB,  when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+X)/2 *NRB)

RBStart, low= 1, when LCRB < ceil(2*X*NRB), and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

When 1/3≤X≤1/2

RBStart,Low≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( 2/3 *NRB), and

RBStart, low= 1, and

RBStart,High = NRB – max(1, floor(LCRB/2)) – LCRB

For case 3 that symmetric extension at the both sides of the UE CBW by X*NRB at each side of the UE CBW

When 0<X≤1/2

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart,High= (1+2*X)*NRB – max(1, floor(LCRB/2)) – LCRB, when ceil(4*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB)

RBStart,High= NRB – LCRB, when LCRB < ceil(4*X*NRB), and

RBStart,low= max(1, floor(LCRB/2))-X* NRB, when ceil(2*X*NRB) ≤ LCRB ≤  ceil( (1+2*X)/2 *NRB), and

RBStart, low= 1, when LCRB < ceil(2*X*NRB)

Proposal 3: The limited number of the fixed values X can adopt the values of 1/4, 1/3, 1/2.

Proposal 4: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Proposal 5: The extended IBE requirements should be an extension of the IBE requirement of the UE CBW by replacing NRB with NRB,E as below functions:

max-25-10⋅log10(NRB,E/LCRB),
20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,
-57dBm+10log10SCS/15kHz-PRB

Where NRB,E=NRB,E,L +NRB,E,R+ NRB, NRB,E,L is the extended RBs on left side of UE CBW, NRB,E,R is the extended RBs on right side of UE CBW. And no relaxation is needed.

Proposal 6: it doesn’t need define separate UE capabilities for scenario 1 and scenario 2.

Proposal 7: UE need indicate whether supports R19 MPR reduction. If support, UE can further indicate one or more additional extended values it supports, if not indicate the additional extended value, the UE supports default extended value.

Proposal 8: If Network can configure R19 MPR reduction for the UE, the network need enable this feature and configure the extended value.

- Reference

[1] R4-2505099 WF on power domain enhancement, Huawei, RAN4 #114bis

[2] R4-2412351 R19 MPR reduction for single carrier, OPPO, RAN4 #112