# 3GPP TSG-RAN WG4 Meeting #115	R4-2507322
Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.10.3.2.1

Source: 	OPPO

Title: 	Discussion on Tx requirements for NR-NTN HPUE

Document for:	Discussion

- Introduction

In last RAN4 meeting, the HPUE Tx RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved and some open issues need further discussion in this meeting as listed in WF [1]:

Tx requirements Issue 1-1-2: Power boosting option for PC2 Agreement: - FFS on the power boosting option for PC2, including - Whether or not and how to define power boosting for PC2 as proposed above. - The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2. Issue 1-8-2: PC1 MPR Way forward: - To collect MPR analysis and determine MPR for PC1 in next meeting. Issue 1-9: A-MPR Ad-hoc agreements: - Discuss the A-MPR in the draft CRs in next meeting.

This contribution further discussed the UE Tx RF requirements for NR NTN HPUE.

- Discussion

In last meeting, it was agreed PC1 ACLR is [34] dBc and ACLR used in PA calibration should be aligned with ACLR requirements.

In current spec TS 38.101-1, the MPR requirements of NR TN PC1 UE with 1Tx for non-handheld UE is the same with the MPR requirements of NR TN PC3 UE. Since, for PC3 UE, the 30dB ACLR means the Tx leakage is -7dBm in the adjacent channel for the maximum output power 23dBm, and for PC1 UE, the 37dB ACLR means the Tx leakage is -6dBm in the adjacent channel for the maximum output power 31dBm.

Since no PC1 PA in our simulation platform, we adopt similar method of evaluation MPR for TN PC1 UE with 37dB ACLR, here we use PC3 PA with ACLR 26dB to evaluate the MPR for PC1 NR NTN with 34dB ACLR that keep the Tx leakage is -3dBm in the adjacent channel.

Therefore, we calibrated the PC3 PA using 30dB ACLR and 26dB ACLR separately following below simulation assumptions:

- For PA model calibration:

  - DFT-s-OFDM QPSK 20 MHz

  - 100RB0

  - 4dB post PA loss

  - 1dB MPR

  - Carrier leakage 28dBc

  - IQ image:28 dBc

  - CIM3: 60 dBc

  - ACLR: PC3 with 30dB ACLR and PC3 with 26 dB ACLR

  - SEM: existing PC3 SEM

  - EVM: 17.5%

The MPR simulation results for PC3 with 30dB ACLR and PC3 with 26dB ACLR are attached in the Annex. Table 2-1 shows the largest MPR values of the Outer RB allocations and inner RB allocations for PC3 with 30dB ACLR and PC3 with 26dB ACLR separately based on the simulation results, and the difference of MPR for the cases of PC3 with 26dB ACLR and PC3 with 30dB ACLR are shown in Table 2-1:

Table 2-1 MPR for PC3 with 30 dB ACLR and 26dB ACLR

| Modulation |  | MPR (dB) for PC3 with 30dB ACLR |  | MPR (dB) for PC3 with 26dB ACLR |  | Delta MPR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations | Outer RB allocations | Inner RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤0.2 | 0 | ≤ 1.4 | ≤ 0.3 | 1.2 | 0.3 |
|  | QPSK | ≤ 0.9 | 0 | ≤ 2.2 | ≤ 0.9 | 1.3 | 0.9 |
|  | 16 QAM | ≤ 2.0 | ≤ 0.4 | ≤ 2.8 | ≤ 1.9 | 0.8 | 1.5 |
|  | 64 QAM | ≤ 2.1 |  | ≤ 3.6 |  | 1.5 |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 0.9 | ≤ 3.8 | ≤ 2.4 | 0.3 | 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.3 | ≤ 3.8 | ≤ 3.8 | 0.3 | 1.5 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 5.3 |  | 1.8 |  |

From Table 2-1 we can see relaxing the ACLR has larger impact to

- Inner RB allocation for QPSK, 16QAM and 64QAM for CP-OFDM and 16QAM and 64QAM for DFT-s-OFDM;

- Outer RB allocation for Pi/2 BPSK and QPSK for DFT-s-OFDM;

We proposed the MPR for NR NTN PC1 can be increased as below table 2-2 based on the MPR for TN PC1.

Table 2-2 Increased MPR for power class 1 with 26 dB ACLR based on the MPR for TN PC1

| Modulation |  | Increase MPR for NTN PC1 |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 1 | 0.5 |
|  | QPSK | 1 | 1 |
|  | 16 QAM | 1 | 1.5 |
|  | 64 QAM | 1.5 |  |
| CP-OFDM | QPSK | 0.5 | 1.5 |
|  | 16 QAM | 0.5 | 1.5 |
|  | 64 QAM | 1.5 |  |

Proposal 1: NR NTN PC1 using 34dB ACLR.

Proposal 2: NR NTN PC1 with 1Tx can adopt blow MPR:

Table 2-3 Maximum power reduction (MPR) for NR NTN PC1 with 1Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 | ≤ 1.5 | ≤ 0.5 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 1.5 | ≤ 1.5 | ≤ 0.5 |
|  | QPSK | ≤ 2.5 |  | 1 |
|  | 16 QAM | ≤ 3 |  | ≤ 2.5 |
|  | 64 QAM | ≤ 4 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  |  |

- Conclusion

In this paper we give further discussion on the MPR for HPUE NTN PC 1 with 1Tx. The proposals are as following:

Proposal 1: NR NTN PC1 using 34dB ACLR.

Proposal 2: NR NTN PC1 with 1Tx can adopt blow MPR:

Table 2-3 Maximum power reduction (MPR) for NR NTN PC1 with 1Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 | ≤ 1.5 | ≤ 0.5 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 1.5 | ≤ 1.5 | ≤ 0.5 |
|  | QPSK | ≤ 2.5 |  | 1 |
|  | 16 QAM | ≤ 3 |  | ≤ 2.5 |
|  | 64 QAM | ≤ 4 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  |  |

- Reference

[1]R4-2505113, WF on HPUE requirements for NR-NTN, Samsung, RAN4#114bis.

- Annex

MPR simulation results for PC3 with 26dB ACLR for DFT-s-OFDM:

MPR simulation results for PC3 with 26dB ACLR for CP-OFDM:

MPR simulation results for PC3 with 30dB ACLR for DFT-s-OFDM:

MPR simulation results for PC3 with 30dB ACLR for CP-OFDM: