3GPP TSG-RAN WG4 Meeting #115	                                                                     R4-2507344

Malta, MT, 19th-23rd May, 2025

Title: 	PMI simulation results for SCM

Source: 	Huawei, HiSilicon

Agenda item:	7.16.2

Document for:	Discussion

Background

This contribution provides our simulation results according to simulation assumptions summary [1].

- Simulation results

This section provides our simulation results for PMI. The simulation assumptions are captured in clause 2 of [1]. It is noted that following channel models are simulated.

A2 channel model(CDL-C-UMa model scaled from CDL-C model in 38.901) without ray splitting

A2 channel model (CDL-C-UMa model scaled from CDL-C model in 38.901) with ray splitting

A4 channel (Truncated channel model)

Enhanced TDL channel model with Cluster model 1

Enhanced TDL channel model with Cluster model 2

MCS13 is assumed for all cases and wideband granularity is assumed for Single panel I follow PMI cases and random PMI cases.,

Figure 2.2-1: Simulation results for PMI

Table 2.2-1: Simulation summary for PMI

| Chanel model | Rank2 |  |  |  |  |  | Rank4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random PMI |  | Type 1 PMI |  | eType 2 PMI |  | Random PMI |  | Type 1 PMI |  | eType 2 PMI |  |
|  | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP |
| A2 | 8.0 | 9.8 | 1.5 | 3.4 | 1.0 | 2.6 | 17.6 | 19.9 | 10.1 | 13.0 | 10.3 | 14.0 |
| A2 with ray splitting | 7.3 | 9.2 | 2.0 | 3.8 | 1.3 | 2.5 | 14.8 | 17.6 | 7.9 | 11.5 | 9.7 | 13.2 |
| A4 | 8.1 | 9.8 | 1.8 | 4.1 | 2.1 | 3.9 | 16.9 | 19.1 | 10.3 | 12.8 | 11.6 | 14.0 |
| Cluster model 1 | 7.3 | 8.1 | 3.8 | 5.1 | 2.5 | 3.6 | 15.6 | 16.7 | 12.1 | 13.5 | 13.6 | 15.1 |
| Cluster model 2 | 6.5 | 8.0 | 3.1 | 4.9 | 1.7 | 3.6 | 15.5 | 17.1 | 12.4 | 14.7 | 12.6 | 15.2 |

Table 2.2-2: Gama value for PMI

| Chanel model | Rank2 |  | Rank4 |  |
| --- | --- | --- | --- | --- |
|  | Type 1 PMI | eType 2 PMI | Type 1 PMI | eType 2 PMI |
| A2 | 2.12 | 2.31 | 1.8 | 1.69 |
| A2 with ray splitting | 1.94 | 2.19 | 1.76 | 1.58 |
| A4 | 1.96 | 2.02 | 1.65 | 1.78 |
| Cluster model 1 | 1.80 | 1.88 | 1.79 | 1.48 |
| Cluster model 2 | 1.73 | 1.88 | 1.48 | 1.36 |

- Conclusion

This contribution provides our PMI simulation results for SCM

- Reference

[1] R4-2504762 Simulation assumptions for SCM. Huawei, HiSilicon, MediaTek.