3GPP TSG-RAN WG4 Meeting #115									                        R4-2507788

St Julian’s, Malta, May 19 – 23, 2025

Agenda item:			7.10.2

Source:	Ericsson

Title:	On less than 5 MHz CBW for NTN and HPUE for NTN UE capabilities for Rel-19

Document for:	Discussion

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1], where the two main topics are “Less than 5 MHz CBW for NTN” and “HPUE for NR-NTN and IoT-NTN”.

In this contribution, we discuss whether UE capabilities need to be introduced to support these features, in order to potentially add them to the Rel-19 RAN4 UE features list.

# Discussion

## Less than 5 MHz CBW for NR-NTN

In RAN4#114 meeting, there was a following agreement:

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Agreement: - Introduce 3MHz Channel bandwidth for NR-NTN in FR1-NTN bands as optional feature in Rel-19. - Further discuss whether 3MHz channel bandwidth is mandatory on certain bands in Rel-20 if there is a specific request from operators.

Hence, 3 MHz CBW for NR-NTN has been agreed as an optional feature in Rel-19, where the UE indicates its support through the UE capabilities for the support for the symmetric and asymmetric channel bandwidths for 3 MHz CBW for NR-NTN, where the following was agreed in RAN4#114:

Issue 1-2: UE capability Signalling - Agreement: - The exiting UE capabilities signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 is reused for FR1 NTN bands. - The description of exiting UE capabilities signalling should be updated to support FR1 NTN bands.

Since it was concluded that the existing Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 will be reused for FR1 NTN bands, there is no need to introduce new UE capabilities with the same purpose in Rel-19.

Observation 1: Since it was concluded that the existing Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 will be reused for FR1 NR-NTN bands, there is no need to introduce new UE capabilities with the same purpose in Rel-19.

## HPUE for NR-NTN and IoT-NTN

The support for high power classes is going to be optional as well, just as for TN. In TS 38.306, the following ”per band” capabilities are defined for the UE support of power classes different than the default one:

| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], or in clause 6.2 of TS 38.101-5 [34], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT or NCR-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. This capability is not applicable for UEs indicating support of maxOutputPowerATG-r18. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

From the description of ue-PowerClass, ue-PowerClass-v1610 and ue-PowerClass-v1700, it can be noticed that TS 38.101-5 is already referenced to, which means that high power classes for NR-NTN are already supported by those UE capabilities in TS 38.306 specification.

Observation 2: High power classes for NR-NTN are already supported by ue-PowerClass, ue-PowerClass-v1610 and ue-PowerClass-v1700 UE capabilities in TS 38.306.

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: Since it was concluded that the existing Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 will be reused for FR1 NR-NTN bands, there is no need to introduce new UE capabilities with the same purpose in Rel-19.

Observation 2: High power classes for NR-NTN are already supported by ue-PowerClass, ue-PowerClass-v1610 and ue-PowerClass-v1700 UE capabilities in TS 38.306.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.