3GPP TSG-RAN WG4 Meeting # 115														R4-2508101

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.26.1

Source:	Moderator (Huawei)

Title:	WF on requirements for A-IoT BS and CW

Document for:	Approval

Introduction

This document captures the agreements about the RF requirements for A-IoT BS and CW on Rel-19 WI solutions for Ambient IoT in NR (RP-243326). The topic summaries are R4-2500687 (RAN4#114), R4-2504688 (RAN4#114bis) and R4-2507574 (RAN4#115). The previous WFs are R4-2502859 (RAN4 #114) and R4-2505097 (RAN4 #114bis).

Topic #1: A-IoT BS TX

- Issue 1-1: Base station output power

- Agreement:

  - A-IoT BS output power, Prated,c,AC≤ 38 dBm

- Issue 1-3-1: Transmitter OFF power

- Agreement:

  - Transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

- Issue 1-4-2: Modulation quality

- Agreement:

  - Consider following options

    - Option 1: Take the table below as a starting point.

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tari= 103M*15 (us) M = 2, 6, 12 and 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | [0.5~0.15] (A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | [0.65/0.525] Tari | µs |

      - Note 1: The ripple is calculated based on per-chip, comparing peak value of each chip to its own average.

      - Note 2: SIP is excluded for ripple calculation.

    - Option 2: No need to define RF envelop parameters.

- Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

|  | Option 1 |  | Option 2 |  | Option 3 |  | Option 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2D CBW | offset | ACLR | offset | ACLR | offset | ACLR | offset | ACLR |
| 200 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | 100kHz | 30/40dB |
| 400 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | [100]kHz | 30/40dB |
| 600 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |
| 800 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |

- Issue 1-5-1: Occupied bandwidth

- Agreement:

  - The occupied bandwidth for each A-IoT carrier shall be less than the A-IoT BS channel bandwidth.

- Issue 1-6: Transmitter intermodulation

- Agreement:

  - No need

Topic #2: A-IoT BS RX

- Issue 2-1-1: Reference sensitivity level

- Agreement:

  - Define REFSENS for both OOK and BPSK. Depending on the simulations, they may or may not be the same.

  - Use [X dB] as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as [TBD] dBm;

- Issue 2-1-2: LLS assumptions and parameters for SNR

- Agreement:

  - Update simulation results base on 31bits preamble.

  - 1-10% SFO can be considered and declared in the evaluation.

- Issue 2-1-3: SNR value

- Agreement:

  - Encourage companies update simulation results based on 31bit preamble.

- Issue 2-4-1: general blocking

- Agreement:

  - Reuse the in-band blocking requirement from legacy NR Medium Range BS for A-IoT BS

- Issue 2-8: ICS

- Agreement:

  - No ICS requirement is needed to be introduced.

Topic #3: CW

- Issue 3-2-1: OFF power

- Agreement:

  - OFF power is -50dBm, RBW is FFS

- Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

- Issue 3-6: Spurious emissions

- Agreement:

  - The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz

  - Spurious emissions limits are defined as in Table 2.2-5.

- Table 2.2-5: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |

- Issue 3-7: Intermodulation

- Agreement:

  - No need to define intermodulation.

Topic #4: draft TP	 to TS38.914

- Agreement:

  - Postpone all the TPs.

| No. | Section in TS38914 | Volunteer company | RAN4#115 TPs submitted | Revised to |
| --- | --- | --- | --- | --- |
| 1 | 5.3 BS channel bandwidth 5.4 Channel arrangement | CMCC | R4-2505616 |  |
| 2 | 6.1 General 6.2 Base station output power 6.3 Transmit ON/OFF power | CATT, LGE | R4-2506015 R4-2507642 |  |
| 3 | 6.4 Transmitted signal quality | Huawei，ZTE |  |  |
| 4 | 6.5 Unwanted emissions 6.6 Transmitter intermodulation | CMCC，CATT | R4-2506016 |  |
| 5 | 7.1 General 7.2 Reference sensitivity level 7.3 Dynamic range | ZTE, Huawei | R4-2507506 |  |
| 6 | 7.4 In-band selectivity and blocking 7.5 Out-of-band blocking | Ericsson | R4-2507495 |  |
| 7 | 7.6 Receiver spurious emissions 7.7 Receiver intermodulation 7.8 In-channel selectivity | Ericsson, ZTE | R4-2507495 |  |
| 8 | 8.1 General 8.2 CW Output power | Huawei, OPPO |  |  |
| 9 | 8.3 Frequency error 8.4 Unwanted emission | Vivo，ZTE | R4-2506844 |  |