# 3GPP TSG-RAN WG4 Meeting #115	R4-2508276

Malta, Malta, 19th - 23rd May, 2025

Agenda item:			7.27.1

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes for Netw_Energy_NR_enh

Document for:	Information

# Introduction

This document is the Ad-hoc minutes for Netw_Energy_NR_enh with the following topics covered.

Note: The following terminologies from RAN1 are re-used.

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

- Scenario #3A refers to “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to “When SCell activation is completed and SCell is activated” or “After SCell activation is completed and SCell is activated”

Note: The following abbreviations are used in the summary.

- AO-SSB: Always-on SSB

- OD-SSB: On-demand SSB

- FMW: Fast Measurement Window

# OD-SSB general

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW

Sub-issue 1: Lower bound for measurement in FMW

- Background

  - The agreement in last meeting is as follow.

Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value

- Proposals

  - Option 1: Nokia, LG, CMCC, ZTE

    - The lower bound is derived based on the OD-SSB periodicity of 10ms and no further relaxation is allowed if UE indicates the capability.

  - Option 2: vivo

    - The lower bound is also 10ms for EMP in Case 2.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - The lower bound 10ms is applied to FMW in deactivated SCell measurement in both Case 1 and Case 2 if UE indicates the capability.

Discussion

QC: If UE doesn’t report, it means no lower bound.

Xiaomi: Same question as QC. UE can report any value

Nokia: Too complex.

Xiaomi: Does QC mean UE need to report 10ms or no lower bound?

LG: In Case 2, how to understand if both AO-SSB and OD-SSB are 10ms?

Apple: No need to specify special case for Case 2 Alt C-1. In this case, UE will follow 10ms if UE needs lower bound.

Nokia: Do we need to follow lower bound for EMP?

HW: We should add a note to request RAN2’s help for capability design.

Agreement:

Introduce UE capability on whether a lower bound is needed, i.e.,

  - No signalling is needed if UE doesn’t need the lower bound (default value), or,

  - The lower bound for measurement/cell identification period in FMW is derived based on 10ms periodicity in case 1 and case 2

Note: RAN4 requests RAN2 to implement the UE’s capability based on the above agreement.

Issue 2-2-1: capability of lower bound

- Proposals

  - Option 1: Intel, Ericsson, Huawei

    - Introduce optional capabilities signalling for UE lower bound for measurement periodicity in FMW and the corresponding Rel-19 UE feature group.

- Recommended WF

  - RAN4 to agree the option 1.

  - The details can be revised in the feature list directly.

Candidate Feature list:

Type

Option 1: per UE

Option 2: per Band

Option 3: per BC

Discussion

OPPO: prefer Option 2. Follow RAN1 capability.

HW: prefer Option 3. Some UEs can implement the capability depends on the CA combination.

Issue 1-4-3: Additional Processing time for OD-SSB activation

- Background

In RAN4 #114bis meeting, RAN4 achieved agreements as follow.

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

- Proposal

  - Option 1: Qualcomm, vivo, Ericsson, CATT, Nokia, China Telecom, CMCC

    - RAN4 should introduce UE capability with values of 2 ms and 5 ms.

  - Option 2: MTK, Samsung

    - RAN4 should introduce UE capability with values of 3 ms and 5 ms.

  - Option 3: MTK

    - Not to introduce the capability but use 5ms

  - Option 4: CMCC

    - Not to introduce the capability but use 3ms

- Recommended WF

  - Moderator suggests the group to agree the following proposal based on majority views.

    - RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Discussion

Moderator: whether it’s fine to use single value?

QC: Single value is not a good idea, UE had to always follow it in a long time.

MTK: Use capability to this specific delay case is not preferred.

Nokia: NW cares about the value.

Agreement

  - RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

- Background

RAN1 sent an LS to RAN4 as follow.

Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

- Proposal

  - Option 1: Ericsson, Nokia, China Telecom, LG, CMCC, Huawei

    - RAN4 to confirm RAN1’s WS about OD-SSB deactivation processing time.

  - Option 2: Nokia, China Telecom, LG

    - Additional processing time is not needed upon OD-SSB deactivation.

  - Option 3: Qualcomm

    - RAN4 should define the same timeline for activating and deactivating OD-SSB.

- Recommended WF

  - Moderator suggests the group to agree the Option 1.

  - FFS the UE additional processing time for OD-SSB deactivation.

Discussion

QC: If we use a very short timeline, NW stops transmission OD-SSB in this timeline. UE may need longer time to revise the measurement results based on the OD-SSB.

Nokia: First we need to confirm RAN1’s timeline. Second, RAN4 can discuss whether additional processing time is needed.

Qualcomm, Apple: We need additional processing time.

HW: Why do we need this additional deactivation processing time? For example, in SCell deactivation, timeline is 3ms to stop all the procedures.

Apple: maybe sometime, UE may trigger the measurement report since this OD-SSB sample measurement result will be infinite and UE needs to resume other frequency layer immediately.

MTK: Now we define a procedure to prioritize the OD-SSN, but now UE needs to calculate to prioritize other layers.

Nokia, HW, ZTE: It’s same as SCell deactivation. We don’t give additional processing time. No requirement applied for the following procedure after OD-SSB deactivation.

QC: One more OD-SSB needs to be transmitted only for 5ms.

HW: RAN1 already gives the margin.

MTK: There is some potential issue. If UE assumes there is one OD-SSB needs to be transmitted, but NW may believe OD-SSB transmission has stopped and schedule data.

Wayforward

Option 1: No UE additional processing time for OD-SSB deactivation

Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Continue Discussion(Thursday)

Issue 2-2-2: capability of additional processing time

- Proposals

  - Option 1: Intel, Ericsson, Huawei

    - Introduce optional capabilities signalling for UE additional processing time for deactivated SCell measurement and the corresponding Rel-19 UE feature group.

- Recommended WF

  - RAN4 to agree the option 1.

  - The details can be revised in the feature list directly.

Discussion

Issue 1-4-7: L1 measurement in scenario 3B

- Background

In last RAN1 meeting, RAN1 made the agreement as follow.

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Proposals

  - Option 1: CMCC, Ericsson, Huawei

    - When MAC-CE indicates OD-SSB updating in scenario 3B,

      - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB if DRX is not configured,

      - Otherwise, UE still follow legacy DRX based requirement.

    - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4.

  - Option 2: LG

    - RAN4 to identify L1/L3 measurement requirement impact for Scenario#3B.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - When MAC-CE indicates OD-SSB updating in scenario 3B,

      - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB if DRX is not configured,

      - Otherwise, UE still follow legacy DRX based requirement.

    - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and [the addition processing time defined in RAN4].

Discussion

Agreement

  - When MAC-CE indicates OD-SSB updating in scenario 3B,

    - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB.

    - When DRX is configured, follow legacy principle to define requirements

  - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and [the addition processing time defined in RAN4].

Issue 1-4-8: Neighbor cell measurement in the carrier of OD-SSB serving cell before OD-SSB activation in Case 1

- Proposals

  - Option 1: Ericsson, Apple, Nokia, HW

    - UE to perform neighbour cell measurement based on legacy requirement with the configured measurement object.

  - Option 2: HW

    - UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered for both serving Cell and neighbour cell.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed.

    - UE to perform neighbour cell measurement based on legacy requirement with the configured measurement object.

Discussion

Apple: Based on RAN2 conclusion, UE should follow legacy mechanism.

HW: No strong view.

Agreement

- UE to perform neighbour cell measurement based on legacy requirement.

# OD-SSB based deactivated SCell measurement(Case 2)

Issue 1-2-1: Requirement for RAN1 Alt Time-C1

- Background

In last meeting, RAN4 made the agreements as follow.

Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures.

- Proposals

  - Option 1: Ericsson, Nokia, LG

    - Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

- Recommended WF

  - Moderator suggests the group to check whether Option 1 can be agreed.

Discussion

Agreement

Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

# OD-SSB based deactivated SCell measurement(Case 1)

Sub-issue 2: OD-SSB in FMW collided with MG

- Proposals

  - Option 1: vivo, Apple, Xiaomi, LG, CMCC

    - UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

  - Option 1a: Xiaomi

    - RAN4 to clarify the potential impacts on the length of FMW due to OD-SSB collision with MG.

  - Option 2: OPPO, Nokia, ZTE

    - UE to prioritize the OD-SSB carrier/layer within FWM

- Recommended WF

  - Note: When RAN4 designed the NR measurement framework in Rel-15, MG has the highest priority since MGRP has the longest periodicity, after that SMTC has the higher priority since RAN4 believe SMTC periodicity should be equal or larger than SSB periodicity.

Discussion

# SSB adaptation – L1 Requirements impacts

Issue 1-2-2: L1 requirements after transition  - DRX

- Proposals

  - Option 1: For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period. (Huawei, CTC)

    - Option 1a: The requirement when DRX is configured. (Samsung)

      - Following the legacy requirements based on DRX

    - Option 1b: keep (TDRX,TSSB) when DRX cycle≤320 ms in the L1-RSRP for serving cell. (Vivo)

- Recommended WF:

  - Agree on option 1.

Discussion

Apple: what about additional processing time?

Moderator: In other issue.

Agreement

- For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period

Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition

- Proposals

  - Option 1: UE can perform measurement based on the SSB resources after transition period, whether to release the measurement results from resources before transition up to UE implementation. (Samsung, vivo)

  - Option 2: In case the SSB adaptation command is received during the L1 measurement period, it is proposed to restart the L1 measurement being based on samples with the new SSB burst periodicity. (Nokia)

- Recommended WF:

  - Agree on option 1 and no spec impacts.

Discussion

Nokia: Our proposal is for L3 meas.

ZTE: In previous meeting, we had the agreement.

HW: We use a clear definition. From TSSB processing, new SSB periodicity is applied.

QC: No requirement in transition period. For option 1, NW may achieve some benefits from UE side. UE may continue the measurement before and after SSB adaptation.

HW: We don’t define the transition period as legacy.

Agreement

- No spec. impact is identified for releasing the measurement results.

- It’s up to UE implementation.

# SSB adaptation – L3 Requirements impacts

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell

- Proposals

  - Option 1: RAN4 should investigate whether the definition of the transition period, i.e., Tfirst_SSB + meas_period(P2) can also be applied for L3 measurements. (QC)

    - Option 1a: SSB burst periodicity of the SSB transmission, the cell identification and measurement period requirements apply based on the SMTC which is mapped with the indicated SSB burst periodicity from the first SMTC after the first SSB as defined in TBD. (Huawei)

    - Option 1b: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower. (Nokia)

  - Option 2: define L3 measurement requirement at transition, L3 measurement period is the maximum of the period corresponding to the SSB before and after adaptation. (LGE, CTC, CMCC, Ericsson, Samsung, ZTE)

    - Option 2a: No impact on RRM requirement due to additional SMTC configuration. For the requirement during transition period, it is suggested that the requirements follow the largest SMTC periodicity between old SMTC configuration and new SMTC configuration. (Vivo)

  - Option 3: During the transition, the L3 neighbor cell measurement requirements with the legacy SMTC for the non-adapted SSB can be applied. But after the transition, the L3 neighbor cell measurement requirements with additional SMTC configuration for the adapted SSB can be applied . (Xiaomi)

- Recommended WF:

  - Discuss whether to define L3 requirements at transition for SSB adaption, and consider following two options:

    - Option 1: Same at transition for L1 requirements: The cell identification and measurement period requirements apply based on the SMTC which is mapped with the indicated SSB burst periodicity from the Tfirst_SMTC

    - Option 2: Same as legacy: The cell identification and measurement period requirements apply based on the longer period before or after the transition

Discussion

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell

- Proposals

  - Option 1: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC mapped with SSB periodicity. (Samsung (for activated SCell), ZTE, Oppo, CTC, CMCC, Ericsson, Xiaomi)

    - Option 1: UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation (Oppo)

  - Option 2: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication. (Samsung)

    - The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC

    - FFS: Additional UE processing/preparation time for reception of SSB after adaptation

  - Option 3: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements. (LGE)

    - Option 3a: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation. (Oppo)

    - Option 3b: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. (Nokia)

- Recommended WF:

  - For activated SCell, Agree on option 1.

  - For deactivated SCell, discuss following options:

    - Option 1: legacy framework based on SMTC and meascycleScell

    - Option 2: reuse same framework as OD-SSB case#2.

# SSB adaptation – Other requirements impacts

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related

- Proposals

  - Option 1: For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command. (Xiaomi, CMCC, CTC, Ericsson, ZTE)

    - Option 1a: For SCell activation requirements based on the SSB periodicity after SSB adaptation, the requires apply the slot m when UE receives DCI format 2_9 for SSB adaption is no later than the slot n when UE receives SCell activation command (MAC CE or RRC), and no early than X ms before slot n. (Huawei)

- Recommended WF:

  - Agree on option 1.

Nokia: Fine with option 1.

Apple: Do we directly use the new SSB periodicity wo processing time in SCell activation?

Moderator: Another issue.

LG: Do we need to consider the valid Tfirst ssb after SCell activation?

HW: It’s a common issue in NES enh.

QC: Do we expect UE capability that UE can handle the DCI before SCell activation?

HW: We don’t need the capability.

OPPO: What’s the impact in RAN4?

HW: Follow the adapted SSB periodicity for SCell activation.

Xiaomi: Today, the start point is defined based on SCell activation command.

Nokia: Info: ‘no later than’ means ‘no later than in slot level’

Agreement

For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements

- Proposals

  - Option 1: R18 Scell activation with L3 reporting

    - No requirements: Oppo

    - Define requirements: Nokia, Huawei

  - Option 2: multiple SCells activation scenario with SSB adaptation

    - No requirements: Nokia

- Recommended WF:

  - Suggest to discuss the issue jointly with OD-SSB.

Moderator: suggest to treat together with OD-SSB part 1.

OPPO: For SSB adaptation based R18 Scell activation with L3 reporting, no new requirement is expected.

HW: The enhancement is to apply adapted SSB instead of SMTC.

Xiaomi: joint consider the DCI to trigger SSB adaptation and SCell activation

Nokia: DCI is only for SSB, SCell activation is another procedure.

OPPO: We don’t want to apply SSB periodicity in this scenario.

ZTE: If we define the requirement, whether the only impact is to change the SMTC to adapted SSB periodicity.

Moderator: Correct.

# Thursday Ad-hoc

Issue 1-1-1: DCI processing time for SSB adaption

Background:

RAN1#120bis Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst Table 11.5-1: Minimum time gap value d SCS (kHz); Number of slots. 15; 3. 30; 6. 60; 12. 120; 24. 480; 96. 960; 192.

- Proposals

  - Option 1: RAN4 to define the additional processing time for DCI2-9 (QC, Apple)

    - Option 1a: RAN4 should discuss the additional processing time required for DCI 2-9 based SSB adaptation. The MAC CE timeline for OD-SSB activation should serve as a starting point, i.e., {[2ms]/[3ms], 5ms}.(QC)

    - Option 1b: a similar additional preparation time (T_processing as agreed in OD-SSB activation) is needed upon SSB adaptation. (Apple)

    - Option 1c: RAN4 should discuss whether a UE capability for the additional processing delay for DCI 2-9 should be introduced or whether the additional processing delay should be directly captured in the RAN1 specification TS 38.213. (QC)

    - Option 1d: Once the DCI 2-9 timeline has been agreed in RAN4 from a reception point of view in the UE, RAN4 should send an LS to RAN1 about the RAN4 agreement. (QC)

  - Option 2: RAN4 to wait RAN1 progress for processing time for DCI2-9 (CMCC, LGE, Ericsson)

    - Option 2a: DCI 2-9 processing time can be further considered in Tfirst_SSB. (CMCC)

    - Option 2b: RAN4 to wait RAN1 progress for define Tfirst_SSB in L1 requirements at transition. (LGE)

- Recommended WF:

  - Discuss whether to consider additional delay for DCI triggered SSB adaptation for relevant RRM requirements. Candidate value of additional delay can be as follow:

    - 2 ms

    - 3 ms

    - 4 ms

Additional processing time: QC, Apple

Apple: No difference between MAC and DCI. UE will perform the same thing. Compromise to define capability as per BC and go with 4ms.

Nokia: Why not to allow 0ms? 2nd value can be one of those. UE capability

CMCC: Additional delay is for FMW. UE needs to perform OD-SSB meas. Immediately. Here, the situation is different. We have a legacy lower bound other than OD-SSB meas. lower bound. How to capture this 3ms?

HW: No challenge as OD-SSB. As a compromise, we can consider the additional delay in our spec. for transition requirement.

QC: We also have the same view as Nokia for capability. We can use 0/1ms and 4ms.

E///: We don’t think additional processing time needed. To make progress, we can use 0ms as baseline and 2ms.

ZTE: Same view as CMCC. We can compromise to 0ms and 2ms.

Apple: To ZTE, don’t understand lower bound is applied here.

HW: We discuss the same thing in XR. We use 5ms in XR for gap cancellation. Compromise to 2ms no capability,

QC: We think it’s different. 5ms is fine for XR. 2ms can be default value and use 1 bit to indicate 0ms and 4ms.

Apple: The difference is no measurement after cancellation.

E///: We can agree the default value, such as 0ms.

HW: Only L1 transition period requirement can use this value.

QC: It’s not about requirement.

HW: It’s not a purely RAN4 case.

E///: We can discuss transition period later.

Nokia: We think 2ms should be enough. In XR, we also have 3ms.

HW: We can compromise

Apple: FR1: 2ms FR2: 3ms

E///: Single value as 2ms.

CMCC: Even for L1, we don’t think it’s needed. We already have lower bound. The additional processing time only apply to the scenario that the measurement period is larger than the lower bound.

Agreement

RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation.

- FR1: 2ms FR2: 3ms

- FFS L3 transition period measurement

Issue 1-2-2: Requirement for RAN1 Alt Time-C2

- Background

RAN1 #120 Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

- Proposals

  - Option 1: Ericsson, CATT, Nokia, China Telecom, LG, CMCC, ZTE, Huawei, Samsung

    - RAN4 to define the requirement based on the OD-SSB periodicity.

  - Option 1a: Nokia

    - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow.

      - max(5*measCycleSCell, 5*DRX cycles) for FR1, or

      - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

    - Otherwise, the same approach in Case 1 can be reused

  - Option 1b: LG

    - RAN4 to specify requirements at least when periodicity of AO-SSB and OD-SSB are same.

  - Option 2: Qualcomm, OPPO

    - Not define requirements.

Discussion

Qualcomm, Apple: We don’t understand the scenario.

Moderator: RAN1 already defined capability for this case.

Qualcomm: We can compromise here.

OPPO: We can add some restrictions, such as 1b.

E///: We’re not sure. Option 1 is simple.

QC: To OPPO, why we need to compare two periodicities?

Apple: Not support.

Nokia: The scenario is for flexibility for NW. We understand the UE issue and compromise to only follow OD-SSB periodicity.

Agreement

- Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity.

- Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if

  - The periodicity of AO-SSB and OD-SSB are same

- It doesn’t impact the WI’s completion.

Issue 1-3-1: The known cell condition in Case 2

- Background

  - In RAN4 #113 meeting, the following agreement about known cell condition was made.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

- Proposals

  - Option 1: Nokia, Ericsson, CATT

    - The SCell shall be activated as a known SCell no matter if the known condition is determined based on always-on SSB and/or OD-SSB in Case 2 all scenarios.

  - Option 1a: Ericsson, CATT

    - The legacy known cell condition shall be updated to clarify the measured SSB and the SSB used in SCell activation can be different.

      - UE has sent a valid L3-RSRP measurement report with SSB index for the SCell being activated

        - The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

        - The SSB used in SCell activation has the same SSB index as the SSB measured which can be detectable

  - Option 1c: CMCC

    - The current known SCell activation delay methodology can be reused for the following cases:

      - When the SSB measured for known cell condition and the SSB used in SCell activation procedure are the same

      - When the SSB measured for known cell condition is AO-SSB not on sync-raster, and the SSB used in SCell activation procedure is OD-SSB

      - When the SSB measured for known cell condition is OD-SSB, and the SSB used in SCell activation procedure is AO-SSB not on sync-raster

  - Option 2: ZTE

    - For Case 2, it should be clarified that the SSB measured for known condition should be aligned with the SSB used in SCell activation procedure if AO-SSB and OD-SSB are different frequency and/or different QCL.

  - Option 3: CMCC

    - Further study whether the AGC adjustment time should be included in the known SCell activation delay requirement.

      - When the SSB measured for known cell condition is always-on SSB on sync-raster, and the SSB used in SCell activation procedure is OD-SSB, due to the frequency location is different.

- Recommended WF

  - Moderator suggests the group to check whether the following proposal can be agreed and check the wording in CR directly.

    - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

      - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

    - FFS whether apply to Alt Time-C2, if the requirement is defined.

    - FFS whether apply to scenario 3, if the requirement is defined.

    - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

Agreement

- The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination.

  - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

- FFS whether apply to Alt Time-C2, if the requirement is defined.

- FFS whether apply to scenario 3, if the requirement is defined.

- FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

Issue 1-4-10: OD-SSB interruption requirements

- Background

  - In last RAN4 meeting, RAN4 agreed to introduce the interruption requirements as follows.

Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

- Proposals

  - Option 1: LG

    - RAN4 to reuse the principle of interruption requirement as follows:

      - Interruption due to OD-SSB activation/deactivation: reuse the principle of interruption at SCell activation/deactivation

      - Interruption at the end of the FMW: reuse the principle of interruptions during measurements on deactivated SCC

  - Option 2: CMCC, Samsung

    - For Case 1 Scenario 2 and Case 2 Scenario 2 with AO-SSB not on the sync raster, the side condition of interruption shall include the measCycleSCell equal or longer than 640ms.

  - Option 3: Xiaomi, Nokia

    - No interruption of SCell addition/release is allowed due to OD-SSB activation/deactivation.

  - Option 4: Nokia

    - No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

  - Option 5: Ericsson

    - The interruption window end point Tx shall be as follow.

      - OD-SSB activation end point: Time instance A + T_processing

      - OD-SSB deactivation end point: Time instance B + T_processing

      - FMW end point: The end point of FMW + T_processing

- Recommended WF

Discussion

E///: change ‘of’ to ‘at’.

CMCC:

E///:

- No additional interruption at SCell addition/release [before SCell activation] due to OD-SSB activation(scenario 2).

- The interruption window end point ‘Tx’ to check in CR directly.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline

- Background

RAN1 sent an LS to RAN4 as follow.

Agreement - Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

- Proposal

  - Option 1: Ericsson, Nokia, China Telecom, LG, CMCC, Huawei

    - RAN4 to confirm RAN1’s WS about OD-SSB deactivation processing time.

  - Option 2: Nokia, China Telecom, LG

    - Additional processing time is not needed upon OD-SSB deactivation.

  - Option 3: Qualcomm

    - RAN4 should define the same timeline for activating and deactivating OD-SSB.

- Recommended WF

  - Moderator suggests the group to agree the Option 1.

  - FFS the UE additional processing time for OD-SSB deactivation.

Wayforward

Option 1: No UE additional processing time for OD-SSB deactivation

Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Apple: If there is scheduling restriction during the 2ms after OD-SSB deactivation, UE shall follow the scheduling restriction. The legacy scheduling restriction may be applied if a ‘virtual’ OD-SSB has planned to be transmitted from NW side. We can capture in scheduling restriction.

Nokia: We don’t think any special here. We don’t define any requirement for NW switching the cell. Same situation here. Now there is no SSB in this carrier. How to measure it?

E///: We have similar view. No scheduling restriction here.

CMCC: Similar view E///, Nokia.

Apple: In XR, we never require UE to perform meas. in the cancelled gap. It’s allowed UE to skip this occasion.

MTK: We can clarify the measurement restriction and scheduling restriction.

OPPO: Similar as SCell deactivation.

ZTE: We don’t agree there is measurement restriction and scheduling restriction. UE shall perform measurement immediately after the deactivation.

Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell

- Proposals

  - Option 1: RAN4 should investigate whether the definition of the transition period, i.e., Tfirst_SSB + meas_period(P2) can also be applied for L3 measurements. (QC)

    - Option 1a: SSB burst periodicity of the SSB transmission, the cell identification and measurement period requirements apply based on the SMTC which is mapped with the indicated SSB burst periodicity from the first SMTC after the first SSB as defined in TBD. (Huawei)

    - Option 1b: RAN4 to agree measurement period requirements only apply from the first SMTC occasion after reception of the SSB adaptation command, no matter whether the periodicity gets higher or lower. (Nokia)

  - Option 2: define L3 measurement requirement at transition, L3 measurement period is the maximum of the period corresponding to the SSB before and after adaptation. (LGE, CTC, CMCC, Ericsson, Samsung, ZTE)

    - Option 2a: No impact on RRM requirement due to additional SMTC configuration. For the requirement during transition period, it is suggested that the requirements follow the largest SMTC periodicity between old SMTC configuration and new SMTC configuration. (Vivo)

  - Option 3: During the transition, the L3 neighbor cell measurement requirements with the legacy SMTC for the non-adapted SSB can be applied. But after the transition, the L3 neighbor cell measurement requirements with additional SMTC configuration for the adapted SSB can be applied . (Xiaomi)

- Recommended WF:

  - Discuss whether to define L3 requirements at transition for SSB adaption, and consider following two options:

    - Option 1: Same at transition for L1 requirements: The cell identification and measurement period requirements apply based on the SMTC which is mapped with the indicated SSB burst periodicity from the Tfirst_SMTC

    - Option 2: Same as legacy: The cell identification and measurement period requirements apply based on the longer period before or after the transition

Discussion

HW: Not important. Option 2 should be fine. Then we don’t need to further discuss the UE processing time for L3.

CMCC: We don’t consider RRC processing time in transition period requirement. Option 2.

Apple: Fine with Option 2. RRC is different, 10ms.

Xiaomi: If NW trigger fast SSB, no gain here if we follow longer period, but we can compromise.

HW: Longer period is only applied to transition period.

Agreement

RAN4 to define L3 requirements at transition for SSB adaption same as legacy L3 transition period requirement.

- The cell identification and measurement period requirements apply based on the longer period before or after the transition.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell

- Proposals

  - Option 1: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC mapped with SSB periodicity. (Samsung (for activated SCell), ZTE, Oppo, CTC, CMCC, Ericsson, Xiaomi)

    - Option 1: UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation (Oppo)

  - Option 2: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication. (Samsung)

    - The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC

    - FFS: Additional UE processing/preparation time for reception of SSB after adaptation

  - Option 3: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements. (LGE)

    - Option 3a: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation. (Oppo)

    - Option 3b: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. (Nokia)

- Recommended WF:

  - For activated SCell, Agree on option 1.

  - For deactivated SCell, discuss following options:

    - Option 1: legacy framework based on SMTC and meascycleScell

    - Option 2: reuse same framework as OD-SSB case#2.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

RAN4 agreements in RAN4#114bis R4-2504907 Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Way forward: - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo) - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE) - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paging time. (E///, LGE, MTK) RAN1 Agreements in RAN1#120bis Agreement If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

- Proposals

  - Option 1: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (Oppo, Apple, CTC, Huawei, MTK, Nokia, Vivo)

    - Option 1a: the interruption time for UE capable of on demand SIB1 acquisition shall not exceed TOD-SIB1 +TSI-NR + 2*Ttarget_cell_SMTC_period ms. TOD-SIB1 is the time required for UL WUS transmission and RAR reception for on demand SIB1 acquisition of NES cell. (Oppo)

  - Option 2: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (QC, CATT, CMCC)

    - Option 2a: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined. (QC)

  - Option 3: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paging time.

    - Option 3a: If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period. (LGE)

    - Option 3b: UE shall prioritize the paging reception and delay the UL WUS procedure if collision happens.

  - Option 4: Paging interruption is not extended, provided the UL-WUS resource is not overlapping with Paging Occasion. (MTK)

  - Option 5: RAN4 should not define interruption requirements for the case that the UE monitors OD-SIB1 before UL WUS is transmitted. (QC)

- Recommended WF:

  - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell and

  - Considering define test case (s) with longer TSI-NR with reasonable value in performance part.