3GPP TSG-RAN WG4 Meeting #115													R4-2507833

Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:			4.1.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [115][301] BSRF_Maintenance_TEI

Document for:	Information

# Introduction

The scope of this topic summary is BS RF maintenance agenda items. Topics are divided according to the agenda:

Up to Rel-17 maintenance for LTE and NR and TEI:

- BS RF requirements and BS conformance testing	(4.3)

UE/BS EMC requirements	(4.4)	(No Tdocs)

Rel-16/17 TEI (BS RF related)	(4.8) 	(No Tdocs)

Rel-18 and Rel-19 maintenance for LTE and NR, TEI18 and TEI19:

- NR NTN enhancements:
System parameters and UE RF requirements	(5.17.1)

- Rel-18 and Rel-19 non-spectrum related Wis
BS/SAN/non-UE RF requirements	(5.25.2)

Rel-18 TEI
BS RF, demodulation performance and other topics	(5.26.3) 	(No Tdocs)

Rel-19 TEI
BS RF, demodulation performance and other topics	(5.27.3) 	(No Tdocs)

# Topic #1: BS RF requirements and BS conformance testing

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Title/Proposals |
| --- | --- | --- |
| R4-2507096 | NTT DOCOMO, INC, SoftBank Corp., Rakuten | Discussion on introducing additional EESS protection requirement for BS Proposal 1: In preparation for the next regulatory changes in Japan, so EESS protection requirements for 37 GHz – 40.5GHz is introduced. Proposal 2: This discussion is being addressed in maintenance WI. The workload is almost just about discussing EESS protection. Proposal 3: It will be introduced at least in Rel-19. Earlier releases can be discussed. (Draft CR text included as Annex) |

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2506078 | CATT | (NR_IAB-Core)CR for 38.174, Correction on out-of-band blocking co-location requirement Summary of change: 1) Change “IAB Node” to “IAB-MT” in Table 7.5.6-1. 2) Change “BS” to” IAB-DU and IAB-MT” or “IAB-DU” in Table 10.6.4-1. |
| R4-2506082 | CATT | (NR_IAB-Perf)CR for 38.176-1, Correction on out-of-band blocking co-location requirement Summary of change: 1) Change “IAB” to “IAB-DU” in Table 7.5.5.2-1, since the requirement is for IAB-DU instead of IAB. 2) Remove “for WA IAB-DU” for Wanted signal mean power in Table 7.5.5.2-1. 3) Change “IAB node” to “IAB-MT” in Table 7.5.5.4-1, since the requirement is for IAB-MT instead of IAB node. |
| R4-2506086 | CATT | (NR_IAB-Perf)CR for 38.176-2, Correction on out-of-band blocking co-location requirement Summary of change: 1) Change “IAB-Node” to” IAB-DU and IAB-MT” or “IAB-DU” in Table 7.6.5.1.2-1. |
| R4-2507074 | Nokia | (AAS_BS_LTE_UTRA-Core) CR to TR 37.842 on correction of table reference Summary of change: Correct the table reference in clause 5.3.2. |
| R4-2507075 | Nokia | (NR_newRAT-Core) CR to TR 38.817-02 on clarification on resource block size for receiver intermodulation requirements Summary of change: Clarify that ‘the modulated interfering signal channel bandwidth’ refers to those used in the requirement. |
| R4-2507088 | Ericsson | (NR_newRAT-Core) CR to 37.104: Correction of co-existence and co-location requirements Summary of change: “E-UTRA BS” is changed to “BS” in the Notes column. |
| R4-2507089 | Ericsson | (NR_newRAT-Perf) CR to 37.141: Correction of co-existence and co-location requirements Summary of change: “E-UTRA BS” is changed to “BS” in the Notes column and in the preamble to two tables. |
| R4-2507236 | Nokia | CR to 38.174 with clarification for channel bandwidths below 10 MHz Summary of change: Addition of clarification that CBWs below 10 MHz are not supported. |
| R4-2507240 | Nokia | CR to 38.176-1 with clarification for channel bandwidths below 10 MHz Summary of change: Addition of clarification that CBWs below 10 MHz are not supported. |
| R4-2507443 | Huawei, HiSilicon | (NR_newRAT-Core) CR to TR 38.817-02: correction of the interferer bandwidth derivation for the ICS requirements Summary of change: Correction of ICS interfering signal derivation, based on RAN1 specification. |
| R4-2507452 | Huawei, HiSilicon | (NR_newRAT-Core) CR to TS 38.104: correction of OTA Tx IMD wanted signal characteristics Summary of change: OTA Tx Intermodulation requirement correction and inconsistency removal. |
| R4-2507457 | Huawei, HiSilicon | (NR_repeaters-Perf) CR to TS 38.115-2: Correction of the OTA input intermodulation test requirement Summary of change: MU and TT values correction. Input intermodulation test requirement limits correction. |
| R4-2507459 | Huawei, HiSilicon | (NR_repeaters-Perf) CR to TS 38.115-2: Removal of TRP annex Summary of change: Removal of Annex G (TRP) and re-direction of references to related TRP annex in TS 38.141-2. |
| R4-2507461 | Huawei, HiSilicon | (NR_repeaters-Perf) CR to TS 38.115-2: spec quality and consistency improvements Summary of change: - EVM requirement was corrected to explicitly list applicable MCS, - MU and TT values corrections, - Repeater stimulus signal was corrected, - missing references were added, - multiple cross-references to annexes were corrected, - [] were removed, - other minor editorial corrections. |
| R4-2507463 | Huawei, HiSilicon | (AAS_BS_LTE_UTRA-Perf) CR to TS 37.145-2: removal of outstanding FFS Summary of change: Multiple outstanding FFS are removed. Missing Test Requirement derivations are added. |
| R4-2507614 | NTT DOCOMO, INC. | (NR_newRAT-Perf) CR for correction on the unit of OTA ACLR (Rel-15) Summary of change: Correct the unit of BS type 2-O ACLR limit in non-contiguous spectrum. |
| R4-2507874 | ROHDE & SCHWARZ | [37.941] Incorrect MU data for n104 band Summary of change: n104 band will not be represented in 37.941. These changes also need to be made in the excel MU tables, specifically in 37941 - Spreadsheet 1 - FR1 TX MU calculation tables.xlsx |
| R4-2507883 | ROHDE & SCHWARZ | [38.141-2] Incorrect MU data for n104 band Summary of change: Reduced frequency span of second clause to 6 GHz (from 7.125 GHz). That way the MU for n104 band is listed independent of the prior clause. |

# Topic #2: NR NTN enhancements; System parameters and UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2506154 | Ericsson | (NR_NTN_enh-Core) CR to TS 38108 - Missing SAN type 2-O definition Summary of change: Add SAN type 2-O definition. |
| R4-2507078 | Nokia | (NR_NTN_enh-Perf) CR to TS 38.181 on corrections of notes in tables Summary of change: 1) Align reference to Note 2 in Table 4.7.2-1. 2) Add Note ‘1’ in Table 10.3.5-2. |
| R4-2507469 | Qualcomm Incorporated, THALES, Ericsson | (NR_NTN_enh-Core) CR for TS 38.101-5 to clarify Doppler shift issues - Cat F CR Summary of change: To clarify Doppler shif issues in the sections of specturm emission mask with the same wording used in in section 6.5B.3.2 of TS 36.102. |

# Topic #3: Rel-18 and Rel-19 non-spectrum related WIs; BS/SAN/non-UE RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2505338 | Union Inter. Chemins de Fer | CR to TS 38.104: Transmit power for 3MHz CBW in band n100 Summary of change: Addition of the output power restrictions for 3MHz CBW in band n100. |
| R4-2506248 | ZTE Corporation, Sanechips | (NR_FR1_lessthan_5MHz_BW-Perf) CR to TS38.141-1 Introduction of 3MHz channel bandwidth for test configurations Summary of change: Add 3MHz channel bandwidth for test configurations. |
| R4-2506326 | ZTE Corporation, Sanechips | (NB_IOTenh3-Core)CR to TS38.113: FRC correction for performance criteria for continuous phenomena for BS Summary of change: Correct the FRCs to be matched with the NR channel bandwidth, and aligned with that in TS 38.104. |
| R4-2506937 | NEC | (FS_NR_IMT_4400_7125_14800MHz) CR to TR 38.922 on removal of undefined references Summary of change: X |
| R4-2506938 | NEC | (FS_NR_IMT_4400_7125_14800MHz) CR to TR 38.922 on BS spurious emissions for 4400 to 4800 MHz frequency range and spectral mask for 14800 to 15350 MHz frequency range Summary of change: “Basic limit” is replaced by “limit” in the tables for BS spurious emissions for 4400 to 4800 MHz frequency range and spectral mask for 14800 to 15350 MHz frequency range |
| R4-2506939 | NEC | (FS_NR_IMT_4400_7125_14800MHz) CR to TR 38.922 on BS antenna pattern for 14800 to 15350 MHz frequency range Summary of change: Introduce table 6.5.1.2-2 which is copied from table 3 in LS reply to ITU-R WP 5D in R4-2420385. |
| R4-2506940 | NEC | (FS_NR_IMT_4400_7125_14800MHz) CR to TR 38.922 on BS blocking response performance requirements for 4400 to 4800 MHz frequency range Summary of change: Add text “is re-used from TS 38.104 [9]” for out-of-band blocking requirement and blocking requirement for co-location with BS in other bands tables. |
| R4-2507079 | Nokia | (FS_NR_IMT_4400_7125_14800MHz) CR to TS 38.922 on corrections and clarifications on IMT parameters Summary of change: 1) Remove the verbal form ‘shall’ from the TR and align the wordings for the BS blocking response in the three frequency ranges. 2) Clarify only out-of-band blocking radiated requirements are listed in Table 6.3.2.3-1. |