3GPP TSG-RAN4 Meeting #115	R4-2506720

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5563 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_enh2-Perf) CR on TC maintenance for PUCCH Scell activation R18 (cat A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-04-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CSI report configurations are not correct. - The unit should be slot. - CSI report offset is not configured. - The configuration for non-PUCCH SCell should be removed. There is typo to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the CSI reporting configurations. Typo correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test caes are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.3.1, and A.5.5.3.12. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

#### A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2

##### A.5.5.3.11.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell with multiple SCell activation and deactivation delay requirement defined in clause 8.3, and interruption requirement defined in clause 8.2, when one known PUCCH SCell and one unknown SCell to be activated are in FR2.

The supported test configurations are shown in table A.5.5.3.11.1-1 below. The general test parameters are given in table A.5.5.3.11.1-2 and cell-specific test parameters in table A.5.5.3.11.1-3 below. OTA related test parameters are shown in table A.5.5.3.11.1-4.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, one E-UTRA cell, and three NR cells. Before the test starts the UE is connected to Cell 1 (PCell) on the E-UTRA carrier and Cell 2 (PSCell) on the NR carrier in FR2, but is not aware of Cell 3 (PUCCH SCell) or Cell 4 (SCell) on the NR carriers both in FR2. Cell 2 and Cell 4 are in the primary PUCCH group, and Cell 3 is in the secondary PUCCH group. In addition, Cell 2 and Cell 4 are in primary Timing Advance Group (pTAG), and Cell 3 is in the secondary Timing Advance Group (sTAG). Cell 1, Cell 2 and Cell 3 have constant signal levels throughout the test. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

There are two sub tests in this section.

-	For Test 1 (valid TA case), UE is provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG.

-	For Test 2 (invalid TA case), TimeAlignmentTimer of sTAG expires before UE receives the activation command

At the beginning of T1 the UE receives an RRC message by which the Cell 3 (PUCCH SCell) and Cell 4 (SCell) are configured on NR. The test equipment sends a single MAC message for activation of both Cell 3 and Cell 4 within 3 s for UE power class 2/3/4 or 4 s for UE power class 1 after RRM reports is sent for Cell 3.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. In the same MAC PDU, the test equipment activates the TCI state of RMC CORESET. In slot #m, the test equipment also sends an RRC message to configure the CSI-RS resources for both Cell 3 and Cell 4.

During T2, the UE shall be able to report valid CSI on PUCCH SCell for the activated PUCCH SCell at latest in

-	slot m+ Tactivate_total_PUCCH_SCell as defined in clause 8.3.13.

During T2, the UE shall be able to report valid CSI on PCell for the activated SCell at latest in

-	slot m+ Tactivate_total_other_SCell. as defined in clause 8.3.13.

Any PCell and PSCell interruption due to activation of PUCCH SCell shall occur in the slot m+1+THARQNR slot length to m+1+Tactivate_total_PUCCH_SCell+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Any PCell and PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to m+1+Tactivate_total_other_SCell+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of both Cell 3 and Cell 4, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3 msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.5.5.3.11.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.3.11.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3,4 | One E-UTRAN radio channel (1) and three NR radio channels (2,3,4) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.2 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2 in FR1. |
| Configured deactivated PUCCH SCell |  | Cell 3 | Configured deactivated secondary cell with PUCCH on NR RF channel number 3 in FR2 |
| Configured deactivated SCells |  | Cell 4 | Configured deactivated secondary cell on NR RF channel number 4 in FR2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| SCell measurement cycle (measCycleSCell) | ms | 160 | For both Cell 3 and Cell 4 |
| T1 | s | 7 | During this time the PSCell shall be known and the SCells configured, PUCCH SCell detected but SCell not detected. |
| T2 | s | 1 | During this time the UE shall activate both the SCells. |
| T3 | s | 1 | During this time the UE shall deactivate the SCells. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.5.5.3.11.1-3: Cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  | freq2 |  |  | freq3 |  |  |
| Duplex mode | Config 1,2 |  | TDD |  |  |  |  |  |  |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.3.1 |  |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 100: NPRB,c = 66 |  |  |  |  |  |  |  |  |
| DL initial BWP configuration | Config 1,2 |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1,2 |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
| UL initial BWP configuration | Config 1,2 |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1,2 |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 2 |  | ms | Not Applicable |  |  | 0 |  |  | 0 |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration | Config 1,2 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| CSI-RS configuration | Config 1,2 |  | CSI-RS.3.1 TDD |  |  | N/A | CSI-RS.3.1 TDD |  | N/A | CSI-RS.3.1 TDD |  |
| CSI reporting periodicity | Config 1,2 | slot | 40 |  |  | 40 |  |  | - |  |  |
| CSI reporting offset | Config 1,2 | Slot | 4 |  |  | 4 |  |  | - |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  | N/A | SSB.1 FR2 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.3.11.1-4: OTA related test parameters

| ParameterNote 6 | Unit | Cell 2 and 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15 kHzNote4 | -112 |  |  | -112 |  |  |
| Note1 | dBm/SCSNote3 | -102.97 |  |  | -102.97 |  |  |
|  | dB | 14 |  |  | N/A | 14 | 14 |
| SS-RSRPNote2 | dBm/SCS Note4 | -88.97 |  |  | N/A | -88.97 | -88.97 |
|  | dB | 14 |  |  | N/A | 14 | 14 |
| IoNote2 | dBm/95.04 MHz Note4 | -59.81 |  |  | -73.98 | -59.81 | -59.81 |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: All parameters apply for configuration 1 and 2 NOTE 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

### <End of Change 1>

### <Start of Change 2>

#### A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX

##### A.5.5.3.12.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell and DL SCell activation and deactivation times are within the requirements stated in clause 8.3.13, when the PUCCH SCell in FR2 and DL SCell in FR2 is unknown to the UE at the time of activation.

The supported test configurations are shown in table A.5.5.3.12.1-1 below. The test parameters are given in tables A.5.5.3.12.1-2 and cell-specific parameters in A.5.5.3.12.1-3 below. OTA related test parameters are shown in table A.5.5.3.12.1-4.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Before the test starts the UE is connected to Cell 1(PCell) on the E-UTRA carrier and Cell 2 (PSCell) on the NR carrier in FR2, but is not aware of Cell3 (PUCCH SCell) and Cell4(DL SCell2) on the NR carriers both in FR2. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test. SCC of Cell 3 and SCC of Cell 4 are on a same band.

At the beginning of T1 the UE receives an RRC message by which the Cell 3 and Cell 4 becomes configured on NR. The test equipment sends a single MAC message for activation of both SCells.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. In the same MAC PDU, the test equipment activates the TCI state of RMC CORESET. In slot #m, the test equipment also sends an RRC message to configure the CSI-RS resources for Cell 3 and Cell 4.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.5.5.3.12.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.3.12.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3,4 | One E-UTRAN radio channel (1) and three NR radio channels (2,3,4) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.2 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2 in FR1. |
| Configured deactivated SCells |  | Cell 3 (PUCCH SCell 1), Cell 4(DL SCell 2) | Configured deactivated secondary cell on NR RF channel number 3 and RF channel number 4, both in FR2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| SCell measurement cycle (measCycleSCell) | ms | 160 | For both Cell 3 and Cell 4 |
| TimeAlignmentTimer | ms | 1280 | Cell 1 and Cell 2 in pTAG. |
| TimeAlignmentTimerSTAG | ms | 1280 | Cell 3 and Cell 4 in sTAG |
| T1 | s | 1 | During this time Cell 3 and Cell 4 is configured and undetected. |
| T2 | s | 2 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.5.5.3.12.1-3: Cell specific test parameters

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  | freq2 |  |  | freq3 |  |  |
| Duplex mode | Config 1,2 |  | TDD |  |  |  |  |  |  |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.3.1 |  |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 100: NPRB,c = 66 |  |  |  |  |  |  |  |  |
| DL initial BWP configuration | Config 1,2 |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1,2 |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
| UL initial BWP configuration | Config 1,2 |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1,2 |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 2 |  | ms | Not Applicable |  |  | 0 |  |  | 0 |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,2 |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,2 |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration | Config 1,2 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| CSI-RS configuration | Config 1,2 |  | CSI-RS.3.1 TDD |  |  | N/A | CSI-RS.3.1 TDD |  | N/A | CSI-RS.3.1 TDD |  |
| CSI reporting periodicity | Config 1,2 | slot | 40 |  |  | 40 |  |  | - |  |  |
| CSI reporting offset | Config 1,2 | slot | 4 |  |  | 4 |  |  | - |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  | N/A | SSB.1 FR2 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.3.12.1-4: OTA related test parameters

| ParameterNote 6 | Unit | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | NA |  |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 |  | NA |  |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15 kHzNote4 | Link only, see clause A.3.7A |  |  | -112 |  |  | -112 |  |  |
| Note1 | dBm/SCSNote3 |  |  |  | -102.97 |  |  | -102.97 |  |  |
|  | dB |  |  |  | -∞ | 14 | 14 | -∞ | 14 | 14 |
| SS-RSRPNote2 | dBm/SCS Note4 |  |  |  | -∞ | -88.97 | -88.97 | -∞ | -88.97 | -88.97 |
|  | dB |  |  |  | -∞ | 14 | 14 | -∞ | 14 | 14 |
| IoNote2 | dBm/95.04 MHz Note4 |  |  |  | -73.98 | -59.81 | -59.81 | -73.98 | -59.81 | -59.81 |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: All parameters apply for configuration 1 and 2 NOTE 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

### <End of Change 2>