3GPP TSG-RAN WG4 Meeting#115                                      R4-2507573

Malta , MT, 19th – 23th May, 2025

Agenda item:			7.26.1

Source:	Moderator (CMCC)

Title:	Topic summary for [115][134] A-IoT_device

Document for:	Information

Introduction

This summary focuses on the R19 ambient IOT work item under agenda 7.26.2, 7.26.3.2, 7.26.5, including general, device and OTA requirements.  The summary document in RAN4#114 bis meeting is R4-2504687. The way forward agreed in RAN4#114 is R4-2505096.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2505541 | A-IoT general aspects | Huawei, HiSilicon | 7.26.2 |
| R4-2505619 | Discussion on A-IoT System parameters | CMCC | 7.26.2 |
| R4-2505753 | Discussion on AIoT system parameters | Xiaomi | 7.26.2 |
| R4-2506014 | Discussion on system parameters for A-IoT | CATT | 7.26.2 |
| R4-2506229 | Discussion on system parameters for ambient IoT | Spreadtrum,UNISOC | 7.26.2 |
| R4-2506840 | Discussion on the systeme parameter of AIoT | vivo | 7.26.2 |
| R4-2507182 | on system parameter | OPPO | 7.26.2 |
| R4-2507496 | A-IoT general overview | Ericsson | 7.26.2 |
| R4-2507505 | Discussions on General aspect for A-IoT | ZTE Corporation, Sanechips | 7.26.2 |
| R4-2505618 | Discussion on A-IoT device RF requirements | CMCC | 7.26.3.2 |
| R4-2505755 | Discussion on AIoT device 1 RF requirements | Xiaomi | 7.26.3.2 |
| R4-2506033 | Discussion on Ambient IoT device 1 RF requirements | CATT | 7.26.3.2 |
| R4-2506162 | REFSENS procedure, SFO and timing aspects | Qualcomm Incorporated | 7.26.3.2 |
| R4-2506230 | Discussion on RF requirements for ambient IoT device 1 | Spreadtrum,UNISOC | 7.26.3.2 |
| R4-2506842 | Discussion on the RF requirement for device 1 | vivo | 7.26.3.2 |
| R4-2507128 | RF requirements of ambient IoT device 1 | Sony | 7.26.3.2 |
| R4-2507181 | on RF requirements for device 1 | OPPO | 7.26.3.2 |
| R4-2507497 | A-IoT device requirement overview | Ericsson | 7.26.3.2 |
| R4-2507507 | Discussion on RF requirement of Ambient IoT device | ZTE Corporation, Sanechips | 7.26.3.2 |
| R4-2507632 | Discussion on RF requirement for A-IoT device 1 | LG Electronics UK | 7.26.3.2 |
| R4-2507722 | On the RF requirements for Ambient IoT Device | Huawei, HiSilicon | 7.26.3.2 |
| R4-2505395 | on Ambient IoT device OTA tests | Huawei, HiSilicon | 7.26.5 |
| R4-2505477 | Discussion on OTA testing for A-IoT device 1 | Ericsson Korea Partners Co Ltd | 7.26.5 |
| R4-2505620 | Discussion on OTA test method for A-IoT device | CMCC | 7.26.5 |
| R4-2505742 | Discussion on Ambient IoT Testability | Qualcomm Incorporated | 7.26.5 |
| R4-2507129 | Consideration on the OTA test of ambient IoT device 1 | Sony | 7.26.5 |
| R4-2507179 | on OTA test aspect | OPPO | 7.26.5 |
| R4-2506845 | draft CR to 38.870 on test method of AIoT device | vivo |  |
| R4-2506846 | Discussion on the OTA test method for device | vivo |  |

System parameters

- Topic 2-1: R2D bandwidth

Issue 2-1-1: R2D transmission bandwidth

Agreement in RAN4#114bis: - For RAN4 requirements, only define R2D transmission bandwidth configuration as 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS. - Define R2D channel bandwidth with assumption of 90% spectrum utilization.

Proposal 1 (CATT):

- The channel bandwidth, transmission bandwidth configuration and total guardband for different number of RBs are shown in figure 1 and table 1

- The guardband adjacent to the low channel edge should be symmetric to the guardband adjacent to the high channel edge.

- The guardbands for different number of RBs are shown in table 2.

Proposal 2 (Spreadtrum, UNISOC): Define R2D channel bandwidth: 200 kHz, 400 kHz, 600 kHz and 800 kHz with 15 kHz SCS

Proposal 3 (OPPO):

- The guard band should be 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

- The largest channel bandwidth for the each NR band can be the upper limit of the R2D channel bandwidth.

- Define the A-IoT R2D channel bandwidth as figure below:

Proposal 4 (ZTE): propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

Recommended WF:

Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration

- Topic 2-2: D2R bandwidth

Issue 2-2-1: D2R bandwidth

Agreement: - Define D2R channel bandwidth for A-IoT device: - Based on 2SB transmission, - Consider SFO impact - Consider small frequency shift defined in RAN1 - D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

Proposal 1 (Huawei): Define minimum D2R CBW (200 kHz) and maximum D2R CBW (depending on RAN1’s decision). Other D2R CBW FFS.

Observation 1: From device perspective, consider small frequency shift and SFO (assumed as 10^5 ppm) impact, D2R CBW can be calculated as follows:

D2R Channel BW device = (1.1*(R+1) * 2SB Transmission BW without SFO)/2

The 1.1 factor accounts for guard band due to 10% SFO effect.

Observation 2: From A-IoT BS perspective, consider FMDA small frequency shift and SFO (assumed as 10^5ppm) impact, and anti-interference filter, D2R CBW can be calculated as follows:

D2R Channel BW Reader = (1.1*(R+1) * 2SB Transmission BW without SFO)/2/0.9

The 1.1 factor accounts for guard band due to 10% SFO effect.

The 0.9 divisor presents the 90% BS filter spectrum utility (10% guard band)

Observation 3: Defining all calculated D2R channel BWs separately would complicate BS filter design and increase network configuration complexity.

Proposal 2 (CMCC):

- it’s suggested to define D2R CBW for all R, bit duration and chip duration values based on RAN1 conclusion.

- it’s suggested to define RF/testing requirements only for typical CBW. Details CBW can be decided case by case.

Proposal 3 (xiaomi):

- Define D2R transmission BW in RAN4 as following,

  - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

- D2R TBW are defined as the following values pending RAN1 further down selection,

  - 15, 30,	60, 120, 180, 240, 360, 480, 720, 960, 1440, 1920, 2880, 3840 kHz.

- RAN4 do not define CBW for AIoT device and reader side. Capture the Tb, Tchip and R relationship to RAN4 specification to show the SFS factor. SFO impact is implicitly included in SFO requirement.

Proposal 4 (CATT):

- We can decide the channel bandwidth of D2R after RAN1 has conclusions on D2R maximum data rate and SFO.

- If the small frequency shift is controllable and deterministic, the D2R channel bandwidth remains a fixed value. However, if the small frequency shift varies within a certain range, the D2R channel bandwidth must also account for the range of the shift.

Proposal 5 (Spreadtrum, UNISOC): To define all kinds of CBW for D2R based on RAN1’s conclusion about Tb and R. We can take several typical values from small to large for RF requirements.

Proposal 6 (vivo): Take the following table as the starting point of D2R channel bandwidth and transmission bandwidth:

| Btx,D2R (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BCH,D2R (kHz) | 16 | 32 | 63 | 126 | 189 | 252 | 378 |
| Btx,D2R (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| BCH,D2R (kHz) | 504 | 756 | 1008 | 1512 | 2016 | 3024 | 4032 |
| Note: Further update is needed based on RAN1 progress |  |  |  |  |  |  |  |

Proposal 7 (OPPO):

- Define the A-IoT D2R frequency shift Fs based on table 1 and with 10% accuracy as illustrated in figure 4.

- Define the A-IoT D2R transmission bandwidth based on table 2 and with 10% accuracy as illustrated in figure 4.

- The D2R channel bandwidth can be calculated as D2RCBW= 2*Fs+BWTX,D2R

Figure 4 D2R frequency shift and transmission bandwidth requirement

Proposal 8 (Ericsson):

- Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

- Wait Ran1 further down selection of Tb, Tc combination.

- Set the D2R channel bandwidth to OBW(Tb,Tc)+0.2*1/Tc + guard band

Proposal 9 (ZTE): propose RAN4 to define limited carrier bandwidth from BS receiver perspective to simplify the BS implementation and treat the D2R bandwidth and freq shift to CW as scheduling bandwidth and freq position within the carrier bandwidth.

Proposal 10 (Sony): RAN4 waits for RAN1's decision on the small frequency shift value and the chip duration value to determine the transmission bandwidth. However, RAN4 can consider defining a ±10% transmission BW as a guard band to accommodate the SFO.

Proposal 11 (Qualcomm): Do not include baseband harmonics in the channel bandwidth calculation

Recommended WF:

Define D2R channel bandwidth for A-IoT device with following options:

- Option 1: Define limited CBW from BS receiver perspective to simplify the BS implementation

  - option 1-1: Define minimum D2R CBW (200 kHz) and maximum D2R CBW (depending on RAN1’s decision). Other D2R CBW FFS

- Option 2: define CBW based on all Tb, Tc, R combinations as finally approved in RAN1.

  - Option 2-1: using following table as starting point and down-select based on RAN1 conclusion

| Btx,D2R (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BCH,D2R (kHz) | 16 | 32 | 63 | 126 | 189 | 252 | 378 |
| Btx,D2R (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| BCH,D2R (kHz) | 504 | 756 | 1008 | 1512 | 2016 | 3024 | 4032 |
| Note: Further update is needed based on RAN1 progress |  |  |  |  |  |  |  |

  - Option 2-2: using following equation: (1+SFO) *(2*Fs+BWTX,D2R)

- Tb = Tc×2R

- Btx,D2R = 4/Tb assuming 2SB

- frequency shift to CW Fs=1/(2Tchip)

- Option 3: use occupied bandwidth to quantify the CBW and D2R CBW equals to OBW(Tb,Tc)+0.2*1/Tc + guard band

- Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement in RAN4#114bis: - Consider following options for R2D channel raster - Option 1: define 10KHz as R2D channel raster (i.e. same as enhanced channel raster) - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS - Other options are not precluded.

Proposal 1 (Huawei):

- R2D Channel raster from A-IoT BS perspective is 10 kHz.

- R2D channel raster offset can be -7.5kHz or +2.5kHz.

Proposal 2 (CMCC): Define the same channel raster as macro reader for standalone mode

Proposal 3 (xiaomi): 10 KHz channel raster is used for AIoT system

Proposal 4 (CATT):

- The channel raster of R2D should be 10kHz

- RAN4 to specify R2D channel raster to RE mapping are shown in table 3

Proposal 5 (Spreadtrum, UNISOC): Define R2D channel raster as 10 kHz, and the offset can be - 2.5 kHz or 7.5 kHz.

Proposal 6 (vivo): For standalone operation, reuse same channel raster as NR in corresponding band, i.e., 100kHz in band n8. For in-band operation, the enhanced channel raster is used, i.e., 10kHz.

Proposal 7 (OPPO):

- For stand-alone mode,

  - -10kHz channel raster can be used for NR bands with 100kHz channel raster.

  - -15kHz channel raster can be used for NR bands with 15/30kHz channel raster.

- For in-band operation,

  - -10kHz channel raster can be used for NR bands with 100kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

  - -15kHz channel raster can be used for NR bands with 15/30kHz channel raster. For odd number of NRB, ±7.5kHz shift is needed.

Proposal 8 (Ericsson):

- For standalone operation, the legacy NR channel raster should be followed

- For in-band operation, reuse the 10kHz enhanced channel raster for A-IoT BS.

Proposal 9 (ZTE): for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]kHz.

Recommended WF:

- Define 10KHz channel raster

Channel raster offset is -2.5KHz or 7.5KHz offset

- Topic 2-4: Others

Issue 2-4-1: Foffset

Proposal 1 (Huawei):

Apply the Foffset for Ambient IoT as Table 2:

Table 2: Foffset for Ambient IoT

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Ambient IoT for R2D CBW 200/400/600/800kHz | 100 kHz |
| Note：Foffset is start from A-IoT Channel Bandwidth edge |  |

Recommended WF:

Wait for the conclusion BS OBUE requirements in thread [135].

Issue 2-4-2: maximum number of devices allowed to communicate simultaneously with one A-IoT BS

Proposal 1 (CATT): The maximum number of devices allowed to communicate simultaneously with one A-IoT BS needs to be clarified. .

Recommended WF:

TBD

Issue 2-4-3: UE capability

Proposal 1 (Ericsson): No UE capability is defined for A-IoT from RF perspective.

Recommended WF:

According to RAN2 discussion, Rel-19 device 1 will not have any UE capability report. No agreements in RAN4 are needed.

Issue 2-4-4: terminology

Proposal 1 (Qualcomm): R=1 is frequency shift by 1 in RAN4 terminology.

Device RF requirements

- Topic 3-1: Transmit output power

Issue 3-1-1: Transmit output power

Agreement in RAN4#114bis: - Performance metric of transmit output power: - Backscattering loss - Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss - For the Input CW Tx power to define requirements: - FFS on the whether to use one value or a range of CW Tx power - FFS on the value(s) of CW Tx power - Consider following aspects to determine backscattering loss to define requirements: - modulation loss - antenna efficiency - backscattering circuitry loss - impairment margin - minimum distance to the reader - differential radar cross section in two different modulation states - differential reflection coefficient in two different modulation states - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself - Other factors not excluded

Proposal 1 (Huawei): Further study the suitable requirements on backscatter loss based on the physics of backscattering as well as measurement data.

Proposal 2 (LGE): Ambient IoT device 1 backscattering loss used to define requirement is 14.3 dB

- Use 6dB as Modulation loss for Ambient IoT device1

- Use -4.3dB as antenna efficiency for Ambient IoT device 1

- Backscattering circuitry loss is treated as equivalent to modulation loss.

- Use 1dB as backscattering circuitry loss only for internal loss.

Proposal 3 (ZTE): for the back scattering loss of device, propose to consider the -2dB for OOK and -6dB BPSK

Proposal 4 (Ericsson):

- Use the max backscatter loss in the 10 percentile point requirement for OTA test

- Backscatter loss should be better than 15 dB to achieve 15 m with R other than 1.

- Add the test condition to identify the frequency of the test signal , i.e frequency offset of filter center to the frequency of the CW signal.

- Specify the spherical coverage requirement for max backscatter loss associated with Xth percentile

Proposal 5 (OPPO):

- To test the backscattering loss with TRP metrics

- To define backscattering loss as -9dB for OOK and -6dB for BPSK

Proposal 6 (Sony):

- RAN4 can consider adopting -1 dBi realized antenna gain, 0.9 and 1.9 reflection coefficient difference for OOK and BPSK as a starting point to derive the backscattering loss of AIoT devices.

- RAN4 considers verifying the backscattering loss based on the following options:

  - Option 1: EIRP-based measurement of one or more pairs of incident angles and scattering angles.

  - Option 2: TRP-based measurement with one or more incident angles.

- RAN4 needs to take into account a 3 dB polarization mismatch loss if the backscattering loss is measured as average value between two orthogonal polarized CW waves.

Proposal 7 (vivo):

- Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss.

  - The backscatter loss depends on the mismatch between antenna impedance and load impedance which should be similar for different incoming CW power

- The backscatter loss is also derived based on link-budget with 15m coverage target, and the following formula can be used:

  - Backscatter loss=minimum D2R@Reader(no backscatter loss)-REFSESNSReader+IM

- Take the 10 dB backscatter loss for device 1 as starting point.

- The input CW power for emission and backscatter loss is set to -25 dBm as side condition.

Proposal 8 (Spreadtrum, UNISOC):

- We need to assume the tag surrounding material such as cardboard

- Impairment margin can be 3dB

- Define backscattering loss as 10dB

- We need to assume the distance between CW node and tag, such as 15m to get the input CW power

- Based on the link budget, we can define the minimum backscattered power is -33dBm

Proposal 9 (Qualcomm): Maximum output power is met with maximum CW power level. Output power is assumed to scale linearly with CW level until minimum CW level

Proposal 10 (CATT):

- The terminology of Device 1 Tx output power requirements could be defined as Radiated power

- Device 1 radiated power requirement should be defined as the maximum backscattered power of Device 1 at the maximum CW Tx power and at a designated distance

Proposal 11 (xiaomi):

- The maximum input CW power to calculate device 1 radiated power can be assumed as 0 dBm

- EIRP is used as the metric to test transmit output power. FR2 BS EIRP range declaration approach can be considered.

Proposal 12 (CMCC): it’s suggested to only define either Tx backscatter power or Rx REFSENSE with orientation requirements.

Recommended WF:

Some companies discuss the aspects to determine backscattering loss, which are summarized as below:

- Modulation loss: [6]dB

- antenna efficiency/gain: [-1]dB

- impairment margin: [-3]dB

- 3 dB polarization mismatch loss if the backscattering loss is measured as average value between two orthogonal polarized CW waves

- Differential reflection coefficient: [0.9] for OOK, [1.9] for BPSK

While some companies provide the range of backscatter loss collected from commercial capability point of view:

- OOK: [-10] ~ [-2]

- BPSK: [-10] ~ [-6]

It is recommended that:

- Take [10] dB backscatter loss for device 1 for OOK as starting point to define requirements

- Use EIRP as performance metric:

  - Option 1: X percentile point of the CDF, like spherical coverage approach

  - Option 2: like FR2 BS EIRP range declaration approach

- Input CW Tx power:

  - Max input CW Tx power with [1m] or [MCL] distance assumption.

- Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Agreement in RAN4#114bis: - 10% SFO is assumed as the minimum performance for devices. - FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Proposal 1 (OPPO): Not to introduce a new requirement for SFO but together consider with the D2R frequency shift and transmission bandwidth.

Proposal 2 (Sony):

- 10% SFO requirement should be defined for AIoT devices

- the SFO requirement may not need to be tested explicitly if there are other requirements that can guarantee the SFO performance.

Proposal 3 (Vivo): The 10% SFO can be reflected in D2R channel bandwidth and implicitly verified in unwanted emission. No need to define a dedicated RF requirement for SFO.

Proposal 4 (Spreadtrum, UNISOC): To define SFO requirement. If SFO defined, SFO can be obtained by calculating the deviation number of sampling points.

Proposal 5 (Qualcomm):

- Define spectral confinement based on +/- 10 % maximum error in sample clock

- Define TR->D minimum and maximum as +/- 10 % of the nominal value

Proposal 6 (xiaomi): Define SFO requirement for device 1

Proposal 7 (CMCC): no SFO

Recommended WF:

No need to SFO if D2R CBW definition will include SFO

FFS whether to explicitly define SFO requirements or not.

Issue 3-2-2: Modulation quality requirements

Agreement in RAN4#114bis: - Define modulation quality requirements for both BPSK and OOK. - Consider to define same modulation quality metric for both BPSK and OOK if possible. - Reference measurement process needs to be discussed

Proposal 1 (Huawei):

- To identify the quality metric and corresponding requirements for backscatter modulation, the following need to be studied:

  - The reference demodulation process

  - The sources and characteristics of the impairments, and how to model them

  - To quantify the impact of the backscatter impairments to demodulation

- In case a more suitable modulation quality metric cannot be defined in time, the backscatter loss can be viewed as a performance metric to evaluate how effective the backscatter modulation is. The lower the backscatter loss is, the more effective the modulation.

Proposal 2 (Ericsson):

- BLER can be tested for the backscattered modulation quality as a directional requirement.

- Test feasibility should be discussed before specifying the modulation signal quality requirements.

Proposal 3 (Sony): There is no need to define additional modulation quality requirements for OOK since the modulation depth can be verified via a backscattering loss test. Further discussion on BPSK is needed to determine if any EVM-based requirement shall be defined or if it is sufficient with the backscattering loss test.

Proposal 4 (Spreadtrum, UNISOC): There is no need to define modulation quality requirements. The modulation quality requirements can be reflected by demodulation requirements

Proposal 5 (CATT): For modulation quality, a less stringent EVM requirements than BPSK EVM requirement could be defined for Device 1 (only applies for ON chip)

Proposal 6 (xiaomi):

- EVM can be a candidate metric for modulation quality requirements

- Device 1 Tx signal SNR is defined [10] dB higher than receiver DEMOD SNR threshold

Recommended WF:

- Further discuss with following options

  - Option 1: no need to define modulation quality requirements at least for OOK

    - Option 1-1: no need for OOK, the modulation depth can be verified via a backscattering loss test

    - Option 1-2: no need for OOK and BPSK, The modulation quality requirements can be reflected by demodulation/performance requirements, e.g. BLER

  - Option 2: EVM at least for BPSK

- Topic 3-3: Emission requirements

Issue 3-3-1: SEM requirements

Agreement in RAN4#114bis: - Discuss whether to define EIPR or TRP as performance metric for emission requirements - Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary. - FFS on CW maximum input power level for testing.

Proposal 1 (Huawei):

- Sufficient guard band should be considered when defining the SEM requirements, owing to the presence of odd-order harmonics in the backscattered signal

- It’s preferrable that the SEM and spurious emission requirements are forward-compatible with device 2 which is to be introduced in Rel-20.

Proposal 2 (Ericsson): The TRP metric should be used for SEM and Spurious requirement.

Proposal 3 (OPPO): To use -25dBm/10kHz TRP for the inside OOB range requirement, the same as EU regulation emission mask.

Proposal 4 (vivo):

- The emission requirement is only verified with 15 kHz transmission bandwidth.

- The input CW power for emission and backscatter loss is set to -25 dBm as side condition.

Proposal 5 (Spreadtrum, UNISOC): Define TRP requirements for unwanted emissions.

Proposal 6 (Qualcomm): Emission are met with maximum CW input level

Proposal 7 (xiaomi):

- TRP is used for the emission requirements, some simplified test method can be used

- Emission requirement is only tested for no small frequency shift mode

Proposal 8 (CMCC):

- use TRP as performance metric for emisison requirements

- SEM applies from the edge of CBW with flat model assumption. 16dBc is suggested as baseline with following assumptions.

| parameter | value |
| --- | --- |
| Data rate | We consider 7kbps. Larger data rate is the worst case. |
| coding | The FEC impact for final spectrum performance is tiny. But this will impact finally chip rate, we use 1/3code rate convolution code. Compared with miller code, Manchester code has wider spectrum characteristics, it’s suggested to use Manchester code. |
| Modulation scheme | The spectrum characteristics of OOK modulation will much concentrate on center but BPSK will have wider spectrum bandwidth, we can use BPSK. |

Recommended WF:

- Use TRP as performance metric

- The applicable frequency range for SEM

- Option 1: from the CBW edge to OOB boundary. Odd-order harmonics will be reflected into SEM limit

- Option 2: reserve sufficient guard band, owing to the presence of odd-order harmonics in the backscattered signal

- Define flat SEM, FFS for the value

  - Option 1: -25dBm/10kHz, the same as EU regulation emission mask

  - Option 2: [16dBc] based on baseband signal characteristics with following detailed parameters

| parameter | value |
| --- | --- |
| Data rate | We consider 7kbps. Larger data rate is the worst case. |
| coding | The FEC impact for final spectrum performance is tiny. But this will impact finally chip rate, we use 1/3code rate convolution code. Compared with miller code, Manchester code has wider spectrum characteristics, it’s suggested to use Manchester code. |
| Modulation scheme | The spectrum characteristics of OOK modulation will much concentrate on center but BPSK will have wider spectrum bandwidth, we can use BPSK. |

- CBW: only consider with min CBW

- Side condition

  - Emission requirement is only tested for no small frequency shift mode

  - Input CW level: max input level with [1m] or [MCL] assumption, use the same input CW level as backscattering loss.

Issue 3-3-1: spurious emission requirements

Agreement in RAN4#114bis: - Discuss whether to define EIPR or TRP as performance metric for emission requirements - Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary. - FFS on CW maximum input power level for testing.

Proposal 1 (Huawei): It’s preferrable that the SEM and spurious emission requirements are forward-compatible with device 2 which is to be introduced in Rel-20.

Proposal 2 (Ericsson): The TRP metric should be used for SEM and Spurious requirement.

Proposal 3 (OPPO): To use the D2R channel bandwidth as the OOB boundary.

Proposal 4 (vivo): The following emission requirement is defined for device 1.

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| Note: The requirement is not applied within D2R channel bandwidth |  |  |

Proposal 5 (Spreadtrum, UNISOC): Define TRP requirements for unwanted emissions. For different channel bandwidth, define D2R channel bandwidth plus minimum D2R channel bandwidth as the boundary between A-IoT out of band and spurious emission domain.

Proposal 6 (xiaomi):

- TRP is used for the emission requirements, some simplified test method can be used

- Emission requirement is only tested for no small frequency shift mode

Proposal 7 (CMCC):

- use TRP as performance metric for emisison requirements

- the OOB is suggested to be defined as Fb as in ITU, i.e. max of 500kHz and 10 times NB where NB equals to D2R CBW.

Recommended WF:

- Use TRP as performance metic

- OOB boundary

  - Option 1: D2R CBW

  - Option 2: D2R channel bandwidth plus minimum D2R channel bandwidth

  - Option 3: max of 500kHz and 10 times NB where NB equals to D2R CBW

- CBW: only consider with min CBW

- Side condition

  - Emission requirement is only tested for no small frequency shift mode

  - Input CW level: max input level with [1m] or [MCL] assumption, use the same input CW level as backscattering loss.

- Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement - Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies - Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory - At least to define maximum gain direction sensitivity requirements - FFS on how to different sensitivity requirements on different orientations

Proposal 1 (Huawei):

- Consider to define the minimum sensitivity requirement for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional sensitivity levels can be added in the future if needed.

Table 3: Device Sensitivity at peak antenna gain direction

| Sensitivity Level | Minimum requirement (dBm) |
| --- | --- |
| L1 | ≤ [-33~-35] |

- Consider to define the minimum EIS spherical coverage requirements for the typical D1T1 indoor factory deployment scenario in Rel-19. Additional coverage levels can be added in the future if needed.

Table 4: EIS spherical coverage

| Spherical Coverage Level | Minimum requirement |
| --- | --- |
| E1 | ≤ X dBm at Y%-tile CCDF, X and Y are TBD |

- The AIoT paging and CFRA procedure can be used to evaluate the receiver sensitivity with success rate target of >=[90%]. The detailed parameters of the PRDCH and PDRCH (i.e. FRC) are TBD.

Proposal 2 (ZTE):

- define REFSENS requirement as -34dBm

- propose to consider the following R2D FRC as starting point for further discussion.

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

Proposal 3 (Ericsson):

- Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design

- Depending the maximum BS transmission power, REFSENS can be either -26 dBm or -20 dBm

- Manufacture should declare a better REFSENS than min REFSENS derived with link budget

- Averaging EIS can be specified instead of the maximum EIS if a dipole antenna is used in device

Proposal 4 (Sony):

- It is proposed that the REFSENS requirement of the AIoT devices with -40 dBm TIS or -42 dBm EIS be defined in the peak direction for all R2D BW

- RAN4 adopts 10% MDR as the metric to test REFSENS requirement for AIoT devices.

Proposal 5 (Vivo):

- The REFESNS of device 1 is defined as -30 dBm

- No need to define sensitivity requirements in different orientation.

Proposal 5 (Spreadtrum, UNISOC): The reference sensitivity can be -29dBm

Proposal 6 (Qualcomm):

- Ensure with the process for testing REFSENS that device decodes the R2D correctly

- 10% false alarm rate and 10% miss detection rate should be included in the REFSENS test procedure

- Assume the device under test in a known device in REFSENS testing process

- Use known pagingID and CFRA in REFSENS test procedure

- REFSENS is met with lowest CW power level

Proposal 7 (xiaomi):

- Device REFSENS can be defined with the minimum requirement [-23 ~ -30] dBm

- Use the same approach with Tx power to define the requirements for different directions

Proposal 8 (CMCC):

- REFSENSE is suggested as -30dBm

- define spherical coverage like Rx requirements

Recommended WF:

- The REFSENS values proposed are in the range of  [-42dBm~-20dBm], consider using [-34dB] as starting point to define REFSENS requirements

- Define orientation requirements considering following options:

  - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage

  - Option 2: only EIS spherical coverage requirements

  - Option 3: averaging EIS

  - Option 4: peak EIS in the peak direction

- Testing metric: 10% false alarm rate and 10% miss detection rate

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

- Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement in RAN4#114: - Define maximum input level, consider following options: - Option 1: using [1m] distance assumption from reader to device - Option 2: using square law appliance range - Option 3: using dynamic range of device receiver

Proposal 1 (Huawei): set the maximum input level to [-5 ~-3] dBm for AIoT devices.

Proposal 2 (Ericsson): 6.5 dBm should be specified as maximum input power, 38 – 31.5(1m) =6.5 dBm

Proposal 3 (Sony):

- The maximum input level of R2D needs to be defined to ensure the RF-ED receiver operates in the square-law region of the diode and that the signal can be self-mixed to the DC.

- Consider a maximum input power level in the range between -20 and -10 dBm as starting point.

  - typical square law region for a Schottky diode is below -20 dBm

Proposal 4 (vivo): The maximum input power of device is -20 dBm

Proposal 5 (Spreadtrum, UNISOC): Using dynamic range of device receiver to define maximum input level. The maximum input level can be -2dBm.

Proposal 6 (Qualcomm): Define a range for CW input power for the device similarly as max input level

Proposal 7 (CATT): The MCL should be discussed first to check if maximum input power requirement is needed. If the MCL needs to be determined, the value of MCL in TR 38.769 could be considered

Proposal 8 (xiaomi): 0 dBm is defined for the maximum input power for device 1

Proposal 9 (CMCC): [0]dBm max input power with 30dB dynamic range assumption

Recommended WF:

- Define maximum input level at peak antenna direction, consider following options:

  - Option 1: using [1m] distance assumption from reader to device, i.e. 6.5dBm assuming 38dBm Tx power

  - Option 2: using square law appliance range, in the range from -20 to -10dBm

  - Option 3: using dynamic range of device receiver, e.g. 30dB

Issue 3-5-2: Occupied bandwidth

Agreement in RAN4#114bis: - Further discuss whether to define the occupied bandwidth for device 1.

Proposal 1 (Huawei):  Consider not to define OBW requirement for backscattering devices

Proposal 2 (Ericsson): Specify the occupied bandwidth test to test the SFO imperfection.

Proposal 3 (vivo): The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

Proposal 4 (Spreadtrum, UNISOC): No need to define occupied bandwidth.

Proposal 5 (xiaomi): OBW requirement for device 1 is not defined if there’s no regulation request

Proposal 6 (CMCC): no need for occupied bandwidth requirements

Recommended WF:

Do not define occupied bandwidth requirements.

Issue 3-5-3: RF requirements within D2R CBW

Proposal 1 (CMCC): no needs for RF requirements within D2R CBW.

Recommended WF:

No need for RF requirements within D2R CBW

OTA test method

- Topic 4-1: General

Issue 4-1-1: testing frequencies

Agreement: - Only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies - Check whether three frequencies are applicable for all requirements.

Proposal 1 (CMCC): list all low, middle and high frequency ranges as general part in TR 38.870 and leave RAN5 to determine detailed testing frequency range case by case for each RF requirements

Proposal 2 (Vivo): Take the following table as the starting point of test parameter

Table 1: Tx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (kHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK/BPSK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Table 2: Rx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (MHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Recommended WF:

List all low, middle and high frequency ranges as general part in TR 38.870 and leave RAN5 to determine detailed testing frequency range case by case for each RF requirements

Wait for the conclusion of core part requirements for detailed measurement parameters.

Issue 4-1-2: Others

Proposal 1 (Huawei): The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

Recommended WF:

The OTA tests for other requirements can be conducted based on the baseline methodology for sensitivity and backscattering. The detailed updating/simplification for specific requirements can be discussed in conformance requirements.

- Topic 4-2: Test setup

Issue 4-2-1: testing antenna and CW antenna relationship

Agreement: Whole sphere scan is supported The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion). - FFS on relative positions between reader antenna and CW

Proposal 1 (Huawei): Consider following CW positions:

- Option 1: CW antenna is fixed relative to reader with 25 degree offset

- Option 2: CW antenna is fixed relative to DUT (e.g. always pointing to the maximum gain direction of DUT).

Proposal 2 (Ericsson): relative position of the reader antenna and the CWT node with ∆∅=∅R-∅CW∈[-180°, 180°] and ∆θ=θR-θCW∈[-180°, 180°].

- Device should declare whether it has a symmetric radiation pattern

- If the device has a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0] or ∆∅=[0, 30°, 60°, 90°], ∆θ=0

- If the device does not have a symmetric radiation pattern, ∆∅ =[-90°, -60°,-30°,0, 30°, 60°, 90°], ∆θ=0

Proposal 3 (CMCC): it’s suggested to use figure 7.2-1 as the baseline with fixed located testing antenna and CW antenna and 3D-rotated devices. There is no specified relative positions between testing and CW antenna as long as target isolation is achieved by isolation materials.

Proposal 4 (Qualcomm): To reuse the OTA test system for legacy TRP and TRS measurement, only one fixed separation between the reader antenna and CW node should be implemented. The separation value is FFS. RAN4 to check whether the link antenna in Figure 1 and Figure 2 can be used to transmit CW signal.\

Proposal 5 (Sony): RAN4 shall clarify if the CW antenna is fixed with respect to the AIoT devices or if it is fixed with respect to the test system.

Proposal 6 (vivo): No need to define the relative positions between measurement antenna and CW source.

Recommended WF：

Discuss with following options

- Option 1: CW fixed relative to reader antenna, i.e. fixed separation between reader antenna and CW

- Option 2: CW fixed relative to devices

- Option 3: relative position of the reader and CW with multiple values of ∆∅=∅R-∅CW∈[-180°, 180°] and ∆θ=θR-θCW∈[-180°, 180°].

- Option 4: no need to define the relative positions between CW and reader antenna

Issue 4-2-2: how to avoid the isolation between CW node and measurement antenna

Agreement: - Further discuss power difference between backscatter signal and the CW signal leakage, e.g. 6dB, 12dB, etc. - Encourage TE vendors provide the feasibility of TE extracting D2R power when CW source is transmitting.

Proposal 1 (Huawei): The CW isolation condition is defined as 50-60 dB, and CW interference cancellation is up to TE implementation.

Proposal 2 (Ericsson): The difference between backscattered signal and “residual” CW leakage should be higher than [12]dB

Proposal 3 (CMCC):

- use 6dB as the baseline for power difference between backscatter signal and CW leakage. In addition to, if final isolaiton is less than 50dB, we can use 50dB in stead as used in RFID spec, i.e. 50dB and lower bound for isolation.

- add follow testing step to ensure target isolation from CW node to testing antenna as below:

  - Step 2: set the [rated] transmit power for CW signal. Measure and recode the received power from CW node and calculate the isolation at testing antenna. If the isolation meet target value, then goto step 3, otherwise update isolation material to ensure target isolation.

Proposal 4 (Qualcomm): It is proposed to take [17dB] as the starting point for the isolation between CW node and measurement antenna

Proposal 5 (Sony): The minimum offset between the CW and testing antenna depends on the acceptable coupling level between the CW and testing antenna, so further input from TE vendors may be needed here

Proposal 6 (vivo): The isolation between CW and measurement antenna should ensure the CW leakage is at least 15dB lower than the D2R

Recommended WF：

- FFS whether CW interference cancellation is up to TE implementation and whether to define minimum cancellation capability.

  - Note: may need to wait for the conclusion of core RF requirements, for example about how to treat CW phase noise

- candidate values of power difference between backscatter signal and the CW signal leakage is   6, 12, 15dB, further downselect to one

  - Further discuss the isolation between CW and measurement antenna based on above target power difference

  - Further discuss whether need to verify such power difference during testing procedure, i.e. add extra step to verify such power difference

Issue 4-2-3: 3D scan and measurement grid

Agreement: - For TRP, TRS, EIRP and EIS requirements (depending on core requirements) measurement grid: - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point.

Proposal 1 (Qualcomm):

- If EIRP is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

- If EIS is selected as the performance metric for backscattering loss testing, the test direction should be declared by device to avoid the 3D scan to find the peak/best direction.

Recommended WF：

Wait for the core requirement conclusion which will discuss whether to use peak direction/ xth percentage/ average value.

Issue 4-2-3: Polarization of CW antenna

Proposal 1 (Sony): The CW signal shall be sent in two polarized sequentially, and backscattered loss shall be defined as the averaged value between the two polarizations

Recommended WF：

Wait for the conclusion of backscatter loss requirements.

- Topic 4-3: Test procedure

Issue 4-3-1: testing side condition of R2D reference sensitivity

Agreement: - Use following as starting point: - Measuring device’s correct response and correct respond timing relationship between R2D and the corresponding D2R transmission with a minimum mean power applied for device antenna. - FFS on how to set the side condition including CW signal power.

Proposal 1 (Ericsson):

- The device should declare its CW sensitivity, i.e. the CW threshold at which the device is capable to backscatter.

- When testing for R2D sensitivity, the received CW signal at the device antenna should be at least 3dB higher than its CW sensitivity.

Proposal 2 (vivo):  Same CW condition for device Tx requirement verification is applied to sensitivity test.

Proposal 3 (Huawei): The CW signal level is set such that the power at DUT’s antenna is R2D signal power + X dB

Recommended WF:

Further discuss with following options:

- Option 1: the received CW signal at the device antenna should be at least 3dB higher than its CW sensitivity which is based on declaration

- Option 2: Same CW condition for device Tx requirement verification is applied to sensitivity test

- Option 3: The CW signal level is set such that the power at DUT‘s antenna is R2D signal power + X dB

Issue 4-3-2: Test procedure for receiver RF requirements

Agreement: - Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc. - Step 2: Place DUT based on UE positioning guidelines. - Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions. - Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level. - Step 5: Determine whether DUT can send the correct response in D2R channel within timing window, e.g., TR2D_min - Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid. - Step 7: FFS on performance metric. FFS on how to specify the performance metric.

Proposal 1 (HW): TP for test procedures for receiver requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The transmit power of R2D signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device.

  - The CW signal level is set such that the power at DUT’s antenna is R2D signal power + X dB, and CW signal is transmitted continuously within the time period, during which A-IoT device is expected to response

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship.

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The test result for each measurement grid depends on performance metric defined in core requirements. (e.g. PASS/Not or sensitivity value for each grid).

Note 2: When correct response is not detected for one measure grid, whether to increase the transmit power of signal generator to repeat the test for the same grid depends on performance metric defined in core requirements.

Note 3: X could be different value for different requirements (e.g. negative value for maximum input power).

Proposal 2 (Qualcomm):  The Step 4- setup 7 of test procedure of reference sensitivity for A-IoT device 1 can be modifed as below:

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. TE send A-IoT Paging with Paing ID in CFRA mode. The transmit power of signal generator shall be set as that the received power at DUT’s antenna is equal to minimum reference sensitivity requirement of device. FSS on the CW signal level.

- Step 5: Determine whether TE can receive the correct response in D2R channel within timing window, Tmin and Tmax.

- Step 6: Repeat Step 3-5 until complete all the tests.

- Step 7: Determine whether the device can pass the REFSENS requirements such as 90 % success rate and 10% miss detection rate.

Proposal 3 (vivo): The REFSENS is tested at EIRP max direction and no need to perform the EIS peak search.

Recommended WF:

Wait for the conclusion of issue 4-3-1 and conclusion of REFSENSE core  requirements which will include the discuss of test side condition.

Issue 4-3-3: Test procedure for Tx RF requirements

Agreement: - Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc. - Step 2: Place DUT based on UE positioning guidelines. - Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions. - Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB” - Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1 - Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid. - Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP) - Step 8: FFS on performance metric. FFS on how to specify the performance metric.

Proposal 1 (HW): TP for test procedures for backscatter requirements

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal.

  - The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

  - The CW signal level is set as TBD at DUT’s antenna

- Step 5: Determine whether DUT can send the correct response in D2R channel with correct timing relationship. If the correct response is received, measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 4-5 until complete all the testing point in the measurement grid.

Note 1: The performance metric depends on core requirements discussion.

Note 2: Candidate values of X can be 10 dB which depends on core requirements discussion (e.g. CW conditions)

Proposal 2 (Qualcomm): In Step 4 of test procedure for backscattering loss, the test configuration for input power level of CW signal should be decided together with core requirement definition for backscattering loss. Additionally, the power level of R2D should be set as sensitivity+[X]dB. Take X = [3] as the starting point.

Proposal 3 (OPPO): Two step method can be used to test the backscattering loss.

- The RX power of CW can be a calibration step before the test for the device begin.

- To consider the calibration of CW RX power with the 20cm distance of CW and device.

- To use the calibration result of CW RX power minus the TX output power of device as the backscattering loss of device.

Recommended WF:

Further discuss whether to update current procedure with above proposal 3.

- Topic 4-4: Energy source and energy harvesting

Issue 4-4-1: energy source and energy harvesting

Agreement: - Device declaration on the required energy conditions. - External energy source directly connected to the device with cable and/or battery is excluded.

Proposal 1 (Ericsson):

- The device should declare:

  - If it can harvest energy in parallel of performing R2D reception and/or D2R transmission.

  - If not, charging time and discharging time for both R2D reception and D2R transmission

- Device testing should consider the device unavailability time due to charging time

Proposal 2 (Qualcomm): From AIoT testing PoV, at least the time to complete charging, and the duration the device can remain ON after charging should be declared. FFS on other needed device declaration information

Proposal 3 (Sony): The UE charging time duration and periodicity may need to be declared in the OTA test

Recommended WF:

Further discuss whether device need to declare following or not

- D1:  The device should declare

  - If it can harvest energy in parallel of performing R2D reception and/or D2R transmission.

  - If not, charging time and discharging time for both R2D reception and D2R transmission

- D2: the time to complete charging, charging duration and charing periodicity

- Topic 4-5: MU

Issue 4-5-1:MU assessment

Agreement: - Use following as baseline for further discussion. UID; Description of uncertainty contribution; Details in clause. Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2). 1; Mismatch of transmitter chain; Reused. 2; Insertion loss of transmitter chain; Reused. 3; Influence of the measurement antenna cable; Reused. 4; CW for backscatter measurement or R2D for sensitivity measurement: uncertainty of the absolute output level; FFS. 5; Sensitivity measurement: output level step resolution; Reused. 6; Measurement distance; Reused. 7; Quality of quiet zone; Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length.. 8; DUT sensitivity drift; FFS. 9; Uncertainty related to the use of phantoms; No needs. 10; Coarse sampling grid; - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point. 11; Random uncertainty; Reused. 12; Frequency Response; Reused. Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1). 13; Uncertainty of network analyzer; Reused. 14; Mismatch of transmitter chain; Reused. 15; Insertion loss of transmitter chain; Reused. 16; Mismatch in the connection of calibration antenna; Reused. 17; Influence of the calibration antenna feed cable; Reused. 18; Influence of the measurement antenna cable; Reused. 19; Uncertainty of the absolute gain/radiation efficiency of the calibration antenna; Reused. 20; Measurement distance; Reused. 21; Quality of quiet zone; Reuse the same QZ size and minimum range length as in 38.870. i.e. 30cm QZ size and 90cm minimum range length.. Systematic Errors. 22; Systematic Error related to TRS grids; Reused.

Proposal 1 (Huawei): adopt the MU Table 1 and Table 2 in R4-2505395 as baseline

- In Table 1, a value of 0.65dB is entered for UID 4 with its associated description modified. The value of 0.65dB comes from UID 4 in TRS MU Table B.5.1-2 in TR 38.870. UID 8 is removed as CW power reaching DUT should be well above DUT sensitivity.

- In Table 2, a value of 0.27dB is entered for UID 4 with the corresponding changes in its description. The value of 0.27dB comes from UID 4 in TRP MU Table B.4.2-2 in TR 38.870. A value of 0.12dB is entered for UID 8. The value of 0.12dB comes from entry 8 in TRS MU Table B.5.1-2 in TR 38.870.

Proposal 2 (CMCC):

- last meeting agreement for MU is for TRS not for TRP. Certain sources of MU are different for TRP and TRS.

- the MU table for TRP is listed in R4-2505620

- the MU of measurement distance source is the same as hand only mode considering in last meeting there is no

Recommended WF:

- More clarification of last meeting agreements

  - Further check whether last meeting agreement of MU is only for TRS. For TRP using following table as baseline

  - the MU of measurement distance source using the same value as hand only mode

Table 1 MU for Tx requirements

| UID | Description of uncertainty contribution | Reuse or not |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of receiver chain | reused |
| 2 | Insertion loss of receiver chain | reused |
| 3 | Influence of the measurement antenna cable | reused |
| 4 | Measurement Receiver: uncertainty of the absolute level | FFS |
| 5 | Measurement distance | reused |
| 6 | Quality of quiet zone | reused |
| 7 | DUT Tx-power drift | FFS |
| 8 | Uncertainty related to the use of phantoms | No needs |
| 9 | Coarse sampling grid | - For TRP and EIRP, use 30 degree as starting point - For TRS and EIS, use 45 degree as starting point |
| 10 | Random uncertainty | reused |
| 11 | Frequency Response | reused |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | reused |
| 13 | Mismatch of receiver chain | reused |
| 14 | Insertion loss of receiver chain | reused |
| 15 | Mismatch in the connection of calibration antenna | reused |
| 16 | Influence of the calibration antenna feed cable | reused |
| 17 | Influence of the measurement antenna cable | reused |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | reused |
| 19 | Measurement distance | reused |
| 20 | Quality of the Quiet Zone | reused |
| Systematic Errors |  |  |
| 21 | Systematic Error related to TRP grids | reused |