3GPP TSG-RAN WG4 Meeting #116						R4-2509008

Bengaluru, India, 25th – 29th August 2025

3GPP TSG RAN WG1 #121							     R1-2504839

St Julian's, Malta, May 19th – 23rd, 2025

Title:	LS on maximum transmission power for STxMP

Release:	Release 18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

Name:	 Fanbo

E-mail Address: asen.fanbo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

In 38.101-2 (Table 1), the maximum transmission power PCMAX,f,c,k for STxMP is defined, with k=0 for the first indicated TCI state and k=1 for the second indicated TCI state. While, according to 38.213 (Table 2), PCMAX,f,c,k is applied for the k-th TCI state, which implies k=1 for the first indicated TCI state and k=2 for the second indicated TCI state. In addition, in 38.321 (Table 3), it is also assumed k=1 for the first indicated TCI state and k=2 for the second indicated TCI state. In other words, The description of PCMAX,f,c,k in RAN1 and RAN2 specifications are aligned, but not aligned with the description of PCMAX,f,c,k in RAN4 specification.

Table 1. Definition of maximum transmission power for STxMP in 38.101-2.

### 6.2K.4 Configured transmitted power for simultaneous transmission to multiple directions A UE configured for simultaneous transmission to multiple directions can configure two maximum output powers. The configured UE maximum output power PCMAX,f,c,k for each of joint/UL TCI states k (k=0,1) indicated for simultaneous transmission to multiple directions of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax

Table 2. Description of maximum transmission power for STxMP in 38.213

### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l - if the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is configured with multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel, and the UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in PUSCH transmission occasion i, the UE determines the PUSCH transmission power PPUSCH,b,f,c, k(i,j,qd,l) for the k-th indicated TCI-State or TCI-UL-State as PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] - else, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] where, - PCMAX,f,c,k(i) is the UE configured maximum output power for the k-th indicated TCI-State or TCI-UL-State defined in [8-2, TS 38.101-2] for carrier f of serving cell c in PUSCH transmission occasion i . - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5] for carrier f of serving cell c in PUSCH transmission occasion i.

Table 3. Description of maximum transmission power for STxMP in 38.321

#### 6.1.3.81 Enhanced Single Entry PHR for multiple TRP STx2P MAC CE The Enhanced Single Entry PHR for multiple TRP STx2P MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-2b. The two PHs together with two PCMAX,f,c,k for the Serving Cell are reported if UE is configured with twoPHRMode and multipanelSchemeSDM or multipanelSchemeSFN, as specified in clause 5.4.6. It has a fixed size and consists of four octets defined as follows (Figure 6.1.3.81-1): - R: Reserved bit, set to 0; - Power Headroom k (PH k): This field indicates the power headroom level for k = 1, 2, where PH 1 is associated with the first TCI-State or TCI-UL-State for a real or reference PUSCH transmission and PH 2 is associated with the second TCI-State or TCI-UL-State for a real or reference PUSCH transmission, as specified in TS 38.213 clause 7.7.1 [6]. PH fields for a Serving Cell are included in ascending order based on k. The length of the field is 6 bits. The reported PH and the corresponding power headroom levels are shown in Table 6.1.3.8-1 (the corresponding measured values in dB are specified in TS 38.133 [11]);

RAN1 has discussed the above issue. Instead of changing both RAN1 and RAN2 specification, RAN1 thinks it may be simper to update RAN4 spec, so that RAN1, RAN2 and RAN4 specifications are aligned.

2. Actions:

To RAN4

ACTION:  RAN1 respectfully suggests RAN4 to change the value range of k in PCMAX,f,c,k to {1, 2}.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #122	25th – 29th August 2025		India, IN

TSG RAN WG1 Meeting #122-bis	13th– 17rd October 2025	Czech, CZ