3GPP TSG-RAN WG4 #116	R4-2509031

Bengaluru, India, August 25-29, 2025

Agenda item:	7.20.4.1		Enhancement for UE-initiated/Event-driven Beam Management

Source:	Qualcomm Incorporated

Title:	Additional core requirements for UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 MIMO Phase 5 was started. Several agreements have been made for UI-initiated/event-driven beam management, and additional open issues have been identified in [1] in RAN4 #115

In this paper we provide our view for the open issues identified in the way forward in [1] in RAN4 #115.

# Discussion

In RAN4 #115 good progress was made on the definition of the measurement period in event triggered measurement reporting. It was agreed that RAN4 will define separate requirements for the case when timeRestrictionForChannelMeasurement is configured or not. An LS has been sent to RAN1 to check whether RAN1 sees any issue in this agreement [2]. In addition, it was agreed that in case the measurement periods are different for current beam and new beam, TL1_measurement period is not larger than the maximum of the L1 measurement periods.

The main open point for the measurement period is whether to specify requirements both for the cases where no time window and that a time window is defined (Issue 1-3).

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

In RAN1 #120bis, RAN1 made the agreement that the UE starts a timer for the new beam when event-2 is obtained in case the timer for the new beam is not yet running. In RAN1 #121, the details of the time window have been defined. RAN1 agreed that the time window eventDetectionTimeWindowLength-r19 has a length between 4 ms and 1280 ms and that the counting factor eventInstanceCount-r19 takes on values between 2 and 16.

RAN1 #120bis Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

RAN1 #121 Agreement

Regarding triggering event determination, on candidate values of supported RRC parameters,

- Support the following as RRC candidate values for a threshold value eventThreshold-r19 for trigger event detection regarding Event-2 or Event-7.

  - Option-2: 0, 1, …, 30, 31 dB

- Support the following as RRC candidate values for a threshold value eventThreshold-Event1-r19 for trigger event detection regarding Event-1.

  - Reusing RSRP-Range in RRC

    - Note: only values 14, …,113 in RSRP-Range are valid

- Support the following as RRC candidate values for the time window length for triggering event determination eventDetectionTimeWindowLength-r19

  - 4, 5, 8, 10, 16, 20, 40, 80, 160, 320, 640, 1280 ms

- Support the following as RRC candidate values for the counting threshold eventInstanceCount-r19

  - Option-1: 2, …, 16

Since RAN4 has agreed that the existing beam sweeping factor N is reused for UE-initiated/Event-driven beam management, the window-based approach needs to take the beam sweeping factor N, the factor P and the factor M Î {1, 3} into account in case it is decided that requirements in RAN4 are defined. It is still unclear from this agreement in Option 4 how the RAN4 measurement procedure fits together with the agreement in RAN1. According to RAN4 agreements, the measurement period is M*P*N*Trs with N = 8 and P ≥ 1and M ≥ 1. Assuming Trs = 20 ms, the measurement period in RAN4 is at least 160 ms if no window is defined. That means for many values of eventDetectionTimeWindowLength-r19, no requirements can be defined by RAN4. Based on this unclarity how existing RAN4 agreements fit together with the RAN1 agreements, RAN4 should discuss whether a LS is sent to RAN1 seeking further clarity. In our opinion the existing measurement procedure in RAN4, mainly due to Rx beam refinement, is a sliding window-based approach while the RAN1 agreement seems rather to define a fixed window-based approach.

Proposal 1: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement.

However, it has been agreed in RAN4 #113 that the UE does not do any filtering of L1-RSRP measurements if a configurable time window is applied. If no filtering is applied, it is not obvious whether additional reporting delay requirements for the configurable time window needs to be defined. The measurement would simply be extended by a factor based on the configured value M. Hence, we do not see that defining additional requirements tests much more functionality of the UE.

Observation 1: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 2: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

The next open point is Issue 1-5 whether to define new unified switching delay requirements.

#### Issue 1-5: Whether to specify new unified TCI switching delay requirements?

Agreement:

- The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management

  - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion.

Whether latency reduction for TCI switching can be achieved was controversially discussed in RAN1 #120bis and no conclusion could be reached. No further discussion took place in RAN1 #120. Hence, RAN1 does not have any conclusion on this topic. The RAN4 requirement for MAC CE based TCI state switching for a known TCI state is that the switch occurs in slot n + THARQ + 3Nslot + TOk (Tfirst_SSB + TSSB-proc). TOk = 1 if the target TCI state is not in the active TCI state list, other TOk = 0. For an unknown TCI state, additional term TL1-RSRP is added to the switching delay. The condition for a known TCI state is that a measurement report has been sent for the target TCI state and the TCI switch command is received within the last 1280 ms since the last transmission of the RS resource for this measurement. Since the device has sent an event-triggered measurement report and there is no reason for the network to delay the TCI state switch, it should be assumed that the network switches beams within 1280 ms. Hence, the delay reduction should not address TL1-RSRP but Tfirst_SSB + TSSB-proc. But the term Tfirst_SSB + TSSB-proc does not appear in the requirement in case TOk = 0. Hence, the network has already a possibility to reduce the switching delay by ensuring that TOk = 0. The condition is that the target beam is already in the active TCI state list. In our opinion it is an unnecessary optimization to introduce reduced TCI state switching latency for beams that are not in the active TCI state list. Instead, it should be ensured by the network that the target TCI state is already in the active TCI state list.

Observation 2: Any discussion about enhancing the TCI state switch latency should assume known TCI states and focus on removing Tfirst_SSB + TSSB-proc from the minimum requirement.

Observation 3: Removal of the term Tfirst_SSB + TSSB-proc from the minimum requirement can already be ensured by the network if it is ensured that the target TCI state is already in the active TCI state list.

Proposal 3: RAN4 should not introduce reduced TCI state switch requirements in Rel-19 for UE-initiated/event-driven beam management.

The next open point is Issue 1-6 whether to define requirements if the indicated TCI state changes during the measurement.

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

<Way forward >

- Option 1: (Nokia, Ericsson, MTK)

  - No need to capture this scenario in RAN4 requirements

- Option 2: (Xiaomi, Samsung)

  - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart.

- Option 3: (Apple)

  - Add the applicability rule of event triggered L1 measurement:

    - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE.

    - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

The case that the TCI state changes during the measurement time can happen in a real field deployment, but it depends very much on the true measurement time of the device. We do not see how we can define a meaningful test for such a scenario in RAN4. For the case that the device has a shorter measurement time than the minimum requirement it is difficult to define a test case that verifies this behavior. It should be left to implementation to reset the measurement in case the TCI state changes.

Observation 4: It is difficult to define a meaningful test case for a TCI state change during measurement time in case the device has a measurement period that is significantly lower than the RAN4 requirements.

Proposal 4: It should be left to implementation to reset the measurement in case the TCI state changes during measurement period.

In Issue 1-8 it is discussed whether a UE capability for the support of event triggering reporting criteria should be introduced.

#### Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

- Option 1: (Samsung, Apple, Huawei, Ericsson)

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2: (Nokia, MTK)

  - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

It is not clear to us why such a new capability is needed. L1-RSRP is part of channel state feedback and RAN1 has already a framework for the number of supported CSI calculations per component carrier and across all component carriers. In RAN1 #121 it was decided that the occupied CSI reporting units is OCPU = 1 for the beam report transmission procedure for UE-initiated/event-driven beam reporting. This is the same number of occupied CSI processing units as for legacy L1-RSRP measurement. Hence, we believe that the RAN1 framework fully covers the control of the UE load and no additional UE capability in RAN4 is needed.

Observation 5: L1-RSRP is part of channel state feedback and RAN1 has already a framework for the number of supported CSI calculations per component carrier and across all component carriers. RAN1 framework fully covers the control of the UE load.

Proposal 5: No additional UE capability for support of event triggering and reporting criteria is needed in RAN4.

Proposal 6: Since we propose that no UE capability is introduced for event triggering and reporting criteria, no discussion is needed whether the UE capabilities are commonly or separately defined for Rel-19 and Rel-19 MIMO.

#### Issue 1-9: Compatibility of capabilities with LTM

<Way forward >

- If such capabilities are defined in Issue 1-8, then discuss on:

- Option 1:

  - The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO.

- Option 2:

  - The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement.

Observation 1: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 2: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

Observation 2: Any discussion about enhancing the TCI state switch latency should assume known TCI states and focus on removing Tfirst_SSB + TSSB-proc from the minimum requirement.

Observation 3: Removal of the term Tfirst_SSB + TSSB-proc from the minimum requirement can already be ensured by the network if it is ensured that the target TCI state is already in the active TCI state list.

Proposal 3: RAN4 should not introduce reduced TCI state switch requirements in Rel-19 for UE-initiated/event-driven beam management.

Observation 4: It is difficult to define a meaningful test case for a TCI state change during measurement time in case the device has a measurement period that is significantly lower than the RAN4 requirements.

Proposal 4: It should be left to implementation to reset the measurement in case the TCI state changes during measurement period.

Observation 5: L1-RSRP is part of channel state feedback and RAN1 has already a framework for the number of supported CSI calculations per component carrier and across all component carriers. RAN1 framework fully covers the control of the UE load.

Proposal 5: No additional UE capability for support of event triggering and reporting criteria is needed in RAN4.

Proposal 6: Since we propose that no UE capability is introduced for event triggering and reporting criteria, no discussion is needed whether the UE capabilities are commonly or separately defined for Rel-19 and Rel-19 MIMO.

# References

- R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #115, May 2025

- R4-2508391, LS on event-triggered L1-RSRP reporting, RAN4 #115, May 2025