3GPP TSG-RAN WG4 Meeting # 116											                 R4-2509044

Bengaluru, India, 25 - 29 August 2025

Agenda item:			4.4.1

Source:	Moderator (Xiaomi)

Title:	Topic summary for [116][201] Maintenance_up_to_R16

Document for:	Information

Introduction

This document provides summary for RRM related Tdocs submitted to the following AI

4.4.1		Up to Rel-16 maintenance 	[WI code]

Recommended issues to online discussion:

- Sub-topic 1-1

- Sub-topic 1-2

- Sub-topic 2-1

Topic #1: Timing adjustment for CA and DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509468 | Apple | - Defining a behavior based on the condition of meeting a certain requirement is not feasible. - The behavior defined should not restrict UEs that can handle TTD>MTTD. - The UE shall be allowed to continue adjustment if it doesn’t cause UE TX EVM degradation. - If UE stops transmission, NW might not know the reason and assume its link failure. - In case UE stops transmission, UE shall send an indication to NW via signaling on the UL via PCell or other TAG. - Introducing signaling for this might not be feasible from earlier release. - A more practical solution could be to introduce UE behavior to stop adjustment when TTD exceeds the limit. - In case TTD exceeds MTTD during gradual timing adjustment on any TAG, UE shall continue gradual timing adjustment unless it impacts UE TX EVM, otherwise UE shall stop gradual timing adjustment on that TAG. - In case TTD exceeds MTTD after receiving new TA command, UE may apply the new TA command if UE TX EVM is not degraded, otherwise UE shall stop transmission on SCell or PSCell. - UE shall stop transmission when TAC is received/ stop gradual timing adjustment if TTD exceeds MTTD in the following cases: Inter-band CA Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation Intra-band EN-DC if UE doesn’t support asynchronous operation Inter-band NE-DC if UE doesn’t support asynchronous operation Inter-band NR-DC if UE only supports synchronous operation Proposal #6: Further discuss applicable release for the clarification to UE behavior with TTD exceeds a limit. |

Open issues summary

Sub-topic 1-1: UE behavior when TTD > MTTD

- Proposals

  - Option 1 (Apple CR R4-2509469):

    - In any of the configurations if such adjustment results in transmit timing difference larger than the maximum timing difference in section 7.5 the gradual timing adjustment is applied if UE TX EVM is not degraded, otherwise UE shall stop gradual timing adjustment.

- 	Inter-band CA
-	Intra-band EN-DC if UE doesn’t support asynchronous operation
-	Inter-band NR-DC

    - For the cases where the UE shall be capable of handling the maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1, if the UE receives a TA command that causes uplink transmission timing difference to exceed the value in Table 7.5.2.1-1, the UE can apply the TA command if UE TX EVM is not degraded, otherwise the UE shall stop UL transmission on PSCell.

- Recommended WF

  - Further discussion.

Sub-topic 1-2: Applicable release for updated Timing adjustment requirement

- Proposals

  - Option 1 (Apple):

    - Further discuss applicable release for the clarification to UE behavior with TTD exceeds a limit.

- Recommended WF

  - The issue has existed since Rel-15 and lasts for many years. Suggest to further discuss the impact of introducing changes in Rel-15/16 Vs. Rel-19:

    - Assess backward compatibility risks for deployed UEs if clarified in earlier release.

    - Evaluate possible impact to network (e.g. NW adaptation to new UE behaviours)

Topic #2: CSSF SA mode requirements for outside gaps in R15

Companies’ contributions summary

| R4-2510549 | Huawei, HiSilicon | Proposal 1: The CSSF requirements of NE-DC and SA operation should be introduced in separate sections, and NE-DC and SA operations do not share the same CSSF table or requirements. |
| --- | --- | --- |

Open issues summary

Sub-topic 2-1: CSSF SA mode requirements for outside gaps in R15

- Proposals

  - Option 1 (Huawei):

    - Proposal 1: The CSSF requirements of NE-DC and SA operation should be introduced in separate sections, and NE-DC and SA operations do not share the same CSSF table or requirements.

- Recommended WF

  - Further discussion.