3GPP TSG-RAN WG4 Meeting #116		    								               R4-2509093

Bengaluru, India, August 25 – 29, 2025

Agenda item:			7.3.1

Source:	Moderator (Apple)

Title:	Topic summary for [116][126] NR_LBCA_Sw

Document for:	Information

# Introduction

This document summarizes the contributions submitted for the general and RF related considerations on the low band carrier aggregation via switching work item (AI 7.3.1 and 7.3.2).

## Summary of company contributions

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509010 | RAN1 | LS on Low NR band carrier aggregation via switching |
| R4-2509202 | Apple | Draft TR 38.768 v1.1.0 Low NR Band Carrier Aggregation via Switching |
| R4-2509248 | CATT | TP for TR 38.768 to modify the general description Align general clause with WID |
| R4-2509249 | CATT | draft CR for TS 38.101-1 to introduce LB-LB CA via switching CR to 38.101-1 #draft v19.2.0 Reason for change: To introduce the RF requirements for LB-LB CA via switching Summary of change: To introduce the operating CA band combinations via switching in clause 5.2A.3. To introduce the Transmit ON/OFF time mask for inter-band CA via switching in clause 6.3A.3.3.7. To introduce the Reference sensitivity power level for Inter-band CA via switching in clause 7.3A.2.5. To introduce Switching time mask and DL RMC for inter-band CA via switching in clause A.8 Clauses affected: 5.2A.3 (New), 6.3A.3.3.7 (New), 7.3A.2.5 (New), A.8 (New) |
| R4-2509965 | Apple | TP to TR38.768 on low NR band aggregation via switching - Change 1: Include 70ms switching period value in all the clauses where the switching period values are captured. - Change 2: Add the applicability of the inter-band carrier aggregation aspects for a UE which supports the low NR band carrier aggregation via switching - Change 3: The configuration aspects related to low NR band carrier aggregation via switching - Change 4: Impact on MSD requirements - Change 5: Impact on ΔRIB,c and ΔTIB,c requirements - Change 6: Aspects related to the RMCs |
| R4-2509246 | CATT | Discussion on LB-LB CA via switching Proposal 1: RAN4 can discuss the following two options and decide how to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching. Option 1: Creating a new table/clause to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching. Option 2: Creating a NOTE in the clause 5.2A.2.1 to indicate which band combinations can support the new feature, i.e. low-low bands CA via switching. Proposal 2: its a low hanging fruit for RAN4 to extend this feature to the other FDD+SDL low-low bands CA combinations in Rel-19, at least for CA_n29-n71 which has been captured in the specification. Proposal 3: according to observation 4, 5 and6, its proposed to specify the minimum requirements for switching gap from UE perspective below. (see contribution for details) Proposal 4: From RAN4s perspective, its recommended for RAN1/RAN2 to specify the maximum number of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 and LBCA-SwitchinggGap-SCelltoPCell-r19 considering the following scenarios based on our calculation about switching gap. Proposal 4-1: maximum value of LBCA-SwitchinggGap-Duration-PCelltoSCell-r19 should be equal to or larger than three OFDM symbols with 15kHz SCS, no matter what the is. Proposal 4-2: For LBCA-SwitchinggGap-SCelltoPCell-r19, the maximum value should be equal to or larger than the following values based on the different maximum cell coverage to be supported. Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. 5-1: LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’. 5-2: Before and/or after switching gaps (PCelltoSCell and SCelltoPCell), 2 symbols Type B PDSCH is scheduled in the DL slots except for SSB slots. 5-3: one symbol PDCCH at the first symbol in each DL slot except for SSB slots. 5-4: The diagram about DL RMC for Low-Low bands inter-band CA via switching is shown in figure 2. |
| R4-2509247 | CATT | Replied LS on Low NR band carrier aggregation via switching |
| R4-2509569 | Apple, Telus | (NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1 CR to 38.101-1 #2898 v19.2.0 Reason for change: The applicable low band CA configuration for the example band combination, the time mask requirements, and MSD applicability are defined in this CR to introduce the feature to the RF specification. Summary of change: Change 1: clarify the applicability of simultaneous Rx/Tx requirements to band combinations which are supported via switching in Clause 5.2A.2 Change 2: introduce the indication of support via switching for the example band combination (CA_n5A-n29A) in Clause 5.5A.3.1 Change 3: define the applicability of the ON/OFF time mask for carrier aggregation via switching in Clause 6.3A.3.3.1 Change 4: define the ON/OFF time mask for the UL transmissions in the new Clause 6.3A.3.3.7. Change 5: provide a reference for the RMCs applicable to REFSENS testing of devices which support low band aggregation via switching Change 6: define the SCS15 FDD and SDL RMCs applicable to REFSENS testing of devices which support low band aggregation via switching Clauses affected: 5.2A.2, 5.2A.2.1, 5.5A.3.0, 5.5A.3.1, 6.3A.3.3.1, 6.3A.3.7(new), 7.3A.2.3, A.3.2.2 |
| R4-2509821 | Xiaomi | View on LB CA with switching Proposal 1: For co-current operation description in UE feature set, propose to follow the WID description: (see contribution for details) Proposal 2: Introduce Uplink Tx on/off time mask on FDD carrier for LB CA switching as following: |
| R4-2510012 | LG Electronics | Discussion on low band carrier aggregation via switching Proposal 1: LS regarding updated switching gap should be sent to RAN1 and RAN2. Proposal 2: it is necceary to change MRTD value in TR38.768 corresponding to our companion Tdoc[4] . |
| R4-2510057 | LG Electronics UK | LS for parameter update request with switching gap |
| R4-2510272 | vivo | Discussion of UE RF impact for LB-LB switching Proposal 1: There is no need to specify Rx part in time mask requirements. Proposal 2: Further discuss whether and how to introduce new RMCs or related requirements. |
| R4-2510316 | Ericsson | Remaining RF requirements for low-band CA via switching Proposal 1: the transmission time masks specify the locations of transient periods for UL slots paired with DL slots adjacent to switching gaps, or containing switching gaps, for the cases UL signals are not dropped and when the network schedules transmissions as expected by the UE. These time masks also specify the location of a switching period (UE capability) and its timing relation to trailing/leading transient periods of the timing advanced UL within the switching gap. |
| R4-2510317 | Ericsson | Draft CR for 38.101.1: time masks for DL-only CA via switching CR to 38.101-1 #draft v19.2.0 Reason for change: Introduce transmission masks for low-band DL CA via switching with the UL configured in one UL band. Summary of change: The transmission time masks specify the locations of transient periods for UL slots paired with DL slots that are adjacent to switching gaps, or containing switching gaps, for the cases UL signals are not dropped and cases for which the network schedules transmissions as expected by the UE. These time masks also specify the location of a switching period (subject to UE capability) and its timing relation to trailing/leading transient periods of the timing advanced UL within the switching gap configured by the NW. Clause 6.3A.3.3.1 (general requirements for inter-band CA): reference to a new clause Clause 6.3A.3.3.7 (new): time masks for the DL introduced for transmissions configured by higher layers assuming NTA = 0 s that defines collision cases for these transmissions (these masks also applicable for scheduled transmissions when NTA = 0 s) scheduled/triggered and overlapping with the switching gap for NTA 0 s. The switching period must occur before the first symbol of the timing advanced UL such that this symbol is not corrupted (transmissions not dropped) For UL slots paired with DL slots not adjacent to or not containing a configured switching gap the requirements in sub-clause 6.3.3 apply. The time masks apply for a receive timing difference between cells of up to MRTD for colocation. Clauses affected: 6.3A.3.3.1, 6.3A.6.6.7 (new) |
| R4-2510322 | Spreadtrum, UNISOC | Remaining issues on UE RF requirements for LB-LB CA switching Proposal 1: Take the Figure 1 as the UL time mask for LB-LB CA switching with non-CA in the UL. |
| R4-2510968 | Qualcomm Incorporated | LB CA Switching Observation 1: UL TA in switching times masks must accommodate RRM discussion conclusions. Observation 2: UL RMCs and DL FRCs are needed for FDD and SDL for this feature. Proposal: UL RMCs and Dl FRS are defined for this with needed RRC parameters that is needed in the configuration of the pattern for the UE and placed in RAN4 specifications |
| R4-2511066 | MediaTek Inc. | draftCR to 38.101-1: Time mask for LB-LB switching CR to 38.101-1 #draft v19.2.0 Reason for change: To complete the Rel-19 time mask requirements for NR_LBCA_Sw-Core Summary of change: Introduce the Rel-19 time mask requirements for low band CA via switching. Clauses affected: 6.3A.3.3 |
| R4-2511072 | Samsung | Views on remaining issues on low band carrier aggregation via switching Proposal 1: It is supported by the proposed clarification, which is not to apply simultaneous Rx/Tx capability for low NR band inter-band configurations supported via switching based on the capability indication. Proposal 2: Figure 2 should be the baseline for further CR discussion while others such as the relationship with DL FDD carrier or switching gap can be captured in the TR as they are already there. |
| R4-2510465 | Huawei, HiSilicon | Discussion on LB-LB switching Proposal 1: The time mask for LB-LB switching shows the switching period location for FDD-SDL switching. Proposal 2: RAN4 capture Figure 1 as the time mask for LB CA via switching. Proposal 3: For Rel-19 sub-1GHz FDD-SDL band combinations, the capability should be separately reported from BandCombinationList for inter-band CA. Proposal 4: The sub-1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration in Rel-19. |
| R4-2510466 | Huawei, HiSilicon | draftCR to 38.101-1: Introduction for LB-LB CA via switching CR to 38.101-1 #draft v19.2.0 Reason for change: The Rel-19 time mask requirements for low band CA via switching are not specified Summary of change: Introduce the Rel-19 time mask requirements for low band CA via switching. Clauses affected: 6.3A.3.3 |
| R4-2510467 | Huawei, HiSilicon | TP to TR38.768 Switching time mask Text proposal on wording description of switching period location and time mask (to illustrate switching period) to TR 38.768 based on the conclusions by RAN4 and the RAN1 |
| R4-2511228 | Nokia | Considerations for UL time mask for NR_LBCA_Sw Proposal 1: UL and its transition period shall be consistently aligned with the switching period and slot boundary in a singular manner for both FDD-SDL and SDL-FDD transition covering all switching periods. Proposal 2: The requirements for the UL time mask should be defined assuming TTA = 0us. Proposal 3: For FDD-SDL transition and TTA = 0us we propose that end of the last symbol of UL aligns with the start of the switching gap. Proposal 4: For SDL-FDD transition and TTA = 0us we propose that the first symbol of the start slot of the UL aligns with end of the switching gap. |

# Topic #1: CA configuration

## Summary of open issues

Issue 1-1: Whether CA configurations which are supported with switching can be captured in the configuration tables

- Option 1: Yes (see Apple R4-2509569/R4-2509965, CATT R4-2509246, Huawei R4-2510465)

  - Proposal 1a: To extend this feature to the other FDD+SDL low-low bands CA combinations in Rel-19, at least for CA_n29-n71 which has been captured in the specification.

- Option 2: No

- Recommended WF: Discuss further

Issue 1-2: Whether the applicability of simultaneous Rx/Tx needs to be clarified for configurations which are supported via switching

- Option 1: Yes (see Apple R4-2509569/R4-2509965, CATT R4-2509246, Huawei R4-2510465, Samsung R4-2511072)

- Option 2: No

- Recommended WF: Discuss further

Issue 1-3: Whether a liaison to RAN2 is useful to clarify the impact of low band switching on BandCombinationList

- Option 1: Yes (see Huawei R4-2510465)

  - See Proposal 3 in the paper and draft LS text in the paper’s annex

- Option 2: No

- Recommended WF: Discuss further

# Topic #2: Tx ON/OFF mask

## Summary of open issues

Issue 2-1: Whether the term “actual switching gap length” should be further refined (currently captured in Clause 5.2.2 of TR38.768)

- Option 1: Yes (see CATT R4-2509246)

  - Observation 4: it’s not necessary to consider SDL transient period when calculating the switching gap.

  - Observation 5: for FDD to SDL switching scenario, whether to include FDD UL trailing end transient period (10µs) depends on whether T_TA is less than the transient period (10µs).

  - Observation 6: from UE minimum requirements’ perspective, the receive timing difference among the closest DL slot timing boundaries (between PCell and SCell) or (between SCell and PCell) could be any value between negative MRTD and Positive MRTD.

- Option 2: No

- Recommended WF: Discuss further

Issue 2-2: Whether RAN4 should provide information to RAN1 to help set the values for LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and LBCA-SwitchingGap-SCelltoPCell-r19

- Option 1: Yes, with updated definition (see CATT R4-2509246)

- Option 2: Yes, with current definition (see LG R4-2510012/R4-2510057)

- Option 3: No

- Recommended WF: Discuss further

Issue 2-3: Whether Tx ON/OFF mask should include collision handling rules agreed by RAN1

- Option 1: Yes (see Ericsson R4-2510316)

- Option 2: No

- Recommended WF: Discuss further

Issue 2-4: Whether non-zero TA should be included in the Tx ON/OFF mask

- Option 1: Depends on RRM discussion (see Qualcomm R4-2510968)

- Option 2: No

- Recommended WF: Discuss further

Issue 2-5: Which time mask figure to use in the specification

- Option 1: Use the figure we previously agreed for the TR (Figure 5.3.2.1-1 in TR38.768)

- Option 1a: Revised version based RAN1 agreement (switching gap always located in switching from carrier) as below from (R4-2509821)

ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL PCell/carrier 1 (from R4-2509821)

  - Xiaomi (R4-2509821), vivo (R4-2510272), Spreadtrum (R4-2510322)

  - Moderator’s note: considering this figure was approved for the TR, we can treat it as the baseline, which we could further update during the meeting in case there is consensus

- Option 2: Use the figure labeled “alt-2” in pre-meeting offline draft (shown below)

  - Samsung (R4-2511072), Huawei (R4-2510465/R4-2510466/R4-2510467), Apple (R4-2509569)

- Option 3: Use the figure which takes collision handling rules into account

  - Ericsson (R4-2510316/R4-2510317)

- Option 4:  Use a figure that aligns UL transient period explicitly with switching periods.

  - Nokia (R4-2511228)

  - Option 5: Time mask proposal for LB CA switching

Figure 6.3A.3.3.7-1: Time mask for low band CA via switching from SDL carrier to FDD carrier, where the switching period is located in SDL carrier

Figure 6.3A.3.3.7-2 Time mask for low band CA via switching from FDD carrier to SDL carrier, where the switching period is located in FDD carrier

  - MediaTek (R4-2511066)

- Recommended WF: Discuss further

# Topic #3: UE capability

## Summary of open issues

Issue 3-1: Feature list and TR updates to include 70 us switching period

- Proposal: update the TR with the additional value (Samsung R4-2511072/R4-2511073, Apple R4-2509965, Xiaomi R4-2509821), and companies are encouraged to help ensure this update is also captured in the UE RF feature list (thread 137)

- Recommended WF: The proposal is based on the agreed WF from RAN4 #115 [R4-2508118] and is agreeable

Issue 3-2: Feature group description update

- See Proposal 1 in Xiaomi’s R4-2509821

- Recommended WF: Discuss further

# Topic #4: RMC

## Summary of open issues

Issue 4-1: Whether RMC definition can be discussed in the performance phase of the work item

- Proposal (taken from an observation in Samsung, R4-2511072): Given the situation of this WI discussion so far, RMC for REFSENS performance could be discussed at the last meeting of the WI, but it also could be continued until the performance part considering the impact on other specifications.\

- Recommended WF: Discuss further

Issue 4-2: Definition of the DL RMC

- Option 1: Specify fixed reference channels for FDD and SDL (configured with switching) with an allocation pattern aligned with the RAN1 (see Apple R4-2509965/ R4-2509569)

- Option 2: Increase period of RMC to 40 slots and use the LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’ (see CATT R4-2509246)

- Option 3: Further discuss among different periodicity options (see Qualcomm R4-2510968)

- Option 4: Further discuss whether and how to introduce new RMCs or related requirements (vivo, R4-2510272)

- Option 5: Tentative switching pattern for RRM RMC as proposed (Xiaomi, R4-2509821)

Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

- 0 as FDD carrier, 1 as SDL carrier

- 1 switch within 10 slots

- UL TA assume as 0 us for RMC definition and test

- Recommended WF: Discuss further

Issue 4-3: Whether to specify the UL RMC

- Option 1: UL RMC also needs to be specified for low band CA via switching (see Qualcomm R4-2510968, Apple R4-2509965)

- Option 2: No UL RMC is needed

- Option 3: Further discussion is needed

- Recommended WF: Discuss further

# Topic #5: Others

## Summary of open issues

Issue 5-1: Impact on other RX requirements

- Proposal: capture in the TR the impact of the feature on ΔRIB,c and ΔTIB,c, and MSD requirements (Apple R4-2509965)

- Recommended WF: Discuss further

# Topic #6: Text proposals and draft CRs

## Summary of open issues

Issue 6-1: TP for TR 38.768 to modify the general description (CATT, R4-2509248)

- Recommended WF: approve the TP

Issue 6-2: TP to TR38.768 on low NR band aggregation via switching (Apple, R4-2509965)

- Recommended WF:

  - Change 1: similar changes to R4-2511073

  - Change 2, Change 3: await conclusion of Topic 1 (CA configuration)

  - Change 4, 5: await conclusion of Topic 5 (Others)

  - Change 6: await conclusion of Topic 4 (RMC)

Issue 6-3: TP for 38.768 on switching period (Samsung, R4-2511073)

- Recommended WF: approve the TP

Issue 6-4: TP to TR38.768 Switching time mask (Huawei, R4-2510467)

- Recommended WF: await conclusion of Topic 2 (Tx mask)

Issue 6-5: Draft TR 38.768 v1.1.0 Low NR Band Carrier Aggregation via Switching (Apple, R4-2509202)

- Recommended WF: for after-meeting email approval

Issue 6-6: Draft CRs to introduce the feature to the RF specification

- CATT [R4-2509249]

- Ericsson [R4-2510317]

- MediaTek [R4-2511066]

- Huawei [R4-2510466]

- Apple, Telus [R4-2509569] (NOTE: this is CR, intended to be revised to capture the agreed content during the meeting)

- Recommended WF:

  - Discuss technical aspects of the proposals

  - Revise R4-2509569 to capture the agreed outcome

# Annex: List of agreements from RAN1 #121 (for information)

Please refer to the liaison received from RAN1 in R4-2509010.

Agreements made in RAN1#120bis

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Agreements made in RAN1#121

Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA