3GPP TSG-RAN WG4 Meeting #116	R4-2509108

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on 6 Layer MIMO performance requirements

Agenda item:	7.1.4

Document for:	Discussion

Introduction

The ongoing work item on UE RF enhancements in Rel-19 [3] includes the following objective:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During RAN4#114, the ongoing discussion led to an agreement that 6 Layer MIMO was feasible for FWA Devices, but not for handheld.

During RAN4#115 several issues were agreed [2], but many were left open; in this document Nokia provides views on some of those open issues.

In this document we present views to help define performance requirements for 6Rx devices

Discussion

Test applicability rule for 6Rx test case

At RAN4#115, three options were considered for defining applicability rules for 6Rx testing:

- Option 1: Introduce a UE declaration and define applicability rules across all cases if RAN2 does not introduce explicit capability signalling.

- Option 2: Introduce applicability rules only for 6-layer 6Rx cases.

- Option 3: Postpone applicability rule definition until RAN2 completes capability signalling.

Nokia believes that RAN4 should wait for completion of RAN2 work for further discussion on applicability rules

RAN4 shall postpone applicability rule definition until RAN2 completes capability signaling.

Performance requirements for 4 Layers

Regarding the performance requirements for 4 Layers Nokia believes that a moderate correlation, in accordance with a handheld UE should be used, such as that provided by ULA Medium A with α = 0.3 and β = 0.07179

RAN4 shall use ULA Medium A correlation with α = 0.3 and β = 0.07179

Test cases for 4 MIMO layers for FWA and Handheld UE

In order to reduce test cases for 6Rx devices for a late release of 5G Advanced, Nokia understands that only a single test case needs to be defined for 4 MIMO layer cases.

RAN4 shall use one single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

MCS for 4 Layers

For 4 Layer cases, MCS17 (Table 1) should be used to provide an initial perspective; however different MCS may work better for requirements derivation

RAN4 shall MCS 17 (Table 1) initially to explore requirements derivation for 4 Layers

Test applicability rule for FWA

In order to reduce test cases for 6Rx devices for a late release of 5G Advanced, Nokia understands that a device can skip the corresponding 4 layer 6Rx test if it passes a more stringent 6 layer case.

RAN4 shall specify an applicability rule where a device can skip corresponding 4 layers test if it passes the 6 layers tests.

DMRS configuration for 6 MIMO layers case

In order to enable the performance to be achieved of a 6Rx device, Nokia believes that is important a dense DMRS configuration is used, and in accordance with a Rel-15 DMRS type, but is open for discussion on this matter.

RAN4 could use Rel-15 Type 1 DMRS 2+2, but discussion should occur during RAN4#116.

SDR requirements for 1024QAM with 4 layers

As 1024 QAM  may be difficult to achieve with 6Rx devices, Nokia believes that this should not be required, equally 3GPP should aim to reduce the test cases on a late release feature to only the ones that are critical for deployment.

RAN4 to not Define SDR requirements for 1024QAM with 4 layers

CQI test for 6Rx

Nokia believes that a CQI test should not be required, equally 3GPP should aim to reduce the test cases on a late release feature to only the ones that are critical for deployment.

RAN4 to not define CQI test for 6Rx

RI test for only Rank 6

Nokia believes that a RI test should not be required, equally 3GPP should aim to reduce the test cases on a late release feature to only the ones that are critical for deployment.

RAN4 to not define RI tests for 6Rx

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4 for performance requirements derivation, and presents some of the Nokia views on this topic for discussion during RAN4#116.

The following Observations and Proposals were made:

Proposal 1: RAN4 shall postpone applicability rule definition until RAN2 completes capability signaling.

Proposal 2: RAN4 shall use ULA Medium A correlation with α = 0.3 and β = 0.07179

Proposal 3: RAN4 shall use one single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

Proposal 4: RAN4 shall MCS 17 (Table 1) initially to explore requirements derivation for 4 Layers

Proposal 5: RAN4 shall specify an applicability rule where a device can skip corresponding 4 layers test if it passes the 6 layers tests.

Proposal 6: RAN4 could use Rel-15 Type 1 DMRS 2+2, but discussion should occur during RAN4#116.

Proposal 7: RAN4 to not Define SDR requirements for 1024QAM with 4 layers

Proposal 8: RAN4 to not define CQI test for 6Rx

Proposal 9: RAN4 to not define RI tests for 6Rx

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- R4-2508727, “WF for 6Rx UE performance requirements”, Huawei, RAN4#115, Malta, May 2025

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024