3GPP TSG-RAN WG4 Meeting #116	R4-2509165

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.13.3

Source:	MediaTek Inc.

Title:	Discussion on UE demodulation requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

In this contribution, we share our views on the remaining issues for UE demodulation requirements with 8Rx MMSE-IRC receiver in [1].

- Discussion

Definition of MMSE-IRC receiver capability for 8Rx UE Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

In Rel-18 WI NR_ENDC_RF_FR1_enh2, RAN4 introduced two types of 8Rx receiver implementations. Consequently, RAN4 defined two new UE features in Table 5.1.1.3-1 of TS38.101-4: the Baseline SU-MIMO 8Rx receiver and the Simplified SU-MIMO 8Rx receiver. In Rel-19, RAN4 further considered scenarios involving inter-cell and intra-cell interference. Based on our understanding, the 8Rx requirements in Rel-18 (which do not account for ICI and MU-MIMO) and Rel-19 (which do account for ICI and MU-MIMO) are derived from the same assumptions regarding MMSE-IRC receiver implementation. We believe there is no issue in referencing “MMSE-IRC” in the definitions of the simplified and baseline 8Rx receivers. Therefore, we are comfortable with either Option 1 or Option 2.

Proposal 1: For the definition of simplified and baseline receiver, we are OK to both Option 1 and Option 2 to mention MMSE-IRC in the definition.

CQI calculation algorithm and Receiver coverage Agreement - Define one set of requirements. Configuration for CQI requirements Agreement: - Use [-2dB] SINR for requirement definition. - TBD for the Gamma value. Companies are encouraged to propose the value for Gamma in the next meeting.

At the last meeting, RAN4 reached a consensus to define a unified set of CQI requirement for 8Rx MMSE-IRC baseline and simplified receiver. The remaining issues to be addressed are the testing SINR and gamma values. Based on our simulation results presented in [2] and Table 1, we propose adopting SINR = -2 dB and a gamma value of 2 for both FDD and TDD.

Table 1. Gamma values when SINR = -2dB

|  | Gamma value for baseline receiver | Gamma value for simplified receiver |
| --- | --- | --- |
| FDD | 2.82 | 2.87 |
| TDD | 2.55 | 2.43 |

Proposal 2: For both FDD and TDD, use SINR = -2dB and Gamma = 2 to define 8Rx MMSE-IRC receiver CQI requirements.

# Conclusion

In this contribution, we present our views on UE requirements for devices equipped with an 8Rx MMSE-IRC receiver. Our proposals are outlined below:

Proposal 1: For the definition of simplified and baseline receiver, we are OK to both Option 1 and Option 2 to mention MMSE-IRC in the definition.

Proposal 2: For both FDD and TDD, use SINR = -2dB and Gamma = 2 to define 8Rx MMSE-IRC receiver CQI requirements.

# Reference

- R4-2508620, Way forward for [115][323] NR_demod_Ph5_Part2_UE, Nokia

- R4-2509166, Simulation results for UE demodulation requirements with 8Rx MMSE-IRC receiver, MediaTek