3GPP TSG-RAN WG4 Meeting #116	R4-2509290

# Bengaluru , IN, 25th – 29th Aug, 2025

Title: 	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Source: 	CATT

Agenda item:	7.25.3.3

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_Mob_Ph4_Part2 was approved and draft CR were endorsed [1], but there still an issue left. This document will further discuss the remaining RRM issues about Conditional LTM, and give our understanding and proposals.

- Discussion

- Tmeasure

In the last meeting, RAN4 discussed the components of Tmeasure defined in Conditional LTM delay, which is measureent time. According to the L1-RSRP/L3 measurement SSB/CSI-RS in the event, it has different specific definitions, and the agreements are copied as follows [1]:

< Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

For CSI-RS based L1-RSRP measurement, RAN4 agreed that Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period for known cell. For unknown cell case, since R18 LTM didn’t define the related requirements for unknown cell, so we think that CSI-RS based L1-RSRP measurement should follow R18 LTM and the requirements will be only applied to known case, and TPSS/SSS is not needed.

For the conditions for TCSI-RS_SFN_intra = 0, we think that the legacy conditions defined for CSI-RS based L3 measurement can be a reference, which is copied as follows [2]:

The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in table 9.10.2.5-3 for FR1. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

When deriveSSB-IndexFromCell is enabled, the time period used to acquire the SFN information(TCSI-RS_SFN_intra) is equal to 0.

Proposal 1: In C-LTM delay, CSI-RS based L1-RSRP measurement requirements will be only applied to known case, and TPSS/SSS is not needed.

Proposal 2: For CSI-RS based L1-RSRP measurement requirements, when deriveSSB-IndexFromCell is enabled, TCSI-RS_SFN_intra is equal to 0.

- Conclusion

This document discussed the issues for conditional Intra-CU LTM and presented following proposals:

Proposal 1: In C-LTM delay, CSI-RS based L1-RSRP measurement requirements will be only applied to known case, and TPSS/SSS is not needed.

Proposal 2: For CSI-RS based L1-RSRP measurement requirements, when deriveSSB-IndexFromCell is enabled, TCSI-RS_SFN_intra is equal to 0.

- Reference

[1]     R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

[2]     3GPP TS 38.133, NR Requirements for support of radio resource management