3GPP RAN WG4 Meeting #116	R4-2509299

Bengaluru, India, August 25th – August 29th, 2025

Title: 	Discussion on RRM core requirements testing framework for positioning

Source: 	CATT

Agenda item:	7.17.5

Document for:	Discussion

Introduction

Impacts of AI/ML based positioning on RRM requirements were discussed in last meeting and the agreements were captured in WF [1]. In this contribution, we will continue to discuss open issues summarized in [2] and analyse potential impacts on RRM core requirements.

Discussion

Report mapping for sample based timing measurement reporting

In last meeting, RAN4 discussed how to define reporting requirement for sample-based timing measurement and the following agreement was reached:

RAN4#115 Agreement: - Use the value of the legacy UL RTOA mapping for 1st sample - FFS for the additional sample reporting

RAN4 agreed to reuse the value of the legacy UL RTOA mapping for the 1st sample and further discussion is needed for the additional sample reporting. Till RAN1#121 meeting, RAN1 have reached sufficient agreements [3][4][5][6][7] for RAN4 to make a conclusion for this issue:

Agreement - RAN1#119: For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement – RAN1#120: For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement – RAN1#120: For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement – RAN1#116bis: For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement – RAN1#118: For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement – RAN1#121: For AI/ML positioning Case 3b, for gNB channel measurements reported to LMF, regarding the power information (if included), - reuse the existing measurement report mapping table for SRS-RSRPP in 38.133.

Figure 1 illustrates the mechanism of sample-based positioning measurement report:

Figure 1 Sample-based positioning measurement report

RAN4 agreed to reuse the UL RTOA reporting requirement for the 1st sample, the time instance of which is relative to the reference time T0+tSRS as defined in TS 38.215. And for the additional samples, their time instances can be derived in a cumulative way relative to the 1st sample, e.g., the time instance of the 2nd sample equals to the time instance of the 1st + sample, where sample is integral times of timing granularity T which equals to 2kxTc and k = {0...5}. Meanwhile, the maximum number of additional samples is 127 and therefore the report mapping tables proposed in Proposal 1 can be defined for additional sample report:

Proposal 1: RAN4 to define the following report mapping requirements for additional sample report:

Table 1: Measurement report mapping for k=0

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 1 | Tc |
| sample_001 | sample = 2 | Tc |
| sample_002 | sample = 3 | Tc |
|  |  | … |
| sample_063 | sample = 64 | Tc |
| sample_064 | sample = 65 | Tc |
| … | … | … |
| sample_124 | sample = 125 | Tc |
| sample_125 | sample = 126 | Tc |
| sample_126 | sample = 127 | Tc |

Table 2: Measurement report mapping for k=1

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 2 | Tc |
| sample_001 | sample = 4 | Tc |
| sample_002 | sample = 6 | Tc |
|  |  | … |
| sample_063 | sample = 128 | Tc |
| sample_064 | sample = 130 | Tc |
| … | … | … |
| sample_124 | sample = 250 | Tc |
| sample_125 | sample = 252 | Tc |
| sample_126 | sample = 254 | Tc |

Table 3: Measurement report mapping for k=2

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 4 | Tc |
| sample_001 | sample = 8 | Tc |
| sample_002 | sample = 12 | Tc |
|  |  | … |
| sample_063 | sample = 256 | Tc |
| sample_064 | sample = 260 | Tc |
| … | … | … |
| sample_124 | sample = 500 | Tc |
| sample_125 | sample = 504 | Tc |
| sample_126 | sample = 508 | Tc |

Table 4: Measurement report mapping for k=3

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 8 | Tc |
| sample_001 | sample = 16 | Tc |
| sample_002 | sample = 24 | Tc |
|  |  | … |
| sample_063 | sample = 520 | Tc |
| sample_064 | sample = 528 | Tc |
| … | … | … |
| sample_124 | sample = 1000 | Tc |
| sample_125 | sample = 1008 | Tc |
| sample_126 | sample = 1016 | Tc |

Table 5: Measurement report mapping for k=4

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 16 | Tc |
| sample_001 | sample = 32 | Tc |
| sample_002 | sample = 48 | Tc |
|  |  | … |
| sample_063 | sample = 1024 | Tc |
| sample_064 | sample = 1040 | Tc |
| … | … | … |
| sample_124 | sample = 2000 | Tc |
| sample_125 | sample = 2016 | Tc |
| sample_126 | sample = 2032 | Tc |

Table 6: Measurement report mapping for k=5

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 32 | Tc |
| sample_001 | sample = 64 | Tc |
| sample_002 | sample = 96 | Tc |
|  |  | … |
| sample_063 | sample = 2048 | Tc |
| sample_064 | sample = 2080 | Tc |
| … | … | … |
| sample_124 | sample = 4000 | Tc |
| sample_125 | sample = 4032 | Tc |
| sample_126 | sample = 4064 | Tc |

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM requirements and following proposals are provided:

Proposal 1: RAN4 to define the following report mapping requirements for additional sample report:

Table 1: Measurement report mapping for k=0

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 1 | Tc |
| sample_001 | sample = 2 | Tc |
| sample_002 | sample = 3 | Tc |
|  |  | … |
| sample_063 | sample = 64 | Tc |
| sample_064 | sample = 65 | Tc |
| … | … | … |
| sample_124 | sample = 125 | Tc |
| sample_125 | sample = 126 | Tc |
| sample_126 | sample = 127 | Tc |

Table 2: Measurement report mapping for k=1

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 2 | Tc |
| sample_001 | sample = 4 | Tc |
| sample_002 | sample = 6 | Tc |
|  |  | … |
| sample_063 | sample = 128 | Tc |
| sample_064 | sample = 130 | Tc |
| … | … | … |
| sample_124 | sample = 250 | Tc |
| sample_125 | sample = 252 | Tc |
| sample_126 | sample = 254 | Tc |

Table 3: Measurement report mapping for k=2

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 4 | Tc |
| sample_001 | sample = 8 | Tc |
| sample_002 | sample = 12 | Tc |
|  |  | … |
| sample_063 | sample = 256 | Tc |
| sample_064 | sample = 260 | Tc |
| … | … | … |
| sample_124 | sample = 500 | Tc |
| sample_125 | sample = 504 | Tc |
| sample_126 | sample = 508 | Tc |

Table 4: Measurement report mapping for k=3

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 8 | Tc |
| sample_001 | sample = 16 | Tc |
| sample_002 | sample = 24 | Tc |
|  |  | … |
| sample_063 | sample = 520 | Tc |
| sample_064 | sample = 528 | Tc |
| … | … | … |
| sample_124 | sample = 1000 | Tc |
| sample_125 | sample = 1008 | Tc |
| sample_126 | sample = 1016 | Tc |

Table 5: Measurement report mapping for k=4

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 16 | Tc |
| sample_001 | sample = 32 | Tc |
| sample_002 | sample = 48 | Tc |
|  |  | … |
| sample_063 | sample = 1024 | Tc |
| sample_064 | sample = 1040 | Tc |
| … | … | … |
| sample_124 | sample = 2000 | Tc |
| sample_125 | sample = 2016 | Tc |
| sample_126 | sample = 2032 | Tc |

Table 6: Measurement report mapping for k=5

| Reported Quantity Value, sample_i | Measured Quantity Value, sample | Unit |
| --- | --- | --- |
| sample_000 | sample = 32 | Tc |
| sample_001 | sample = 64 | Tc |
| sample_002 | sample = 96 | Tc |
|  |  | … |
| sample_063 | sample = 2048 | Tc |
| sample_064 | sample = 2080 | Tc |
| … | … | … |
| sample_124 | sample = 4000 | Tc |
| sample_125 | sample = 4032 | Tc |
| sample_126 | sample = 4064 | Tc |

References

[1] R4-2508080, WF on NR_AIML_air_part1, Qualcomm, RAN4#115.

[2] R4-2507570, Topic summary for [115][131] NR_AIML_air_part1, Qualcomm, RAN4#115.

[3] Chair Notes RAN1#116bis, RAN1#116bis.

[4] Chair Notes RAN1#118, RAN1#118.

[5] Chair Notes RAN1#119, RAN1#119.

[6] Chair Notes RAN1#120, RAN1#120.

[7] Chair Notes RAN1#121, RAN1#121.