3GPP TSG-RAN WG4 Meeting #116                                                                                                      R4-2509337

Bengaluru, India, 25th –29th August, 2025

Source: 	CATT

Title: 	On co-existence simulation results for NR NTN Ku-band

Agenda item:	7.7.3

Document for:	Approval

- Introduction

In RAN4# 115 meeting, the running document on Ku band co-existence simulation assumptions (running document)[1]  is provided to consider for NTN Ku-band co-existence simulation.

In this contribution, we provide our some coexistence simulation results for Ku-band for NR NTN.

- Discussion

Our evaluation result is based on running document on Ku band co-existence simulation assumptions (running document)[1]. Where parameters in Table 6a.2.2.2-2 are updated to align those in “Circular Aperture TR 38.863 ” column of Table 6a.2.2.2-1.

Table 6a.2.2.2-2: Other NTN UE parameters

| Characteristics | VSAT (Parabolic) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 2.5 dB for both LEO and GEO |
| Tx transmit power | 10 Watts for GEO and for LEO |
| Tx antenna gain | 36.6 dBi |
| Antenna elevation angle | 35˚ and 90˚ |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. |  |

The highlighted four scenarios in WF[1]  are copied in following table.

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| Scenario 1 | TN with NTN | NTN UL | TN UL | 14 GHz | ACLR NTN UE to be varied/defined. ACS TN gNB fixed |
| Scenario 4 | TN with NTN | TN DL | NTN UL | 14 GHz | ACLR TN gNB fixed. ACS NTN SAN to be varied/defined |
| Scenario 5 | TN with NTN | TN DL | NTN DL | 11 GHz | ACLR TN gNB fixed. ACS NTN UE to be varied/defined |
| Scenario 6 | TN with NTN | NTN DL | TN DL | 11 GHz | ACLR NTN SAN to be varied/defined. ACS TN UE fixed |

The evaluation results for highlighted four scenarios are shown in following sub-clauses.

2.1 Scenario 1: 14GHz NTN UL interfering TN UL

GEO

The evaluation results of required ACIR for scenario 1 for GEO are shown in Table 2.1-1.

Table 2.1-1: Required ACIR for Scenario 1 for GEO

LEO

The evaluation results of required ACIR for scenario 1 for LEO-600 are shown in Table 2.1-2.

Table 2.1-2: Required ACIR for Scenario 1 for LEO-600

The worst interpolated ACIR @5% throughput loss for Scenario 1 in above Table 2.1-1 and Table 2.1-2 are summarized in Table 2.1-3.

Table 2.1-3: Worst interpolated ACIR @5% throughput loss for Scenario 1

2.2 Scenario 4: 14GHz TN DL interfering NTN UL

GEO

The evaluation results of required ACIR for scenario 4 for GEO are shown in Table 2.2-1.

Table 2.2-1: Required ACIR for Scenario 4 for GEO

LEO

The evaluation results of required ACIR for scenario 4 for LEO-600 are shown in Table 2.2-2.

Table 2.2-2: Required ACIR for Scenario 4 for LEO-600

The worst interpolated ACIR @5% throughput loss for Scenario 4 in above Table 2.2-1 and Table 2.2-2 are summarized in Table 2.2-3.

Table 2.2-3: Worst interpolated ACIR @5% throughput loss for Scenario 4

2.3 Scenario 5: 11GHz TN DL interfering NTN DL

GEO

The evaluation results of required ACIR for scenario 5 for GEO are shown in Table 2.3-1.

Table 2.3-1: Required ACIR for Scenario 5 for GEO

LEO

The evaluation results of required ACIR for scenario 5 for LEO-600 are shown in Table 2.3-2.

Table 2.3-2: Required ACIR for Scenario 5 for LEO-600

The worst interpolated ACIR @5% throughput loss for Scenario 5 in above Table 2.3-1 and Table 2.3-2 are summarized in Table 2.3-3.

Table 2.3-3: Worst interpolated ACIR @5% throughput loss for Scenario 5

2.4 Scenario 6: 11GHz NTN DL interfering TN DL

GEO

The evaluation results of required ACIR for scenario 6 for GEO are shown in Table 2.4-1.

Table 2.4-1: Required ACIR for Scenario 6 for GEO

LEO

The evaluation results of required ACIR for scenario 6 for LEO-600 are shown in Table 2.4-2.

Table 2.4-2: Required ACIR for Scenario 6 for LEO-600

The worst interpolated ACIR @5% throughput loss for Scenario 6 in above Table 2.4-1 and Table 2.4-2 are summarized in Table 2.4-3.

Table 2.4-3: Worst interpolated ACIR @5% throughput loss for Scenario 6

2.5 Summaries

From above Table 2.1-3, Table 2.2-3, Table 2.2-3, and Table 2.4-3, we have the following observation.

Observation 1: The worst interpolated ACIR@5% throughput loss for Scenarios 1, 4, 5 and 6 are shown in the following table.

| ACIR（worst case btw 35/90 deg） |  |  | worst interpolated ACIR[dB] @ 5% throughput loss |
| --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array | 31.55 |
|  |  | Parabolic | 18.67 |
|  | LEO | Phase array | 27.77 |
|  |  | Parabolic | 14.52 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array | 37.77 |
|  |  | Parabolic | 38.43 |
|  | LEO | Phase array | 40 |
|  |  | Parabolic | 40 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array | 40 |
|  |  | Parabolic | 33.58 |
|  | LEO | Phase array | 40 |
|  |  | Parabolic | 38.41 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array | 0 |
|  |  | Parabolic | 0 |
|  | LEO | Phase array | 0 |
|  |  | Parabolic | 0 |

For scenario 1, it is for defining VSAT ACLR, the worst interpolated ACIR @5% throughput loss for NTN UL interfering TN UL is 31.55dB.  From perspective of co-existence, simulation shows that VSAT ACLR needs to be defined as 31.55dB.

Observation 2: From perspective of co-existence, simulation result shows that VSAT ACLR needs to be defined as 31.55dB.

For scenario 4, it is for defining SAN ACS, the worst interpolated ACIR @5% throughput loss for TN DL interfering NTN UL is 40dB. From perspective of co-existence, simulation shows that SAN ACS needs to be defined as 40dB.

Observation 3: From perspective of co-existence, simulation result shows that SAN ACS for GEO and LEO needs to be defined as 40dB.

For scenario 5, it is for defining VSAT ACS, the worst interpolated ACIR @5% throughput loss for TN DL interfering NTN DL is 40dB. From perspective of co-existence, simulation shows that VSAT ACS needs to be defined as 40dB.

Observation 4: From perspective of co-existence, simulation result shows that VSAT ACS needs to be defined as 40dB.

For scenario 6, it is for defining SAN ACLR, the worst interpolated ACIR @5% throughput loss for NTN DL interfering TN DL is 0dB, which shows that co-existence scenario 6 needs smaller SAN ACLR. In TS 38.108, 14dB ACLR was defined for SAN type 1-O and 12dB ACLR was defined for SAN type 2-O, so for Ku-band NTN, the SAN ACLR can be defined as 13dB, which is equal to (14+12)/2 = 13dB, since the frequency of Ku-band is between frequency for SAN type 1-O and frequency for SAN type 2-O. so, we propose,

Observation 5: Define 13dB SAN ACLR for GEO and LEO class for Ku-band NTN.

- Conclusion

This contribution provides our some coexistence simulation results for Ku-band for NR NTN, and the following Observation is concluded:

Observation 1: The worst interpolated ACIR@5% throughput loss for Scenarios 1, 4, 5 and 6 are shown in the following table.

| ACIR（worst case btw 35/90 deg） |  |  | worst interpolated ACIR[dB] @ 5% throughput loss |
| --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array | 31.55 |
|  |  | Parabolic | 18.67 |
|  | LEO | Phase array | 27.77 |
|  |  | Parabolic | 14.52 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array | 37.77 |
|  |  | Parabolic | 38.43 |
|  | LEO | Phase array | 40 |
|  |  | Parabolic | 40 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array | 40 |
|  |  | Parabolic | 33.58 |
|  | LEO | Phase array | 40 |
|  |  | Parabolic | 38.41 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array | 0 |
|  |  | Parabolic | 0 |
|  | LEO | Phase array | 0 |
|  |  | Parabolic | 0 |

Observation 2: From perspective of co-existence, simulation result shows that VSAT ACLR needs to be defined as 31.55dB.

Observation 3: From perspective of co-existence, simulation result shows that SAN ACS for GEO and LEO needs to be defined as 40dB.

Observation 4: From perspective of co-existence, simulation result shows that VSAT ACS needs to be defined as 40dB.

Observation 5: Define 13dB SAN ACLR for GEO and LEO class for Ku-band NTN.

- References

- R4-2508774, Ku band co-existence simulation assumptions (running document), Ericsson, RAN4#115