3GPP TSG-RAN WG4 Meeting #116	                                                                                            R4-2509361

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	7.6.3.2.1

Document for:	Approval

- Introduction

In RAN4 #115 meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The WF on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

## Sub-topic 1-1 General and Power Class 2 Issue 1-1: Configured Tx power equation Agreement: - Agree to clarify and correct the configured transmitted power equation for NTN - [∆TC,c], ∆TIB,c , [∆TRxSRS] are not applicable to Rel-19 NR-NTN UE - FFS ΔPPowerClass - For ΔPPowerBoost, Rel-18 power boosting should be applicable to NR-NTN. - Regarding how to address these in CR drafting, following approach is agreed. - By referring to 38.101-1 and then exclude non-applicable parameters, e.g. ‘except for...’ Issue 1-2: A-MPR simulation Agreement: - For PC2, to adopt this scheme: - In the simulation assumption, the maximum UL allocation size of [25, 100] RBs is considered. - Other number of RB is not precluded Issue 1-3: AMPR assumption for n255 Agreement: - Agree to use 10dB as filter rejection assumption for GNSS frequencies 1559-1610 MHz for PC2/PC1.5/PC1 AMPR analysis for n255. Issue 1-4: PC2 AMPR Way forward: - To harmonize the AMPR regions in offline activates before next meeting. - Target in next meeting: Come up with numbers in square brackets, and confirmed by other companies. - Offline efforts before next meeting is expected. ## Sub-topic 2-2 Power Class 1 Issue 1-6: PC1 ACLR Agreement: - PC1 ACLR = 34dBc. Issue 1-8 ~ Issue 1-9 PC1 MPR related Agreement: - Interested companies are encouraged to work on two tables with tentative values for PC1 MPR, one for 1Tx and one for 4Tx - Further check these tentative values, especially for the edge RB results by considering the working assumption of L_CRB as [8]RB. - With the understanding the [4Tx] architecture for PC1 is not agreed yet. - If 4Tx would be agreed, it can be further discussed if we can merge two tables then. Table 1. PC1 MPR [with 1Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; [Note 1]; ≤ [0.5]; ≤ [0]. QPSK; [Note 1]; ≤ [1]; ≤ [0]. 16 QAM; [Note 1]; ≤ [2]; ≤ [1]. 64 QAM; [Note 1]; ≤ [2.5]; ≤ [2.5]. CP-OFDM; QPSK; [Note 1]; ≤ [3]; ≤ [1.5]. 16 QAM; [Note 1]; ≤ [3]; ≤ [2]. 64 QAM; [Note 1]; ≤ [3.5]; ≤ [3.5]. NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB.. [Table 2. PC1 MPR with 4Tx] Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ [9.5]; ≤ [2.0]; ≤ [0.5]. QPSK; ≤ [10.0]; ≤ [2.5]; ≤ [0.5]. 16 QAM; ≤ [10.0]; ≤ [3.5]; ≤ [1.5]. 64 QAM; ≤ [10.0]; ≤ [4.0]; ≤ [3.5]. CP-OFDM; QPSK; ≤ [10.0]; ≤ [4.5]; ≤ [2.0]. 16 QAM; ≤ [10.0]; ≤ [4.5]; ≤ [2.5]. 64 QAM; ≤ [10.0]; ≤ [5.0]; ≤ [5.0]. ## Sub-topic 1-3 Power Class 1.5 Issue 1-10: PC 1.5 UE architecture assumption Agreement: - PC1.5 for non-handheld NR-NTN UE is primarily considered with 2Tx in Rel-19 - PC1.5 with 1Tx is not precluded but with low priority. It can be treated once PC1.5 with 2Tx work is done. Issue 1-11: PC1.5 1Tx MPR Way forward: - Further discuss below proposed MPR values for 1Tx non-handheld, considering its low priority as agreed. DFT-OFDM; edge; outer; inner. QPSK; 6.5; 1.5; 0. 16QAM; 6.5; 2.5; 1. CP-OFDM; edge; outer; inner. QPSK; 6.5; 3.5; 2. 16QAM; 6.5; 3.5; 2.5. ## Sub-topic 1-4 Others Issue 1-12: Power boosting for PC2 Way forward: - Further discuss evaluate the PC2 NTN power boosting option with low priority if time allows, and not target to define in Rel-19. - Further discuss following assumptions: - PC2 1Tx PA using existing TN impairment and targeting inner third allocations. - Applicable to both TN and NTN. - FFS applicability of such boosting to multi-Tx based on PC2 PAs

With reference to the discussion on NTN RedCap UE power boosting, the applicability of power boosting to the NTN (RedCap) UE has been discussed several times in parallel in RAN4. The preliminary discussion is to consider whether to introduce power boosting for the PC 3 NTN RedCap UE to increase the transmit power, as shown below:

## Topic 1: Output power enhancement for NR NTN PC3 Agreements: - Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting - FFS for details, capabilities will be needed for feature list - When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations] - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost - Boost is 0.5 dB - FFS on duplex modes - For information only: HD-FDD/FD-FDD support is indicated per band - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting - Rel-18 and Rel-19 power boosting schemes can be simultaneously supported - FFS for details - PC3 ACLR requirements apply for power boosted transmissions - Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

Observation 1: Power boosting is applicable for NTN PC 3 RedCap UE.

Meanwhile, the conclusion of NTN RedCap UE power boosting indicates that the introduction of power boosting may not impact the NTN PC3 RedCap UE RF requirements (such as MPR/AMPR). If the Rel-18 power boosting scheme is adopted, it can be foreseen that no impact on MPR/A-MPR. If the Rel-19 power boosting scheme is adopted, it can be expected that there will be no significant impact on MPR/A-MPR as well, since the escalation of Tx power is mainly due to the reduction of IL. Based on previous discussions of NTN HPUE, it is reasonable to believe that a slightly power boosting for NTN PC2 HPUE may not have additional impact on RF requirements such as MPR as well.

Observation 2: Power boosting might be applicable for NTN PC 2 HPUE.

- Conclusion

This contribution provides our views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Observation 1: Power boosting is already applicable for NTN PC 3 RedCap UE.

Observation 2: Power boosting might be applicable for NTN PC 2 HPUE.

- References

- R4-2508008, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #115