3GPP TSG-RAN WG4 #116	R4-2509393

Bengaluru, India, August 25-29, 2025

Agenda item:	7.23.4		RRM performance requirements

Source:	Qualcomm Incorporated

Title:	RRM performance requirements for Rel-19 NES

Document for:	Discussion

# Introduction

In RAN4 #115 the work on RRM performance requirements for Rel-19 NES started. In this paper we provide our view on RRM performance requirements as identified in [1] in RAN4 #115.

# Discussion

## Test Cases for OD-SSB

The way forward in [1] lists the following scenarios:

Issue 3-1-1: Scenario

RAN4 to discuss whether the following test scenarios can be agreed.

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

We agree to the majority of test scenarios in Issue 3-1-1 with some modifications and additions.

Proposal 1: RAN4 should adopt the majority of test scenarios in Issue 3-1-1 with the following modifications.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Proposal 2: RAN4 should not define test cases for case 2 Alt Time-C2 and for the case when always-on SSB and OD-SSB have different center frequencies.

The way forward in [1] lists in in Issue3-1-3 a list of test cases to be discussed for RAN4. We are fine with most of the test cases being proposed. For fast SCell activation it was mentioned during the core requirement discussions that OD-SSB should only be used for L3 measurements for deactivated SCell. This behavior is already tested in other test cases. For the activation itself, there is no need to test OD-SSB since aperiodic TRS takes over the role of OD-SSB. Hence, fast SCell activation test cases can be skipped completely.

Proposal 3: RAN4 should not define fast SCell activation test cases for OD-SSB.

Regarding the number Scell activation test cases we propose to focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCell activation. The other SCell activation scenarios should have lower priority.

Proposal 4: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCell activation. The other SCell activation test cases should have lower priority.

Issue 3-1-3: Test case list

RAN4 to discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

The way forward in [1] lists the following options for the test configurations. We are fine with these configurations.

Issue 3-1-4: General principle

RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed.

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

Proposal 5: RAN4 should adopts the test configurations listed in Issue 3.1.4.

## Test Cases for OD-SIB1

The way forward in [1] lists the following options for defining test cases:

Issue 3-2-1: Test case

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

In our view, it is sufficient to test UL WUS transmission and OD-SIB1 reception as part of cell reselection. No separate tests for UL WUS and OD-SIB1 reception are needed. We believe that OD-SIB1 reception is not related to RRM performance but rather to protocol or functional performance testing. This can be more adequately tested in RAN5 protocol testing rather than RRM RAN4 performance testing.

Proposal 6: No RRM test cases for OD-SIB1 should be defined in RAN4. Rather this can be left to RAN5 protocol testing.

If RAN4 believes that a RRM performance test case should be introduced, it should be tested that the cell re-selection delay to a newly detectable cell and to an already detected cell is within Δ1 and Δ2 seconds. Default assumption for TSI-NR could be 1280 ms for FR1 intra-frequency, perhaps adding an additional Δ. There is no need to specify more details on UL WUS transmission and OD-SIB1 reception.

Proposal 7: In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested.

## Test Cases for SSB Adaptation

The way forward in [1] lists the following list of test cases:

Issue 3-3-1: Test case list

RAN4 to discuss whether the following SSB adaptation test case list can be agreed.

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- direct SCell activation

- PUCCH SCell activation

In our understanding, deactivated SCell measurements and SCell activation are not supported for SSB adaptation. Hence, no test can be introduced in RAN4 for these scenarios.

Proposal 8: Introduce test cases for SSB adaptation for L1-RSRP, L1-SINR, CBD and L3 measurements.

The way forward in [1] lists the following general principle how to define test cases for SSB adaptation:

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- Both FR1 and FR2

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement

- Only test non-DRX mode

- without SSB time index detection

We agree to the general principle how to define test cases for SSB adaptation.

Proposal 9: RAN4 should adopt the general principle in Issue 3-3-2 how to define test cases for SSB adaptation.

The way forward in [1] lists the following SMTC and SSB configurations:

Issue 3-3-3: SMTC and SSB configuration

- Proposals

  - Option 1:

    - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation.

  - Option 2:

    - SSB periodicity: 40 ms to 20 ms

    - SMTC periodicity: 40ms

  - Option 3:

    - SSB periodicity from 80ms to 20ms

    - SMTC periodicity from 80ms to 20ms

Since for L1-RSRP and L1-SINR reporting, the requirement when the measurement report is sent is an upper bound, differences may only be visible for large deltas in the periodicity before and after SSB adaptation. Hence, the delta of SSB and SMTC periodicity before and after adaptation should be large. Option 3 fulfills this pre-requisite in our view.

Proposal 10: RAN4 should adopt Option 3 in Issue 3-3-3: SSB and SMTC periodicity changes from 80 ms to 20 ms and 20 ms to 80 ms.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #115. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should adopt the majority of test scenarios in Issue 3-1-1 with the following modifications.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Proposal 2: RAN4 should not define test cases for case 2 Alt Time-C2 and for the case when always-on SSB and OD-SSB have different center frequencies.

Proposal 3: RAN4 should not define fast SCell activation test cases for OD-SSB.

Proposal 4: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCell activation. The other SCell activation test cases should have lower priority.

Proposal 5: RAN4 should adopts the test configurations listed in Issue 3.1.4.

Proposal 6: No RRM test cases for OD-SIB1 should be defined in RAN4. Rather this can be left to RAN5 protocol testing.

Proposal 7: In case that OD-SIB1 test cases are introduced in RAN4, the test case should be a cell-reselection test case with a certain value of TSI-NR. Default assumption for TSI-NR could be 1280 ms + Δ ≥ 0 ms. No more details for UL WUS and OD-SIB1 delays need to be tested.

Proposal 8: Introduce test cases for SSB adaptation for L1-RSRP, L1-SINR, CBD and L3 measurements.

Proposal 9: RAN4 should adopt the general principle in Issue 3-3-2 how to define test cases for SSB adaptation.

Proposal 10: RAN4 should adopt Option 3 in Issue 3-3-3: SSB and SMTC periodicity changes from 80 ms to 20 ms and 20 ms to 80 ms.

# References

- R4-25008277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #115, May 2025