3GPP TSG-RAN WG4 Meeting #116           	R4-2509399

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.22.9

Source:	Samsung

Title:	View on demodulation requirements for Ambient-IoT BS

Document for:	Discussion

- Introduction

In the RAN#106 meeting, the new WI related with solutions for Ambient IoT (Internet of Things) in NR was endorsed in [1] with the following objectives

- General scope - The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only - Device (un)availability via Direction 1 in TR 38.769 only - RAN1 scope - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. - RAN2 scope - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. - Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included - RAN3 scope - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including - Inventory and command operations - Device location reporting at reader ID granularity - Note: The above A-IoT functions are supported over the existing NG interface, based on architectures defined by RAN3/SA2 - RAN4 scope - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band - Objective of Performance part WI - Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

As stated in the WID, from RAN4 perspective, RAN4 needs to specify the necessayr demodualtion requirements for device 1 and Ambient-IoT BS.

According to meeting agenda, this is the 1st meeting to dicuss the performance part of Ambient-IoT device 1 and BS. In this contribution, the overall view on the test scope of demodulation requirement for Ambient-IoT BS is provided.

# 2	General Scope

In this section, the view on general scope of demodulation requirement for Ambient-IoT BS is provided

Connectivity topologies

Regarding the connectivity, there are four topologies to support A-IoT operation and two deployment scenarios supported

Figure 1: Topology 1: BS – Ambient IoT device

According to WID, the D1TI was included, as the deployment scenario 1 with connectivity topology 1. With Ambient IoT device indoors and base station indoors, the deployment scenario 1 is characterized according to the following table

Table 1: Characteristic of deployment scenario 1

| Applicable representative use cases | Characteristics | Description (NOTE 1) |
| --- | --- | --- |
| Indoor inventory Indoor sensor Indoor positioning Indoor command | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell |
|  | Connectivity topology | Topology (1), (2), (3) |
|  | Spectrum | Licensed FDD, licensed TDD, unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site or new site |
|  | Traffic assumption | DT and DO |
|  | Device characteristic | Device A or Device B or Device C |

Proposal 1: RAN4 focuses on D1T1-B scenario for specifying the A-IoT BS demodulation requirement

In band and standalone operation

For the A-IoT, the spectrum development support both in-band to NR and standalone, with A-IoT BS located indoor. Form the A-IoT BS demodulation process perspective, there is no different, the requirement should apply for both developments.

Proposal 2: RAN4 focuses on the standalone for specifying the A-IoT BS demodulation requirement.

PDRCH

Based on RAN1 design for A-IoT, only one physical channel is available for D2R link, as PDRCH (Physical Device to reader channel (PDRCH)), which carriers the data and control information. Due to the simple design of the device, dedicated control channel (e.g., 5G PUCCH like channel), and random-access channel (e.g,5G PRACH like channel) are not considered for A-IoT. Reference signals such as DM-RS, PTRS, SRS are considered.

For PDRCH transmission, RAN1 has designed the waveform, modulation, coding, repetition, FEC schemes to support A-IoT. The following is the processing procedure which includes CRC attachment, coding, and modulation/waveform generation where the coding including FEC, TB repetition, and line coding

Figure 2: PDRCH transmission structure

In general, considering it is a new physical layer design, and it is obviously that there is a baseband impact for A-IoT, therefore, RAN4 should define the related demodulation requirement for PDRCH transmission.

Proposal 3: RAN4 to define PDRCH the demodulation requirement for Ambient-IoT

D2R timing

Due to the asynchronous nature of A-IoT communication, timing signals should be transmitted from the device to the reader. A preamble-based D2R timing acquisition signal (D-TAS) is transmitted before the PDRCH to indicate the starting point of the PDRCH. This signal can also be used for SFO, CFO and channel estimation. The D-TAS signal is a binary signal that has good auto-correlation and cross-correlation properties. For D2R channel estimation, CFO estimation, time tracking, etc., a midamble signal is used. The number of midambles used depends on the size of the PDRCH data, but there is a trade-off between the number of midambles and achievable datarates. And the postamble is used to indicate the end of a PDRCH transmission.

Figure3 ： Physical channels and signals for A-IoT, D2R transmission

Observation 1: RAN1 has designed the preamble-based D2R transmission with D-TAS signal to indicate the starting point of the PDRCH transmission, and designed postamble transmission to indicate the end of the PDRCH transmission

The main purpose of D2R preamble transmission, is for the timing acquisition, indicating the start of the D2R transmission in the time domain, and potentially for SFO estimation, CFO estimation, channel estimation and interference estimation. The preamble design including two parts, one is related with sampling frequency training for coarse SFO estimation.

From the D2R preamble signal generation, it is based on m-sequence with the following agreement in RAN1

Agreement - For D2R preamble/midamble, base sequence is generated from m-sequence, where the length of the sequence is 2n-1 - Value(s) of n - Long preamble/midamble is generated based on n = 5 - Working assumption: Short preamble/midamble is generated based on n=3 - Only 1-part preamble/midamble are supported for D2R - Preamble immediately precedes the PDRCH without any gap - Both long and short preamble and midamble are supported based on the working assumption on n - when midamble is present at least the following cases are supported and reader explicitly indicates one of the following cases for PDRCH: - Short preamble and short midamble - Long preamble and long midamble Note: the case of short preamble and long midamble will not be supported - When midamble is not present the reader explicitly indicates short or long preamble for PDRCH For D2R preambles, - For n = 3, adopt m-sequence with following: - Polynomial: x³ + x² + 1 - Initial State: Down-select between 010 or 100 - Resulting Sequence: Down-select between 0 1 0 0 1 1 1 (for 010 initial state) or 1 0 0 1 1 1 0 (for 100 initial state) - For n = 5, adopt m-sequence with following: - Polynomial: x⁵ + x³ + 1 - Initial State: 01001 - Resulting Sequence: 0 1 0 0 1 0 0 0 0 1 0 1 0 1 1 1 0 1 1 0 0 0 1 1 1 1 1 0 0 1 1 - For m-sequence with n =3 for D2R preambles, adopt initial State of 100 and resulting sequence of 1 0 0 1 1 1 0

In general, based on the detection of D-TAS and postamle, the starting point and end of PDRCH transmission, then, the length or TBS of PDRCH transmission can be obtained. Based on that, the A-IoT BS will proceed the PDRCH detection. Different with NR system, since there is no dedicated PDCCH channel to indicate the TBS of data signal, the A IoT BS only relied the preamble detection to obtain the related TBS information of PDRCH. Therefore, we think the preamble detection should be included when defining the demodulation requirement for PDRCH.

Proposal 4: RAN4 should further discuss whether to include the preamble detection when defining demodulation requirement for PDRCH

Random Access in Ambient-IoT

Random Access allows the A-IoT device to identify itself to the reader. In A-IoT, the random-access procedure is triggered by the reader with a paging message. A reader can page a single device, a group of A-IoT devices or all the devices under its coverage. A device can receive the paging as long as it has harvested enough energy. The reader can indicate to the device whether to use contention-based or contention-free random access. The device responds to the paging message with a D2R transmission containing a 16-bit random ID. This step is the A-IoT Msg 1

In general, the random-access transmission for A-IoT device is similar as PRACH transmission in NR, while there is no dedicated random-access channel considered for A-IoT. Therefore, RAN4 should not to define the random-access requirements for Ambient-IoT.

Proposal 5: No Random-access requirement will be defined for Ambient-IoT.

# 3	Test Setup

In this section, the view on the test setup for PDRCH demodulation requirement.

Modulation

Based on RAN1 design, both OOK and BPSK modulation can be supported for D2R transmission, it is up to device implementation to use OOK and BPSK modulation.

Agreement - For D2R BPSK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband. - For D2R OOK modulation - After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in the baseband.

Proposal 6: RAN4 should cover both BPSK and OOK modulation scheme when defining PDRCH requirement

Waveform

For the D2R transmission by backscattering, the waveform is provided by the carrier wave. The D2R baseband signal is not -OFDM

Proposal 7: RAN4 will follow the RAN1 agreement related with waveform when defining PDRCH requirement

Channel coding and FEC

Regarding channel coding and FEC, the following is agreement in RAN1 meeting

Agreement - For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword. - For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, e.g., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12, with the code rate of 1/3. - The constraint length and polynomials for convolutional codes from LTE are reused for D2R FEC. - For the D2R transmission of Ambient IoT, convolutional codes with the end state of the shift registers being the same as the initial state of the shift registers is supported for D2R FEC.

Proposal 8: RAN4 will follow the RAN1 agreement related with channel coding and FEC when defining PDRCH requirement

Repetition

For the D2R transmission, the block level repetition is supported as following

Figure 4: PDRCH transmission with block-level repetition

The number of block-level repetitions, {1, 2} are supported.

Proposal 9: RAN4 can consider the block-level repetition as 2 when defining PDRCH requirement

Maximum TBS

For the D2R transmission, the maximum number of TBS is equal to the maximum message size of 1000 bits for A-IoT.

Proposal 10: RAN4 should further discuss the number of TBS used for defining PDRCH requirements

D2R bandwidth

Regarding the D2R bandwidth, the following was agreed in RAN4 last meeting as

Agreement D2R bandwidth - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1. - Assumed 10% SFO for device - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: - D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: - D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣) - The following are for information: Figure 1-1: Small frequency shift diagram of FDMA without SFO Figure 1-1: SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scale the small frequency shift value - The 2SB transmission BW and small frequency shift both measured in kHz- with and without SFO are as follows - Note: BW and small frequency shift are in kHz, Tc and Tb are in us: 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).

Based on RAN4 agreement, SFO not only affects the D2R transmission bandwidth, but also scale the small frequency shift value. And the assuming 10% SFO for device. RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements

Observation 2: SFO not only affects the D2R transmission bandwidth, but also scale the small frequency shift value

Proposal 11: RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements.

Test metric

Regarding the test metric, the BLER with 1% or 10% was selected for RAN1 A-IoT evaluation. In existing NR demodulation requirement, the SNR with 70% of maximum throughput was applied. RAN4 should further discuss the proper test metric for RAN4 A-IoT demodulation requirement.

Proposal 12: RAN4 should further the proper test metric for defining the A-IoT BS PDRCH demodulation requirement

Channel Model

For A-IoT device, the main deployment should be targeting for indoor scenario with low speed, similar as eMTC and NB-IoT. Therefore, the channel varying is slow. Based on RAN1 evaluation, AWGN and TDL-A channel were selected for feasibility evaluation. We propose to use the AWGN and TDL-A channel as starting point

Proposal 13: RAN4 should further discussion the number of TBS used for defining PDRCH requirement

# 4	Conclusion

In this contribution, the overview on test scope of Ambient-IoT BS demodulation requirement was provided.

Proposal 1: RAN4 focuses on D1T1-B scenario for specifying the A-IoT BS demodulation requirement

Proposal 2: RAN4 focuses on the standalone for specifying the A-IoT BS demodulation requirement.

Proposal 3: RAN4 to define PDRCH the demodulation requirement for Ambient-IoT

Observation 1: RAN1 has designed the preamble-based D2R transmission with D-TAS signal to indicate the starting point of the PDRCH transmission, and designed postamble transmission to indicate the end of the PDRCH transmission

Proposal 4: RAN4 should further discuss whether to include the preamble detection when defining demodulation requirement for PDRCH

Proposal 5: No Random-access requirement will be defined for Ambient-IoT.

Proposal 6: RAN4 should cover both BPSK and OOK modulation scheme when defining PDRCH requirement

Proposal 7: RAN4 will follow the RAN1 agreement related with waveform when defining PDRCH requirement

Proposal 8: RAN4 will follow the RAN1 agreement related with channel coding and FEC when defining PDRCH requirement

Proposal 9: RAN4 can consider the block-level repetition as 2 when defining PDRCH requirement

Proposal 10: RAN4 should further discuss the number of TBS used for defining PDRCH requirements

Observation 2: SFO not only affects the D2R transmission bandwidth, but also scale the small frequency shift value

Proposal 11: RAN4 should further discuss how to select the proper CBW when defining PDRCH requirements.

Proposal 12: RAN4 should further the proper test metric for defining the A-IoT BS PDRCH demodulation requirement

Proposal 13: RAN4 should further discussion the number of TBS used for defining PDRCH requirement

# References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR,