3GPP TSG-RAN WG4 Meeting #116           	R4-2509403

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.13.4

Source:	Samsung

Title:	Simulation results on BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In this contribution, the simulation results with ideal and impair results for BS demodulation with MMSE-IRC are provided for alignment, and requirement definition based on agreed simulation assumption in [1].

# 2	Simulation Results

In this section, the simulation results are provided based on agreed simulation assumption.

Table 1: PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping type | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 12.5 | 14.5 |
| 2 | 1 | 2 | B | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 2.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping type | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 12.0 | 14.0 |
| 2 | 1 | 2 | B | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 3.  PUSCH performance with HomNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 12.6 | 14.6 |
| 2 | 1 | 2 | B | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 4.  PUSCH performance with HetNet scenario under 2Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 12.0 | 14.0 |
| 2 | 1 | 2 | B | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 5.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | 16QAM | 12.8 | 14.8 |
| 2 | 1 | 2 | B | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | 16QAM |  |  |

Table 6.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 11.7 | 13.7 |
| 2 | 1 | 2 | B | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 7.  PUSCH performance with HomNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | 16QAM | 12.7 | 14.7 |
| 2 | 1 | 2 | B | TDLC300-100 | TDLC300-100 | (5.35, N/A) | 1 | 1+1 (2,11) | MCS16 | 16QAM |  |  |

Table 8.  PUSCH performance with HetNet scenario under 2Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | A | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM | 11.7 | 13.7 |
| 2 | 1 | 2 | B | TDLA30-10 | TDLC300-100 | (9.82, N/A) | 1 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 9.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 9.6 | 11.6 |
| 2 | 1 | 4 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 10.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 10.2 | 12.2 |
| 2 | 1 | 4 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 11.  PUSCH performance with HomNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 9.6 | 11.6 |
| 2 | 1 | 4 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 12.  PUSCH performance with HetNet scenario under 4Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 10.3 | 12.3 |
| 2 | 1 | 4 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 13.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 9.9 | 11.9 |
| 2 | 1 | 4 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 14.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 10.5 | 12.5 |
| 2 | 1 | 4 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 15.  PUSCH performance with HomNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 9.8 | 11.8 |
| 2 | 1 | 4 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 16.  PUSCH performance with HetNet scenario under 4Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 10.2 | 12.2 |
| 2 | 1 | 4 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 17.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 5.3 | 7.3 |
| 2 | 1 | 8 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 18.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 5MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 5.8 | 7.8 |
| 2 | 1 | 8 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 19.  PUSCH performance with HomNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM | 5.4 | 7.4 |
| 2 | 1 | 8 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | 16QAM |  |  |

Table 20.  PUSCH performance with HetNet scenario under 8Rx, 15KHz SCS and 20MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.7 | 7.7 |
| 2 | 1 | 8 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 |  |  |

Table 21.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.8 | 7.8 |
| 2 | 1 | 8 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 |  |  |

Table 22.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 10MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.1 | 8.1 |
| 2 | 1 | 8 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 |  |  |

Table 23.  PUSCH performance with HomNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.6 | 7.6 |
| 2 | 1 | 8 | B | TDLC300-100 | TDLC300-100 | (7.28, -2.52) | 2 | 1+1 (2,11) | MCS 16 | MCS16 |  |  |

Table 24.  PUSCH performance with HetNet scenario under 8Rx, 30KHz SCS and 100MHz

| Case 1 | Tx | Rx | Mapping | Channel model (target cell) | Channel model (interferer cells) | (INR1, INR2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SNR@70% (MMSE-IRC) ideal | SNR@70% (MMSE-IRC) impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | A | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.7 | 7.7 |
| 2 | 1 | 8 | B | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1+1 (2,11) | MCS 16 | MCS16 |  |  |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided for alignment and requirement definition.

# References

R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS