3GPP TSG-RAN WG4 Meeting #116           	R4-2509408

Bengaluru, IN, 25th-29th, Aug, 2025

Agenda item:	7.6.5.1

Source:	Samsung

Title:	Discussion on NTN testing NGSO channel model

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the NGSO channel model for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

Issue 1-1-1 Propagation delay model with considering earth’s rotation during signal flight time - Agreement - Downlink propagation delay model at UE reception time t_R with considering the Earth's rotation during signal flight time DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF - FFS whether and how to consider the propagation delay model for UL delay error for RRM uplink timing error cases - Option 1: ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT - Where SV (Satellite) : Receiver UE : Transmitter ∆tUL= tR- tT : Uplink total propagation delay tT : Time when signals are transmitted from UE tR : Time when signals are received by satellite rR : SV position vector on ECI Frame at tT. rT : UE position vector on ECI Frame at tT. VSV : Satellite velocity vector on ECI Frame RUL=rR-rT : Distance between satellite and UE at either tT or tR depending on the reference point. c = 2.99792458 x 108 [m/s] - Other options are not provided

In this contribution, the view on the remaining issue of NTN NGSO testing channel model is provided. Meanwhile, the CR work split is provided.

# 2	Channel Model

RAN4 agreed to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion.  Regarding the propagating delay model, the earth rotation was considered during signal flight time for more accurate estimations of propagation delay.

In the last meeting, RAN4 agreed the channel model for DL side. Regarding the channel model for UL side, RAN4 agreed to further study. Since UL channel model will impact on the UL timing RRM requirement, we think RAN4 should also consider the propagation delay model for UL delay error for RRM uplink timing error cases. For UL side, since satellite is moving, the UL delay error should be considered

Proposal 1: RAN4 should also specify the prorogation delay for UL

ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT

  - Where

SV (Satellite)		: Receiver

UE 				: Transmitter

∆tUL= tR- tT  : Uplink total propagation delay

tT					: Time when signals are transmitted from UE

tR					: Time when signals are received by satellite

rR 					: SV position vector on ECI Frame at tT.

rT 					: UE position vector on ECI Frame at tT.

VSV				: Satellite velocity vector on ECI Frame

RUL=rR-rT	: Distance between satellite and UE at either tT or tR depending on the reference point.

c = 2.99792458 x 108 [m/s]

# 3	CR Work split

Based on the workplan of this work, it is expected to be finished by Feb. 2026. Considering the progress of NTN NGSO channel discussion, and demodulation impact with applying the time-varying Doppler Shift and Propagation delay channel model. The following CR works are expected for both NR NTN and IoT NTN.

| Spec | Section | Company |
| --- | --- | --- |
| Big CR for 38.101-5 |  |  |
| Big CR for 36.102 |  |  |
| 38.101-5 | B.X Channel model |  |
|  | 8.2.1.2, PDSCH demodulation requirement |  |
|  | A.4 Testing related to Satellite Access |  |
|  | Applicability of requirements |  |
| 36.102 | D.X Channel model |  |
|  | 8.2 Demodulation performance requirements for UE category M1 |  |
|  | 8.3 Demodulation performance requirements for UE category NB1 and NB2 |  |
|  | A.3 Testing related to Satellite Access |  |
|  | Applicability of requirements |  |

Proposal 2: Companies are encouraged to provide the draft CR for next meeting

# 4	Conclusion

In this contribution, the view on the NTN testing NGSO channel model is provided.

Proposal 1: RAN4 should also specify the prorogation delay for UL

ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT

  - Where

SV (Satellite)		: Receiver

UE 				: Transmitter

∆tUL= tR- tT  : Uplink total propagation delay

tT					: Time when signals are transmitted from UE

tR					: Time when signals are received by satellite

rR 					: SV position vector on ECI Frame at tT.

rT 					: UE position vector on ECI Frame at tT.

VSV				: Satellite velocity vector on ECI Frame

RUL=rR-rT	: Distance between satellite and UE at either tT or tR depending on the reference point.

c = 2.99792458 x 108 [m/s]

Proposal 2: Companies are encouraged to provide the draft CR for next meeting

# References

R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod, Samsung