# 3GPP TSG-RAN WG4 Meeting #116	                                                	     R4-2509454

# Bangalore, India, 25 – 29 August, 2025

Agenda Item:	7.23.4

Source: 	Apple

Title:	Discussion on RRM performance requirements for NES

Document for:	Discussion

# Introduction

RAN4 RRM performance requirements for NES was initially discussed in the previous meeting. Summary can be found in [1]. In this contribution we provide further discussion on the open issues. After discussion, corresponding proposals are provided.

# Discussion

### Sub-topic 3-1: OD-SSB performance part

Sub-issue 1: Test scope

Issue 3-1-1: Scenario

RAN4 to discuss whether the following test scenarios can be agreed.

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Sub-issue 2: Test case list

Issue 3-1-3: Test case list

RAN4 to discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Overall, we think it is important to introduce test case to verify the new functionality such as Fast Measurement Window. However, RAN4 shall only focus on the ‘delta’ part. Because measurement at physical layer is still same as legacy. It doesn’t make too much sense to repeat testing the same functionality in different activation types such as direct SCell activation, PUCCH Sell activation, SCell Activation with the L3 reporting during activation and so on. The fundamental thing is whether UE can follow the OD-SSB. From this aspect, only testing baseline SCell activation is sufficient.

Proposal 1: test Scenario for OD-SSB:

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as:

  - Case 1 in unknown scenario

  - Case 2 in known scenario

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

Proposal 2: test case list for OD-SSB

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Sub-issue 3: Test configuration

Issue 3-1-4: General principle

RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed.

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 5ms, 20ms. FFS 10ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

We are open to the above test configuration.

### Sub-topic 3-2: OD-SIB1 performance part

Issue 3-2-1: Test case

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

We don’t see the need to have such test, assuming there is signaling test case to verify the overall procedure, including WUS configuration, transmission, OD-SIB1 reception.

Proposal 3: RAN4 not to define OD-SIB1 RRM performance test case.

### Sub-topic 3-3: SSB adaptation performance part

Issue 3-3-1: Test case list

RAN4 to discuss whether the following SSB adaptation test case list can be agreed.

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- direct SCell activation

- PUCCH SCell activation

Basically the fundamental testing point is whether UE can follow the adapted SSB timely. From this point of view, it doesn’t really matter the SSB is used for which functionality. Therefore, the following two cases are enough in our understanding:

- L1-RSRP

- L3 measurement

- normal SCell activation

Proposal 4: test case list for SSB adaptation:

- L1-RSRP

- L3 measurement

- normal SCell activation

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- Both FR1 and FR2

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement

- Only test non-DRX mode

- without SSB time index detection

Issue 3-3-3: SMTC and SSB configuration

- Proposals

  - Option 1:

    - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation.

  - Option 2:

    - SSB periodicity: 40 ms to 20 ms

    - SMTC periodicity: 40ms

  - Option 3:

    - SSB periodicity from 80ms to 20ms

    - SMTC periodicity from 80ms to 20ms

We are open to the above issue 3-3-2 and 3-3-3.

# Conclusion

In this contribution, we provide discussion on performance part of NES. After discussion the following conclusions are provided:

Proposal 1: test Scenario for OD-SSB:

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as:

  - Case 1 in unknown scenario

  - Case 2 in known scenario

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

Proposal 2: test case list for OD-SSB:

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Proposal 3: RAN4 not to define OD-SIB1 RRM performance test case.

Proposal 4: test case list for SSB adaptation:

- L1-RSRP

- L3 measurement

- normal SCell activation

# Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ercisson