3GPP TSG-RAN WG4 Meeting #116	R4-2509530

# Bangalore, India,  25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0081 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band-Core) Introduction of AMPR for the IOT NTN band 254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NTN IOT band 254 has several NS flags, some of which are associated with the corresponding ETSI requirements that contain both out-of-band and in-band emission requirements. Since ETSI in-band emission requirements are not excatly the same as the baseline NB-IOT emission mask, additional power back-off might be needed for some device implementations to be compliant with the ETSI requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A-MPR values are added for the NTN IOT band 254 flags NS_04N, NS_05N, NS_11N and NS_12N. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN IOT band 254 will have uncompleted performance requirements as it will be unclear whether A-MPR is defined or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3, 6.2B3.1(new), 6.2B3.2(new), 6.2B3.3(new), 6.2B3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This revision of the CR has all A-MPR values as [TBD], which should be changed to proper values in the next revision of the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.4.4.5 | 254 | sub-clause 6.2B.3.1 |  |
| NS_05N | 6.5B.4.4.5 | 254 | sub-clause 6.2B.3.2 |  |
| NS_11N | 6.5B.4.4.5 | 254 | sub-clause 6.2B.3.3 |  |
| NS_12N | 6.5B.4.4.6 | 254 | sub-clause 6.2B.3.4 |  |

#### 6.2B.3.1	A-MPR for NS_04N

Table 6.2B.3.1-1: A-MPR for "NS_04N" for Power Class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | [TBD] |  | [TBD] |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | [TBD] |  |  |  |

#### 6.2B.3.2	A-MPR for NS_05N

Table 6.2B.3.2-1: A-MPR for "NS_05N" for Power Class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | [TBD] |  | [TBD] |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | [TBD] |  |  |  |

#### 6.2B.3.3	A-MPR for NS_11N

Table 6.2B.3.3-1: A-MPR for "NS_11N" for Power Class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | ≤ 2 dB |  | [TBD] |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | [TBD] |  | [TBD] |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | [TBD] |  |  |  |

#### 6.2B.3.4	A-MPR for NS_12N

Table 6.2B.3.4-1: A-MPR for "NS_12N" for Power Class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | ≤ 2 dB |  | [TBD] |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | [TBD] | 0 dB |  | [TBD] |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | [TBD] |  | [TBD] |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | [TBD] |  |  |  |