3GPP TSG-RAN WG4 Meeting #116                	                R4-2509594

Bengaluru, IN, 25th - 29th Aug 2025

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			7.25.3.3

Document for:	Discussion

- Introduction

In RAN4#115 meeting, some conclusions on conditional intra-CU LTM have been duplicated as follows [1]. From this meeting, we will continue to discuss the RRM Core requirements on conditional intra-CU LTM.

- Discussion

Issue 1-2-2: Tmeasure

For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS.

In R18 LTM, the requirements on LTM only apply when the target cell is known. So, we think for CSI-RS based L1-RSRP measurement, do not include unknown cell case.

For the conditions for TCSI-RS_SFN_intra = 0, we think the conditions for CSI-RS based L3 measurements could be reused, i.e., the time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). But the final conclusion should follow the CSI-RS based L1 measurement for LTM in part1.

Proposal 1: TCSI-RS_SFN_intra = 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). But the final conclusion should follow the CSI-RS based L1 measurement for LTM in part1.

For CSI-RS based L1-RSRP measurement, do not include unknown cell case.

Issue 1-2-5: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

< Agreement > ad hoc - Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list. - Note: the limitation of 160/480 ms from Rel-18 still applies.

Since in CLTM, no cell switch command, some descriptions need to be updated. We have made revisions in the CR.

Proposal 2: Some descriptions of the values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 need to be updated. They could be found in R4-2509595.

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: TCSI-RS_SFN_intra = 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). But the final conclusion should follow the CSI-RS based L1 measurement for LTM in part1.

For CSI-RS based L1-RSRP measurement, do not include unknown cell case.

Proposal 2: Some descriptions of the values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 need to be updated. They could be found in R4-2509595.

- References

[1] R4-2508283, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom.