3GPP TSG-RAN WG4 Meeting #116	R4-2509620

Bengaluru, India, 25 August 2025 – 29 August 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for SBFD operation

Agenda item:	7.21.5

Document for:	Discussion

Introduction

In the past RAN4 meetings, RAN4 has discussed the impact of the introduction of SBFD symbols on the CLI measurements and RRM requirements in TS 38.133. In RAN4#114bis meeting, RAN4 agreed on the TS 38.133 clauses that would be modified to support SBFD symbols (R4-2504904). A summary of the affected clauses is given below:

Impacted Core requirements and clauses

9.x L1 CLI measurement (new clause)

8.1.3.2 Minimum requirement

8.5.3.2 Minimum requirement

8.5.6.2 Minimum requirement

9.5.4.2 CSI-RS based L1-RSRP Reporting

9.8.4.1 L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

9.8.4.2 L1-SINR reporting with SSB based CMR and dedicated IMR configured

9.8.4.3 L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

9.10.1 Introduction

9.1.2 Measurement gap

8.6 Active BWP switch delay

Others, if identified

In this contribution, we discuss the impact of introducing the SBFD feature on the RRM performance and test cases.

Performance Requirements

Support for sub-band non-overlapped full duplex (SBFD) is being introduced in Rel-19. In SBFD operation, the gNB is able to transmit and receive simultaneously in a TDD carrier, but in non-overlapping frequency resources. Due to different transmitter and receiver imperfections, SBFD faces the challenges of cross-link interference (CLI) between UEs and between gNBs. Figure 1 illustrates the CLI types studied in RAN1 and RAN4, namely 1) gNB self-interference, 2) gNB-to-gNB inter-sector (or intra-site) interference, 3) gNB-to-gNB inter-site interference and UE-to-UE 4) intra-cell and 5) inter-cell interference.

Figure 1: CLI types present in SBFD

During Rel-19, new core requirements have been defined in the NR_Duplex_Evo work in RAN4 RRM: requirements for the UE-to-UE CLI measurements, L1-SRS-RSRP and L1-CLI-RSSI. Apart from that, RAN4 has also defined requirements for cases in which the CSI-RS is punctured, due to overlapping of configured CSI-RS resources with guard-bands and UL subband. It has not been agreed yet on what will be the core requirements for L3 measurements based on CSI-RS.

SBFD introduces cross-link interference between the DL and UL subbands, i.e., in non-overlapping resources.

Regarding the UE-to-UE CLI, the L1-SRS-RSRP measurement is configured in the UL subband of a SBFD symbol, and we think that RAN4 should introduce new performance requirements for L1-SRS-RSRP measurements.

RAN4 to introduce performance requirements for CLI measurements for L1-SRS-RSRP measurements.

The L1-CLI-RSSI measurement can be configured in the UL subband (Method#3) or in the DL subband(s) (Method#1) of a SBFD symbol. In the latter case, the L1-CLI-RSSI measurement resources can be confined within one DL subband (resource type #1) or configured across DL subbands (resource type #2). We think that RAN4 should introduce new performance requirements for L1-CLI-RSSI measurements. However, we do not see the need to cover all possible methods and resource types. In fact, given that a CLI-RSSI measurement is simply a power measurement across the configured time and frequency resources, we do not expect any difference from the performance point of view across the different assumptions on measurement methods and resource types.

RAN4 to introduce performance requirements for CLI measurements for L1-CLI-RSSI measurements.

From performance point of view, there is no difference on whether Method #1 with resource type #1 or Method#1 with resource type#2 or Method#3 are assumed.

In Rel-16, RAN4 introduced requirements for CLI measurements, namely SRS-RSRP and CLI-RSSI, to support dynamic TDD operation. In dynamic TDD, the requirements were defined assuming inter-cell CLI (number 5 in Figure 1), from overlapping frequency resources, but in Rel-19, the focus is on intra-cell UE-to-UE CLI from non-overlapping resources.

The side conditions of the Rel-16 CLI measurements performance requirements are defined under the assumption of inter-cell CLI in overlapping resources. When we consider intra-cell UE-to-UE in SBFD symbols, the CLI comes from non-overlapping frequency resources.

Focusing on intra-cell UE-to-UE CLI for Rel-19 have the following implications:

Timing offset for L1-SRS-RSRP

The cell phase synchronization is zero since both victim UE and aggressor UE are assumed in the same cell. As the propagation delay difference is derived assuming the victim and aggressor UEs are located in the cell center and cell edge respectively, it remains the same irrespective of intra-cell or inter-cell. Hence the residual timing error can be derived assuming cell phase synchronization error is zero and same cell size as in R16. This would mean that, for the FR1 inter-cell scenario, the timing offset considered is up to 1.67 us. Note that for FR1 inter-cell scenario, the timing offset was 4.67 us since the cell phase synchronization error was included. Similarly, for FR2, timing offset is 1.67 us for inter-cell and 0.67 us for intra-cell.

For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase synchronization is zero and same cell size as in Rel-16.

RAN4 to define the performance requirements for L1-SRS-RSRP measurements focusing on the 1.67 us timing offset in FR1 and 0.67 us timing offset in FR2.

SRS Es/IoT

The SRS Es/Iot value may be higher considering the L1-SRS-RSRP measures the interference on UL subband where there is no other DL transmission hence less interference can be expected in the victim UE as compared to existing L3 SRS-RSRP measurements. Current simulation assumptions for L1-SRSP RSRP measurement requirements cover SRS SNR values of 1 dB, 2 dB and 3 dB. We think RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

RAN4 should define a higher value of SRS Es/Iot as side condition for the L1-SRS-RSRP measurement on the UL subband in SBFD operation, when compared to the L3 SRS-RSRP side conditions.

SRS bandwidth

Current simulation assumptions for L1-SRSP RSRP measurement requirements assume SRS bandwidths of 24 PRBs and 48 PRBs. Note that existing SRS-RSRP measurement requirements assume an SRS bandwidth of 48 PRBs. Based on our results, we conclude that the measurement reports show similar performance with both bandwidth values. Given that certain SBFD configurations might have a limited UL subband size, we prefer that L1-SRS-RSRP measurement requirements should consider an SRS bandwidth of 24 PRBs.

The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP performance requirements.

Test cases

- 5.1 Test case coverage

SBFD has been specified on top of existing TDD operation. Therefore, it should be clear that the SBFD aware UE is also tested in TDD mode, considering the existing TDD test case list. The new test cases defined for SBFD aware UEs should be additional to any TDD test cases that are applicable to Rel-19 UEs. In case the SBFD tests are defined in the same clauses as the existing TDD test cases, as a new configuration, RAN4 should make it clear that a TDD and a SBFD configuration should be tested.

TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs.

- 5.2 Test case configuration

In Rel-16, the CLI test cases were defined for standalone (SA) and dual connectivity (DC). In an earlier RAN2 meeting, RAN2 sent an LS to RAN1[3] asking whether SBFD and DC can be configured simultaneously. RAN1 replied that SBFD and NR-DC can be configured simultaneously from RAN1 perspective, although it is up to RAN2 to take a decision whether to support it or not (considering also the potential specification impact to RAN4).

It is under discussion in RAN2 whether SBFD can be configured with DC.

RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN4 on the support of SBFD with DC.

Introduce the SBFD test cases in clauses A6 (SA in FR1) and A7 (FR2) in TS 38.133.

Additionally, there is need to provide a new TDD configuration that supports SBFD symbols. For simplicity, to ensure that the SSBs are overlapping with SBFD symbols, we propose a TDD configuration that does not contain any DL symbols to be used in the SBFD test cases: XXXXU, for example.

Introduce a new TDD UL/DL configuration for RRM tests in FR1 and FR2, for a 30 kHz SCS and 120 kHz respectively: DDDSU.  For the SBFD time domain configuration, all the Downlink and flexible symbols in the TDD configuration are assigned as SBFD symbols, so that the resulting SBFD configuration is XXXXU.

Another important aspect is the test case configuration, in terms of the BW of the channel and of each subband. We note that the feasibility study of SBFD done in Rel-18 has considered a channel BW of 100 MHz, with a DUD configuration, in which the BW of the DL subband – UL subband and DL subband were 40 MHz, 20 MHz and 40 MHz respectively.

The SBFD test case configuration considers at least a FR1 TDD carrier with 100 MHz channel bandwidth, for a 30 kHz SCS, in DUD configuration, in which 40 MHz – 20 MHz – 40 MHz are assigned to DL, UL and DL subbands respectively, following the configuration used for the feasibility study in Rel-18 (TS 38.858). In FR2, the configuration is: 100 MHz channel bandwidth, 120 kHz SCS and a 40 MHz – 20 MHz – 40 MHz DUD.

Due to the large bandwidth size required to accommodate the different SBFD subbands, a new configuration of SSB for the test cases will need to be provided. Furthermore, according to the RAN1 working assumptions, the SSBs are always confined in the DL subband of a SBFD symbol. Currently, there is no such restriction in the existing SSB configurations. Considering these two aspects, we propose the following configuration to be used in the SBFD test cases, in which the values marked in red were adapted from the existing SSB Pattern 2 in FR1 (A.3.10.1.2 from TS 38.133).

Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 100 MHz |
| SSB SCS | 30 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 3 | 4-7 or 2-5 Note 2 |
| Slot numbers containing SSB Note 3 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: Symbols 4-7 is chosen, if the SSB pattern Case B should be used for the current band as indicated by table 5.4.3.3-1 of TS 38.104 [13]; Otherwise, symbol 2-5 is chosen. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves |  |

For FR2, our view is that SSB.1 FR2 configuration can be reused for SBFD symbols, as long as note 1 is modified to ensure that the SSB is placed within a DL subband.

Modify the note for SSB.1 FR2 configuration, to restrict the placement of the SSB in FR2 to RBs confined within the DL subband in SBFD symbols, as follows: “NOTE 1:	In DL or flexible symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13].”

Table A.3.10.2.1-1: SSB.1 FR2: SSB Pattern 1 for SSB SCS = 120 kHz in 100 MHz channel with 2 SSBs per SS-burst

| SSB Parameters | Values |  |
| --- | --- | --- |
| Channel bandwidth | 100 MHz |  |
| SSB SCS | 120 kHz |  |
| SSB periodicity (TSSB) | 20 ms |  |
| Number of SSBs per SS-burst | 2 |  |
| SS/PBCH block index | 0 | 1 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 |
| Slot numbers containing SSB Note 2 | 0 | 0 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |  |
| PRB numbers containing SSBs within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |  |
| NOTE 1: In DL or flexible symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

Test case list

Since new RRM core requirements have been introduced for the support of the SBFD feature, RAN4 should define new test cases, at least to cover the requirements in clauses agreed to be modified in the last RAN4 meeting. In some cases, even though the requirement is the same as legacy NR, RAN4 should define test cases for operation in SBFD symbol to ensure that the requirements are met. This is the case for SSB L1 and L3 measurements.

In the table below, we discuss the clauses with affected Core requirements, as agreed in RAN4 #114bis [1]. In the bottom of the table, RACH test cases are also proposed, to cover the procedure defined by RAN2.

RAN4 to consider the test case list provided in table 1:

Table 1 - List of test cases

| Core requirements clauses | New test case clauses |  | Comments |
| --- | --- | --- | --- |
| 8.1.3.2 | Minimum requirement | A.6.5.1.X (in-sync) FR1 A.6.5.1.Y (out-of-sync) FR1 A.7.5.1.X (in-sync) FR2 A.7.5.1.Y (out-of-sync) FR2 | CSI-RS based RLM A new test case to test the requirements in SBFD symbols should be defined. |
| 8.5.3.2 | Minimum requirement | A.6.5.5.X (FR1) A.7.5.5.X (FR2) | CSI-RS based BFD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.5.6.2 | Minimum requirement | A.6.5.5.X (same as above) A.7.5.5.X (same as above) | CSI-RS based CBD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.6 | Active BWP switch delay |  | - |
| 9.1.2 | Measurement gap |  | - |
| 9.2 | Intra-frequency measurements | A.6.6.1.X Intra-frequency measurements (FR1) A.7.6.1.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing intra-frequency measurements within the legacy requirements. |
| 9.3 | Inter-frequency measurements | A.6.6.2.X Intra-frequency measurements (FR1) A.7.6.2.X Intra-frequency measurements (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing inter-frequency measurements within the legacy requirements. |
| 9.5.4.1 | SSB based L1-RSRP measurement | A.6.6.4.X1 (FR1) A.7.6.3.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.5.4.2 | CSI-RS based L1-RSRP Reporting | A.6.6.4.X2 (FR1) A.7.6.3.X2 (FR2) | Test cases are needed to test L1-RSRP measurement in DU and DUD configurations. |
| 9.8.4.2 | SSB based L1-SINR | A.6.6.8.X1 (FR1) A.7.6.6.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.8.4.1 | L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | A.6.6.8.X2 (FR1) A.7.6.6.X2 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.2 | L1-SINR reporting with SSB based CMR and dedicated IMR configured | A.6.6.8.X3 (FR1) A.7.6.6.X3 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.3 | L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured | A.6.6.8.X4 (FR1) A.7.6.6.X4 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.10.1 | Introduction | A.6.6.10.X (intra-frequency measurements) A.6.6.11.X (inter-frequency measurements) (FR1) A.7.6.7.X (intra-frequency measurements) A.7.6.8.X (inter-frequency measurements) (FR2) | CSI-RS based L3 measurements Test cases are needed to test L3 measurements in DU configurations, and for DUD |
| 9.x | L1-CLI measurement (new clause) | A.6.6.X L1-CLI measurements (FR1 SA) A.7.6.X L1-CLI measurements (FR2) | New measurements L1-CLI-RSSI and L1-SRS-RSRP are defined in Rel-19. There should be test cases for both L1-CLI-RSSI and L1-SRS-RSRP. For L1-CLI-RSSI two test cases could be defined: one for measurement resources confined in one sub-band, and one for measurement resources confined in two sub-bands. |
| Clauses affected – pending RAN4 agreement |  |  |  |
| 6.2.2 | Random Access | A.4.3.2.2.X (FR1) A.7.3.2.2.X (FR2) | Test cases to test the RO type selection and fall back defined by RAN2 |

Conclusion

The following Observations and Proposals were made:

Observation 1: SBFD introduces cross-link interference between the DL and UL subbands, i.e., in non-overlapping resources.

Proposal 1: RAN4 to introduce performance requirements for CLI measurements for L1-SRS-RSRP measurements.

Proposal 2: RAN4 to introduce performance requirements for CLI measurements for L1-CLI-RSSI measurements.

Proposal 3: From performance point of view, there is no difference on whether Method #1 with resource type #1 or Method#1 with resource type#2 or Method#3 are assumed.

Observation 2: The side conditions of the Rel-16 CLI measurements performance requirements are defined under the assumption of inter-cell CLI in overlapping resources. When we consider intra-cell UE-to-UE in SBFD symbols, the CLI comes from non-overlapping frequency resources.

Proposal 4: For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase synchronization is zero and same cell size as in Rel-16.

Proposal 5: RAN4 to define the performance requirements for L1-SRS-RSRP measurements focusing on the 1.67 us timing offset in FR1 and 0.67 us timing offset in FR2.

Proposal 6: RAN4 should define a higher value of SRS Es/Iot as side condition for the L1-SRS-RSRP measurement on the UL subband in SBFD operation, when compared to the L3 SRS-RSRP side conditions.

Proposal 7: The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP performance requirements.

Proposal 8: TDD test cases are also applicable to SBFD aware UEs. The new test cases introduced for SBFD aware UEs are additional to the test cases defined for TDD UEs.

Observation 3: It is under discussion in RAN2 whether SBFD can be configured with DC.

Proposal 9: RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN4 on the support of SBFD with DC.

Proposal 10: Introduce the SBFD test cases in clauses A6 (SA in FR1) and A7 (FR2) in TS 38.133.

Proposal 11: Introduce a new TDD UL/DL configuration for RRM tests in FR1 and FR2, for a 30 kHz SCS and 120 kHz respectively: DDDSU.  For the SBFD time domain configuration, all the Downlink and flexible symbols in the TDD configuration are assigned as SBFD symbols, so that the resulting SBFD configuration is XXXXU.

Proposal 12: The SBFD test case configuration considers at least a FR1 TDD carrier with 100 MHz channel bandwidth, for a 30 kHz SCS, in DUD configuration, in which 40 MHz – 20 MHz – 40 MHz are assigned to DL, UL and DL subbands respectively, following the configuration used for the feasibility study in Rel-18 (TS 38.858). In FR2, the configuration is: 100 MHz channel bandwidth, 120 kHz SCS and a 40 MHz – 20 MHz – 40 MHz DUD.

Proposal 13: Introduce a new SSB configuration for FR1 SBFD test cases, as follows:

Proposal 14: Modify the note for SSB.1 FR2 configuration, to restrict the placement of the SSB in FR2 to RBs confined within the DL subband in SBFD symbols, as follows: “NOTE 1:In DL or flexible symbols, PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. In SBFD symbols, PRBs containing SSB shall be configured in frequency locations completely contained in a DL subband within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13].”

Proposal 15: RAN4 to consider the test case list provided in table 1:

Table 1 - List of test cases

| Core requirements clauses | New test case clauses |  | Comments |
| --- | --- | --- | --- |
| 8.1.3.2 | Minimum requirement | A.6.5.1.X (in-sync) FR1 A.6.5.1.Y (out-of-sync) FR1 A.7.5.1.X (in-sync) FR2 A.7.5.1.Y (out-of-sync) FR2 | CSI-RS based RLM A new test case to test the requirements in SBFD symbols should be defined. |
| 8.5.3.2 | Minimum requirement | A.6.5.5.X (FR1) A.7.5.5.X (FR2) | CSI-RS based BFD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.5.6.2 | Minimum requirement | A.6.5.5.X (same as above) A.7.5.5.X (same as above) | CSI-RS based CBD A new test case to test the requirements in SBFD symbols should be defined. |
| 8.6 | Active BWP switch delay |  | - |
| 9.1.2 | Measurement gap |  | - |
| 9.2 | Intra-frequency measurements | A.6.6.1.X Intra-frequency measurements (FR1) A.7.6.1.X Intra-frequency measurements (FR1) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing intra-frequency measurements within the legacy requirements. |
| 9.3 | Inter-frequency measurements | A.6.6.2.X Intra-frequency measurements (FR1) A.7.6.2.X Intra-frequency measurements (FR1) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing inter-frequency measurements within the legacy requirements. |
| 9.5.4.1 | SSB based L1-RSRP measurement | A.6.6.4.X1 (FR1) A.7.6.3.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.5.4.2 | CSI-RS based L1-RSRP Reporting | A.6.6.4.X2 (FR1) A.7.6.3.X2 (FR2) | Test cases are needed to test L1-RSRP measurement in DU and DUD configurations. |
| 9.8.4.2 | SSB based L1-SINR | A.6.6.8.X1 (FR1) A.7.6.6.X1 (FR2) | The configuration of SSBs in SBFD symbols is not precluded. Therefore, the UE should be capable of performing SSB based L1 measurements within the legacy requirements. |
| 9.8.4.1 | L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured | A.6.6.8.X2 (FR1) A.7.6.6.X2 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.2 | L1-SINR reporting with SSB based CMR and dedicated IMR configured | A.6.6.8.X3 (FR1) A.7.6.6.X3 (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.8.4.3 | L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured | A.6.6.8.X4 (FR1) A.7.6.6.X (FR2) | Test cases are needed to test L1-SINR reporting in DU and DUD configurations. |
| 9.10.1 | Introduction | A.6.6.10.X (intra-frequency measurements) A.6.6.11.X (inter-frequency measurements) (FR1) A.7.6.7.X (intra-frequency measurements) A.7.6.8.X (inter-frequency measurements) (FR2) | CSI-RS based L3 measurements Test cases are needed to test L3 measurements in DU configurations, and for DUD |
| 9.x | L1-CLI measurement (new clause) | A.6.6.X L1-CLI measurements (FR1 SA) A.7.6.X L1-CLI measurements (FR2) | New measurements L1-CLI-RSSI and L1-SRS-RSRP are defined in Rel-19. There should be test cases for both L1-CLI-RSSI and L1-SRS-RSRP. For L1-CLI-RSSI two test cases could be defined: one for measurement resources confined in one sub-band, and one for measurement resources confined in two sub-bands. |
| Clauses affected – pending RAN4 agreement |  |  |  |
| 6.2.2 | Random Access | A.4.3.2.2.X (FR1) A.7.3.2.2.X (FR2) | Test cases to test the RO type selection and fall back defined by RAN2 |

- References

- R4-2504904, WF on SBFD RRM, Huawei

- R4-2503623, summary [114][222] NR_duplex_evo, Huawei

- R2-2503191, LS on simultaneous configuration of SBFD and DC, RAN2