3GPP TSG-RAN WG4 Meeting #116	R4-2509632

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Low Band CA operation

Agenda item:	7.3.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM issues for low band CA via switching between FDD and SDL CC based on the WF [1] and RAN1 agreements on switching pattern below.

RAN1#121 agreements Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

Discussion

General

MRTD assumptions for R19 LB CA via switching

Agreement: - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue.

RAN4 agreed that maximum received timing difference for collocated PCell and SCell for LB CA is 3us. In the last meeting, RF session agreed time mask for switching gap as below

- FDD to SDL switch

  - Actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period.

- SDL to FDD switch

  - Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period

- where the MRTD = [CP length/TAE] assuming that the FDD and SDL carriers are colocated.

The 3us maximum RTD should be applied to time mask for switching gap in RF specification, and no futher discussion for the maximum RTD is needed in RAN4 RRM.

- Observation 1: The 3us maximum RTD should be applied to time mask in RF specification, and no futher discussion for the maximum RTD is needed in RAN4 RRM

Interruption or scheduling restriction requirement

Necessity of interruption requirement in RRM between FDD CC and SDL CC

Agreement: - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement.

Based on RAN1 and RF agreements, switching gap is configured for and located at the “switch from” carrier for UE to switch carrier according to switching pattern. The switching gap length includes TTA, maximum RTD, RF switching time, and transient period. Therefore, no additional interruption requirements in RRM is required.

- Proposal 1: RAN4 to confirm the agreement from the last RAN4 meeting

  - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

SDL SCell related requirement

Activated SDL SCell L3 measurement

To define activated SDL SCell L3 measruement, RAN4 agreed the principle of requirement design as below.

Agreement: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. - FFS - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply - FFS on details of requirement design.

SCell L3 measurements are performed on the overlapped occasions of SMTC and SDL SCell reception duration as shown in Fig 1, so SCell L3 measurement period should be extended by scaling factor related switch pattern.

Figure 1 Example of activated SDL SCell measurement considering switching pattern

The scaling factor could be

- KLBCA = Ntotal / Navailable

- For a window W of duration max(TSMTC, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern, and TSMTC is SMTC periodicity

- Ntotal is the number of SMTC occasions within the window W, and

- Navailable is the number of SMTC occasions overlapped SCell reception duration by switching pattern within window W.

- Proposal 2: Introduce new scaling factor for activated SDL SCell L3 measurement period

  - KLBCA = Ntotal / Navailable

- For a window W of duration max(TSMTC, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern, and TSMTC is SMTC periodicity

- Ntotal is the number of SMTC occasions within the window W, and

- Navailable is the number of SMTC occasions overlapped SCell reception duration by switching pattern within window W.

The scaling factor can be used for PCell measurement period by switching pattern.

Iterruption or scheduling restriction during SDL SCell activation

Agreement: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements FFS - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation. - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern.

RAN4 agreed that Rel-18 SSB based inter-band SCell activation dealy requirement can be reused for SDL SCell activation in LB CA case. As shown in Fig 2, after THARQ and MAC-CE processing, UE should switch SCell to measure SCell SSB and then back to PCell after measuring SCell SSB. Therefore, during the switching PCell to SCell / SCell to PCell and SCell SSB or CSI measurement, scheduling restriction/interruption for PCell should be defined. The scheduling(measurement) restriction/interruption length for SCell measurement in SDL SCell activation procedure can be 2*switching gap length(without TTA) + SSB or CSI measurement duration.

Figure 2 Example of SCell SSB/SMTC measurement for SCell activation disregardless of switching pattern

- Proposal 3: Define scheduling restriction/interruption for PCell to perform SCell measurements after THARQ and MAC-CE processing during SCell activation, and the scheduling restriction/interruption length is

  - ceil( [2*switching gap length (without TTA) + SCell SSB or CSI measurement duration] / NR slot length )

In case the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation, SCell measurement for SCell activation should be prioritized to avoid additional delay of SCell activation delay.

- Proposal 4: If the CQI or L1-RSRP reporting is colliding with SCell measurement during SDL SCell activation, the SCell measurement should be prioritized.

Activation of Switching Pattern

Before SCell activation (i.e., before CSI reporting), measurements for the SCell are performed while switching between the PCell and SCell regardless of the switching pattern. Since RAN4 agreed to apply the switching pattern only for an activated SCell, the switching pattern is applied after CSI reporting. More precisely, it is applied from the first switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) following CSI reporting as shown in Fig 3. Therefore, all measurement requirements related to the switching pattern should be applied only after the first switching gap following CSI reporting.

Figure 3 Example of activation of switching pattern

- Proposal 5: The activation of switching pattern takes effect from the first switching gap after CSI reporting for SCell activation competion, and the RRM requirements affected by the switching pattern should be applied after switching pattern activation.

BFD/CBD requirements for SDL SCell

Regarding BFD/CBD, there can be an issue when the UE declare beam failure in SDL SCell. Usually, a UE evaluate DL beam quality of serving cell utilizing RS set in q0. If every the DL beam quality of RS in q0 are below Qout_LR, physical layer sends BFI (beam failure indication) to higher layer. If the certain number of BFI is occurred within cerain time window, the UE can declare beam failure, and try to transmit PRACH or PUCCH (if schedulingRequestIDForBFR is configured). After transmit PRACH/PUCCH, the UE can transmit MAC-CE providing index of BFD-RS and CBD-RS for a corresponding SCell. The problem is when the UE needs to transmit PRACH/PUCCH, there is no UL symbol in SDL SCell.

- Observation 2: When beam failure is declared in SDL SCell, no UL resource exists to transmit PRACH/PUCCH

We think it is natural to switch back to PCell in case of beam failure. For instance, the UE can switch to PCell right after the UE declares beam failure, or before transmitting PRACH/PUCCH considering RF switching delay. Furthermore, in order to perform beam failure recovery process properly and avoid additional delay due to switching pattern, it would be helpful for the UE to pause the switching pattern during link recovery, and resume the switching pattern when beam failure recovery is completed. For instance, as illustrated in Figure 4, if a UE is configured by schedulingRequestIDForBFR, and beam failure is triggered in SDL SCell, the UE can perform CBD evaluation, and needs to provide PUCCH followed by PUSCH MAC-CE. In order to perform beam recovery procedure faster, CBD evaluation can be performed in SDL SCell, which means that scheduling restriction/interruption can be occurred. Regardless of CBD evaluation, the UE can assume that switching pattern is paused until beam failure recovery is completed. Hence, RAN4 need to consider the requirement during the SCell link recovery procedure without switching pattern (i.e., legacy requirements are applied). During SCell link recovery procedure, scheduling restriction/interruption for PCell would be applied.

- Proposal 6: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggred.

Figure 4 Example of SCell BFR for LB CA

TCI state switch delay requirement for SDL SCell

Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

For TCI switching on SDL SCell considering the switching pattern, TCI state switch delay requirements should be based on only overlapped occasions between SSB/CSI-RS and SDL SCell duration in switching pattern. Considering the switching pattern, the definition of Tfirst-SSB is affected. Tfirst-SSB should be time to first available SSB transmission on the corresponding cell as configured via the switching pattern after either MAC-CE decoding or RRC processing. And the corresponding draft CR [2] for TCI state switch delay is provided in our companion paper.

- Proposal 7: The definition of Tfirst-SSB for TCI state switch delay in LB CA via switching should be

  - time to first available SSB transmission on the corresponding cell as configured via the switching pattern after either MAC-CE decoding or RRC processing

Impact on PCell RRM requirements

PCell L3 measurement

RAN1 agreed to switching pattern with 40 ms periodicity, but there can be a case that PCell SMTC/RS can be overlapped with SCell DL reception duration by switching pattern. Silimar to activated SDL SCell L3 measurement, PCell measurement should be performed on the overlapped occasions of PCell SMTC/RS and PCell reception duration indicated by switching pattern. So, RAN4 needs to consider extending PCell measurement requirement based on switching pattern introducing scaling factor. The scaling factor Kp could be

- Kp = Ntotal / Navailable

- Window W = max(TSMTC, MGRP, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern and TSMTC is SMTC periodicity.

- Ntotal = total number of SMTC occasions within the window W

- Navailable = the number of SMTC occasions not overlapped MG and overlapped PCell reception duration by switching pattern

- Proposal 8: Introduce new sacling factor for PCell measurement requirements after switching pattern activation

  - Kp = Ntotal / Navailable

- Window W = max(TSMTC, MGRP, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern and TSMTC is SMTC periodicity.

- Ntotal = total number of SMTC occasions within the window W

- Navailable = the number of SMTC occasions not overlapped MG and overlapped PCell reception duration by switching pattern

BFD/CBD requirements for PCell

Regarding BFD/CBD for PCell, as same logic stated in BFD/CBD requirement for SDL SCell, the UE needs to transmit PRACH, and perform RACH procedure. It is not helpful to keep switching to SDL SCell when beam failure is triggered in PCell. Since co-located scenario is assumed, beam failure in PCell would degrade the performance of SCell as well as PCell. Thefore, as illustrated in Figure 5, if BFR is triggered for PCell, the UE can stop switching pattern until beam failure recovery is completed for faster receovery process.

- Proposal 9: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for PCell.

Figure 5 Example of PCell BFR for LB CA

Conclusion

In this contribution, we provide our views on remaining RRM issues for low band CA via switching between FDD and SDL CC, and we propose

- Observation 1: The 3us maximum RTD should be applied to time mask in RF specification, and no futher discussion for the maximum RTD is needed in RAN4 RRM

- Proposal 1: RAN4 to confirm the agreement from the last RAN4 meeting

  - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier.

- Proposal 2: Introduce new scaling factor for activated SDL SCell L3 measurement period

  - KLBCA = Ntotal / Navailable

- For a window W of duration max(TSMTC, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern, and TSMTC is SMTC periodicity

- Ntotal is the number of SMTC occasions within the window W, and

- Navailable is the number of SMTC occasions overlapped SCell reception duration by switching pattern within window W.

- Proposal 3: Define scheduling restriction/interruption for PCell to perform SCell measurements after THARQ and MAC-CE processing during SCell activation, and the scheduling restriction/interruption length is

  - ceil( [2*switching gap length (without TTA) + SCell SSB or CSI measurement duration] / NR slot length )

- Proposal 4: If the CQI or L1-RSRP reporting is colliding with SCell measurement during SDL SCell activation, the SCell measurement should be prioritized.

- Proposal 5: The activation of switching pattern takes effect from the first switching gap after CSI reporting for SCell activation competion, and the RRM requirements affected by the switching pattern should be applied after switching pattern activation.

- Observation 2: When beam failure is declared in SDL SCell, no UL resource exists to transmit PRACH/PUCCH

- Proposal 6: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggred.

- Proposal 7: The definition of Tfirst-SSB for TCI state switch delay in LB CA via switching should be

  - time to first available SSB transmission on the corresponding cell as configured via the switching pattern after either MAC-CE decoding or RRC processing

- Proposal 8: Introduce new sacling factor for PCell measurement requirements after switching pattern activation

  - Kp = Ntotal / Navailable

- Window W = max(TSMTC, MGRP, Tswitching_pattern), where Tswitching_pattern is the periodicity of semi-static switching pattern and TSMTC is SMTC periodicity.

- Ntotal = total number of SMTC occasions within the window W

- Navailable = the number of SMTC occasions not overlapped MG and overlapped PCell reception duration by switching pattern

- Proposal 9: RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for PCell.

Reference

[1] R4-2504895, “WF on RRM requirements for NR_LBCA_Sw” Apple

[2] R4-2509633, “Draft CR on active TCI state switch delay requirements for Low band CA via switching.” LG Electronics