3GPP TSG-RAN WG4 # 116	R4-2509652

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.12.3

Source:	MediaTek Inc.

Title:	Discussion on Spatial Channel Modelling

Document for:	Discussion

# Introduction

The Rel-19 study item [1] investigates practical spatial channel modelling methodologies for both SU-MIMO and MU-MIMO demodulation and CSI reporting requirements. It has been identified that the legacy TDL-based models and spatial correlation matrices poorly match with realistic propagation conditions that have been used as the design assumption for the NR downlink precoder (PMI) codebooks.

For spatially selective channel model development, both CDL (Clustered Delay Line) and TDL models are considered as baseline. It has been demonstrated in this study item that by the multi-cluster TX-RX beam steering approach, the simple TDL models can be extended to have desired spatial properties similar to CDL models.

# Discussion

# 2.1 Comparison of Channel Statistics

#### Channel Properties for Comparison

Agreement:

Channel Properties to be used for Comparison only, not for alignment.

Way Forward:

Utilise channel properties to compare across channel model candidates (Enhanced TDL and CDL) in each comparison case utilising the following properties, qualitative assessment discussions to be brought to RAN4#116 based on the following parameters:

- Power Angular/Phase Distribution in Tx and Rx Directions (including comments on stability and diversity)

- Time Coherence as channel correlation between symbols

- Frequency Coherence measured as channel correlation between subcarriers

Contributions will be assessed either offline between meetings or during RAN4#116 to subjectively decide whether channel models fit expectations of a spatial channel.

In this section, we present a text proposal for TR 38.753 that defines and illustrates channel statistics for comparison and alignment for both CDL and TDL channels.

In our view, channel statistics are powerful for describing, understanding, and comparing the properties of channel models and their realizations. As compared to tput simulations, channel statistics provide a simpler alternative that can be used to cross-check channel model implementations between companies, omitting the uncertainty resulting from different receiver algorithms implementations.

Proposal #1: We propose the following chapter “Channel Statistics Comparison” to be included in TR 38.753.

--------- TP to TR 38.753 starts ------------------

X Channel Statistics Comparison

In this chapter, channel statistics for evaluation, comparison, and alignment of channel models are defined and illustrated. Channel statistics can be gathered without the tput simulation burden and uncertainty resulting from different receiver algorithms implementations.

To specify the channel metrics, the following notations are defined. Arrange MIMO channel as a complex-valued 7-dimensional array:

hn,k,m1,m2,l,f, t , where

  - n=0,1,…, Nrx1-1 is RX-antenna index within one polarization group,

  - k=0,1 is the RX-antenna polarization group index,

  - m1=0,1,…, Ntx1-1 is 1st TX-antenna index within one polarization group,

  - m2=0,1,…, Ntx2-1 is 2nd TX-antenna index within one polarization group,

  - l=0,1 is the TX-antenna polarization group index,

  - f is frequency (subcarrier) index for channel frequency response (CFR), and

  - t is time (symbol) index.

Here it is assumed that Ntx1 is the TX-array size in horizontal direction and Ntx2 in vertical direction. For 4TX and 8TX alignment cases, Ntx2=1. It is also worth noting that channel impulse response (CIR) representation can alternatively be used (except for frequency coherence) by replacing the frequency dimension with delay tap index dimension.

Metric definitions and direct measurement from channel realization

Spatial domain power density (SDPD) is defined at RX side as

SDPDrx1θ=Ek,m1,m2,l, f,t1Nrx1n=0Nrx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙n2

and at TX side as

SDPDtx1θ=En,k,m2,l, f,t1Ntx1m1=0Ntx1-1hn,k,m1,m2,l, f, t∙e-j∙θ∙m12

Here, the range for phase shift θ is (-π,π) or equivalently (0,2π). FFT can be utilized for fast calculation of SDPD. Notation E(index list) denotes statistical mean over the listed indexes/dimensions.

The complex-valued time coherence (TC) is defined as

TC∆t=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f, t+∆t

where ∆t=0, 1, 2,…Nt symbols (or multiples of symbols).

The complex-valued frequency coherence (FC) is defined as

FC∆f=En,k,m1,m2,l, f,th*n,k,m1,m2,l, f, t∙hn,k,m1,m2,l, f+∆f, t

where ∆f=0, 1, 2,…Nf subcarriers (or multiples of subcarriers).

The average channel power is assumed to be normalized to unity so that FC0=TC0=1. As a result, the mean value of SDPD is also one.

Theoretical SDPD curves from CDL model parameters

For CDL models, the theoretical expected SDPD can directly be calculated from model parameters such as AOD, ZOD, AOA ray angles φn,mtx1 , φn,mtx2 , φn,mrx1 and ray powers pn,m, where n is the cluster index and m is the ray index. The ray indexing here is assumed to include the effect of applying the fixed coupling pattern of ray angles given in Table 5.1.4.2-1.

The per-ray channel processes are assumed uncorrelated so that their power responses can be summed. At TX side:

SDPDtx1θ=1Ntx1n,mpn,mk=0Ntx1-1expj∙k∙θn,mtx1-θ2

where θn,mtx1=π∙sinφn,mtx1-αtx and αtx=0 is the horizontal TX antenna array rotation angle. At RX side:

SDPDrx1θ=1Nrx1n,mpn,mk=0Nrx1-1expj∙k∙θn,mrx1-θ2

where θn,mrx1=π∙sinφn,mrx1-αrx and αrx=180 is the horizontal RX antenna array rotation angle. The ray powers include the tabulated cluster powers as well as the attenuation effect of the TX-side antenna radiation pattern Aφn,mtx1-αtx,φn,mtx2-βtx, where βtx=10 is the TX-array downtilt angle and φn,mtx2 is the ZOD angle of the ray. The powers are assumed normalized so that n,mpn,m=1.

Theoretical SDPD curves from multi-cluster TDL model parameters

For multi-cluster TDL models, the expected SDPD can directly be calculated from the cluster-specific TX-RX steering parameters θtx1(n), θrx1(n) and from the spatial correlation matrices of the underlying TDL model. For notational convenience, define column vector d of length N as function of phase shift x: dx,N=1ejx⋯ejx(N-1)T. At TX side:

SDPDtx1θ=1Ntx1npn∙dHθtx1(n)-θ,Ntx1∙Ctx1∙dθtx1(n)-θ,Ntx1

where Ctx1 is the Ntx1×Ntx1 spatial correlation matrix in the first TX-array dimension with correlation parameter α1=0.9 for XP-High. Here, power pn of cluster n is the aggregate power of all the channel taps of the cluster. The powers are assumed normalized so that npn=1.

Similarly, at RX side:
SDPDrx1θ=1Nrx1npn∙dHθrx1(n)-θ,Nrx1∙Crx1∙dθrx1(n)-θ,Nrx1

where Crx1 is the Nrx1×Nrx1 spatial correlation matrix in the first RX-array dimension with correlation parameter β=0.9 for XP-High.

Numerical results

In this section, the metrics for the CDL-C A2/A4 and the multi-cluster TDL models B1, B2, and B3 are plotted.

Figure X-1 depicts the SDPD of CDL-C A2 (non-truncated) and A4 (truncated). As can be seen, the TX side is much more spatially selective than the RX side. The difference between models A2 and A4 is very small. The theoretical and measured curves match very well at TX side, while at RX side some difference remains.

Figure X-2 shows the absolute value of FC for CDL-C A2 and A4. As can be seen, the frequency coherence properties of the 24-cluster A2 and the 12-cluster A4 models are somewhat different.

Figure X-3 shows the TC of CDL-C A2 and A4. As can be seen, TC is complex-valued. This implies that the Doppler spectrum is not symmetric around zero frequency. The shapes of TC for A2 and A4 are similar.

Figure X-1: Measured and theoretical SDPD at TX (left) and at RX (right) for CDL-C.

Figure X-2: Measured frequency coherence (FC) for CDL-C.

Figure X-3: Measured time coherence (TC) for CDL-C with 3 km/h UE speed.

The left side of Figure X-4 depicts the SDPD of the TDL cluster models B1, B2, and B3 as detailed in Tables X-1, X-2, and X-3 respectively. Note that the models are defined to be symmetrical so that the phase shifts and correlation coefficients at the TX and RX side are the same. Therefore, also the SDPD at the TX and the RX sides are identical with 8TX-8RX antenna arrays. In general, multi-cluster TDL models need not be symmetrical. The channels are spatially selective, and the theoretical curves match well with the measured curves. The spatial selectivity pattern can be tailored to suit any test purpose by properly selecting the cluster-specific phase shifts and powers.

For comparison, the right side of Figure X-4 depicts the SDPD of legacy TDL channels with different correlation levels. As can be seen, spatial correlation results in spatial selectivity so that the signal is amplified at phase zero. The theoretical curves match with the measured curves very closely.

Figure X-5 shows the absolute value of FC for the three multi-cluster TDL models. As can be seen, the frequency coherence properties are identical for all the models. This is because the channel tap delays and powers are directly inherited from the underlying TDL-C model.

Figure X-6 shows the TC for the three multi-cluster TDL models. Time coherence is in practice real-valued (imaginary part is very close to zero) and it follows the shape of Bessel function. This is due to the symmetric Jakes Doppler spectrum inherited from the underlying TDL-C channel.

Figure X-4: SDPD of multi-cluster TDL-C channels (left) and legacy TDL-C channels (right).

Figure X-5: Measured FC (absolute value) of multi-cluster TDL-C channels.

Figure X-6: Measured TC (real part) of multi-cluster TDL-C channels with 10Hz Doppler.

--------- TP to TR 38.753 ends ------------------

From the plotted channel statistics, we draw the following observations.

Observation #1: The spatial selectivity properties of CDL-C models A2 and A4 are very similar.

Observation #2: The frequency coherence properties of CDL-C models A2 and A4 are different due to the different channel power delay profiles.

Observation #3: The time coherence properties of CDL-C models A2 and A4 are similar.

Observation #4: The spatial selectivity properties of the multi-cluster TDL models depend on the cluster parameter setting.

Observation #5: The frequency and time coherence properties of the multi-cluster TDL models strictly follow the properties of the underlying legacy TDL model.

As an alternative to spatial SDPDθ, power angular distribution PADφ can be specified as function of AOA/AOD by substituting θ=π∙sinφ in the definition of PSD. Note however that while the physical range of AOD/AOA is (-180,180) degrees, the meaningful range of φ is only (-90,90) degrees. This is because from the channel realization h only, it is not possible to distinguish the whole physical angle range. For example, PAD90-x=PAD(90+x), which means that the power responses of physical angles 90-x and 90+x fall on top of each other. Therefore, we prefer applying SDPD instead of PAD. Plotting PAD would give a false impression of the physical meaning of the metric.

# 2.2 CDL channels

In this section, we discuss simulation results and the details of CDL channels.

# 2.2.1 Simulations/alignment in CDL channels

We have further simulated PMI reporting performance for 32TX-4RX. Results are available in separate simulation result document [3].

# 2.2.2 Angular scaling of CDL channel models

In this study item, new channel models (A2/A4) were derived and agreed, starting from CDL-C defined in Table 7.7.1-3 of TR 38.901 [5]. The derived model has now been extensively analysed and simulated. In the scope of Rel-19, the work is completed.

The angular scaling (AS) process has been in a central role in the CDL model derivation in this SI. We finally mostly ignored the process specified in TR 38.827 [6] and tried to follow [5]. The process defined in TR 38.901 V18.0.0 Section 7.7.5.1 is ambiguous at best since the equations seem to address clusters (cluster index n) while the text talks about rays. The authors of TR 38.827 and RAN4 in this study item interpreted it so that scaling is applied to ray angles, not only to tabulated cluster angles.

In the very latest version of TR 38.901 V19.0.0 (2025-06), Section 7.7.5.1 and the related definitions in Appendix A have significantly changed. As a result, the AS process has changed from the earlier versions of TR 38.901. Compared to the scaling process used/agreed in our SI, at least two major differences can be identified against the TR 38.901 V19.0.0:

- In TR 38.901 V19.0.0, scaling is applied only to cluster angles so that the AS (width) of each cluster cASA, ASD,ZSA,ZSD remains unchanged. In this SI, we have scaled the rays so that also per cluster AS cASA, ASD,ZSA,ZSD is scaled.

- In TR 38.901 V19.0.0, the scaling factor is so defined (Appendix A.5) that the desired AS is exactly achieved. In the earlier versions and in our SI, the scaling factor is defined so that the process only approximately achieves the desired AS.

In the future SCM study/work items, it is a good idea to follow the updates made by RAN1 in TR 38.901 V19.0.0. However, in the scope of this SI and Rel-19, it is too late.

For the AS process of this SI, there is no adequate description for it in any version of TR 38.901. Since the AS process in TR 38.901 V18.0.0 was ambiguously written, it is not even clear whether we have followed it or not. To make things clear, we propose that TR 38.753 should include a good quality detailed description of the AS process used in this SI.

Observation #6: RAN1 has significantly changed the AS scaling process definition in TR 38.901 from version V18.0.0 to version V19.0.0.

Observation #7: In the scope of Rel-19 and TR 38.753, it is not possible for RAN4 to follow TR 38.901 V19.0.0.

Observation #8: Since AS process in TR 38.901 V18.0.0 was ambiguously written, it is not clear whether this SI has followed it or not.

Proposal #2: TR 38.753 should include a detailed description of the AS process adopted in this SI.

# 2.4 Multi-cluster TDL channels

#### Cluster Model Options

Agreement:

A unified TDL cluster model to be proposed into TR.

The unified TDL cluster model is implemented in revised R4-2508747 TP to TR38.753 TDL approaches and related Annex.

A unified system model including the notation and equations is provided in TR 38.753. The parameterization options B1, B2 and B3 are provided in harmonized format in Tables 1, 2, and 3. These tables are also included in our proposed TR updates [4].

Table 1: Cluster model B1: 4 TDL-C instances, 4 clusters

| Delay profile | Cluster index | Power (dB) | θtx1 | θrx1 |
| --- | --- | --- | --- | --- |
| TDLC 300 | 1 | 0 | 0 | 0 |
| TDLC 300 | 2 | 0 | -π/2 | -π/2 |
| TDLC 300 | 3 | -9 | π | π |
| TDLC 300 | 4 | -9 | π/2 | π/2 |

Table 2: Cluster model B2: 1 TDL-C instance, 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |
| --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 |
| 2 | 65 | 0 |  |  |  |
| 3 | 70 | -7.7 | 2 | -π/2 | -π/2 |
| 4 | 190 | -2.5 |  |  |  |
| 5 | 195 | -2.4 |  |  |  |
| 6 | 200 | -9.9 |  |  |  |
| 7 | 240 | -8.0 |  |  |  |
| 8 | 325 | -6.6 |  |  |  |
| 9 | 520 | -7.1 | 3 | π | π |
| 10 | 1045 | -13.0 |  |  |  |
| 11 | 1510 | -14.2 | 4 | π/2 | π/2 |
| 12 | 2595 | -16.0 |  |  |  |

Table 3: Cluster model B3: 2 TDL-C instances, 4 clusters

| TDLC 300 delay profile |  |  | TX-RX beam steering |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tap | Delay (ns) | Tap power (dB) | TDL channel 1 Power: 0 dB |  |  | TDL channel 2 Power: -9 dB |  |  |
|  |  |  | Cluster index | θtx1 | θrx1 | Cluster index | θtx1 | θrx1 |
| 1 | 0 | -6.9 | 1 | 0 | 0 | 3 | π | π |
| 2 | 65 | 0 |  |  |  |  |  |  |
| 3 | 70 | -7.7 |  |  |  |  |  |  |
| 4 | 190 | -2.5 |  |  |  |  |  |  |
| 5 | 195 | -2.4 | 2 | -π/2 | -π/2 | 4 | π/2 | π/2 |
| 6 | 200 | -9.9 |  |  |  |  |  |  |
| 7 | 240 | -8.0 |  |  |  |  |  |  |
| 8 | 325 | -6.6 |  |  |  |  |  |  |
| 9 | 520 | -7.1 |  |  |  |  |  |  |
| 10 | 1045 | -13.0 |  |  |  |  |  |  |
| 11 | 1510 | -14.2 |  |  |  |  |  |  |
| 12 | 2595 | -16.0 |  |  |  |  |  |  |

# Conclusion

In this paper we provided our views on the spatial channel modelling. The following observations and proposals are made:

Observation #1: The spatial selectivity properties of CDL-C models A2 and A4 are very similar.

Observation #2: The frequency coherence properties of CDL-C models A2 and A4 are different due to the different channel power delay profiles.

Observation #3: The time coherence properties of CDL-C models A2 and A4 are similar.

Observation #4: The spatial selectivity properties of the multi-cluster TDL models depend on the cluster parameter setting.

Observation #5: The frequency and time coherence properties of the multi-cluster TDL models strictly follow the properties of the underlying legacy TDL model.

Observation #6: RAN1 has significantly changed the AS scaling process definition in TR 38.901 from version V18.0.0 to version V19.0.0.

Observation #7: In the scope of Rel-19 and TR 38.753, it is not possible for RAN4 to follow TR 38.901 V19.0.0.

Observation #8: Since AS process in TR 38.901 V18.0.0 was ambiguously written, it is not clear whether this SI has followed it or not.

Proposal #2: TR 38.753 should include a detailed description of the AS process adopted in this SI.

# Reference

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2508776, “Way Forward for [115][321] NR_SCM”, Nokia

- R4-2509653, “Simulation results of spatial channel modelling”, MediaTek Inc.

- R4-2509654, “TP to TR38.753: TDL approaches and related Annex”, MediaTek Inc.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

- 3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)