3GPP TSG-RAN WG4 Meeting # 116	R4- 2509735

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM impacts of SBFD operation

Agenda item:	7.21.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remain open issues as below on RRM impacts of SBFD operation described in WF [1] and more detail in last meeting’s moderator summary [2].

- Issue 2-1-4: Requirements for RACH requirements

Discussion

Issue 2-1-4: Requirements for RACH requirements - Proposals - Proposal 1 (Nokia, LGE): - The requirements for 4-step RA type in clause 6.2.2.2 are updated to take into account the RO-type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols, and the RO type selection from additional RO to legacy RO, as agreed in RAN2. - Recommended WF - Discuss whether the proposed clarification in P1 is needed in 38133.

In clause 6.2.2.2 need to be considered whether to be updated or not according to the RAN2 agreements. Especially, SBFD RO type switching related agreements are need to adapted to the TS38.133. The SBFD RO type switching related RAN2 agreements are captured as below:

RAN2#127bis RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. RAN2#129bis Random access in SBFD - RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported. RAN2#120 Random access in SBFD - Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. - For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. - The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

According to the agreement, the UE is allowed to switch from SBFD RACH occasions to legacy RACH occasions after certain (configured) number of times of RACH attempt and vice versa. RAN4 need to check whether the description of random access should be updated for RO switch or not as below.

6.2.2.2 Requirements for 4-step RA type The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [7]. The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [3] and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [18] for FR1 and in Table 6.3.4.2-1 of TS 38.101-2 [19] for FR2. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [18] for FR1 and clause 6.3.4.3 of TS38.101-2 [19] for FR2. The UE shall indicate a random access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in clause 5.1.4 in TS 38.321 [7]. The SBFD-aware UE is allowed to switch from additional RACH occasions to legacy RACH occasions or vice versa, after certain (configured) number of times of RACH attempt in RACH occasions. The requirements in this clause apply for UE in SA operation mode or any MR-DC operation mode.

Proposal 1: RAN4 to consider adding following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

- The SBFD-aware UE is allowed to switch from additional RACH occasions to legacy RACH occasions or vice versa, after certain (configured) number of times of RACH attempt in RACH occasions.

Summary

In this contribution, we provide our views on the remain open issues on RRM impacts of SBFD operation described in WF [1] and more detail in last meeting’s moderator summary [2]. Our proposals are as below.

Proposal 1: RAN4 to consider adding following sentence in 6.2.2.2 for clear SBFD-aware UE’s RACH procedure.

- The SBFD-aware UE is allowed to switch from additional RACH occasions to legacy RACH occasions or vice versa, after certain (configured) number of times of RACH attempt in RACH occasions.

Reference

[1] R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon

[2] R4-2505568, “Topic summary for [115][219] NR_duplex_evo”, Moderator (Huawei, HiSilicon)