3GPP TSG-RAN WG4 Meeting # 116	R4-2509765

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on event triggered L1 measurement reporting for mobility

Agenda Item:	7.25.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Event-triggered L1 reporting

- Discussion

Reporting capability

- Issue 2-4: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Yes (Xiaomi, CMCC, CATT, MTK, ZTE, OPPO, CTC, E///) - Option 1a: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO) - Option 1b: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS. (Nokia) - Option 2: Open to let RAN1 discuss the number of events supported by UE at first. (MTK) - Option 2a: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo)

There are some discussions about the UE capability of supported number of events. Generally, we are fine to define capability for supported number of events for L1 based reporting for LTM. Since CSI-RS based LTM is also considered in L1 event triggered reporting, the relation between L3 reporting and L1 reporting, and relation between CSI-RS based L1 measurement/reporting and SSB based L1 measurement/reporting will have impact on the capability definition. it’s also FFS how to count frequency layer. Therefore, we prefer to discuss the detail of UE capability for in later stage when design is clearer.

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Issue 2-4: Impact on other LTM requirements Agreement in RAN4#114bis: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS Candidate options: - Proposal 1: considering it is invalid that only event entering condition is configured and leaving condition is not configured, update previous agreement to: (Huawei) - For FR1: - UE has sent an event-triggered L1 report due to the entering condition is met and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, and leaving condition of the event is not met - The related cell switch delay and TCI state activation requirements are applicable - The above optimization is not applicable to FR2 - Proposal 2: Apply to both FR1 and FR2: When UE has sent event-triggered L1 report and leave condition of the event is configured and UE has not sent report on leave before receiving the cell switch command, delay requirements are applicable. (Nokia) - Proposal 3: Either a) introduce no further conditions than UE having sent L1 event-triggered report and condition continuing to be met for the case without leave condition configured, or b) extend applicability from the existing e.g. to 5 seconds based on event-triggered L1 report. - Proposal 4: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) - Proposal 5: add the following condition on agreements under issue 2-7 in RAN4#114bis: (Apple) - Number of active TCI does not exceed that UE can support. - Proposal 6: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases. (MTK) - Proposal 7: For both FR1 and FR2, the TCI state is considered as known during 1280ms after event-triggered L1 report. And the related cell switch delay and TCI state activation requirements are applicable. (ZTE) - Proposal 8: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///) - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured

The rationale is that, in the absence of a configured leaving condition, the validity of an event becomes indeterminable. Without a clearly defined exit threshold, it is not possible to assess whether the event remains applicable at a given time, thereby introducing ambiguity into measurement interpretation and subsequent procedure execution.

Proposal 2: For FR1, it is proposed that the cell switch delay and TCI state activation delay requirements apply only when the UE has transmitted an event-triggered L1 report, the reporting condition remains fulfilled upon reception of the corresponding command, and the configured leaving condition remains unsatisfied.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: RAN4 to define UE capability for Event Triggering and Reporting Criteria for LTM.

Proposal 2: For FR1, it is proposed that the cell switch delay and TCI state activation delay requirements apply only when the UE has transmitted an event-triggered L1 report, the reporting condition remains fulfilled upon reception of the corresponding command, and the configured leaving condition remains unsatisfied.

- Reference