3GPP TSG-RAN WG4 Meeting #116									R4-2509782

Bengaluru, India, 25th – 29th August, 2025

Agenda item:	7.6.4.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 discussed whether to introduce HPUE for NTN less than 5MHz, and the related WF was approved in [1]. This contribution discusses HPUE for NTN less than 5MHz and provides our proposals.

Discussion

Issue 2-2: HPUE for 3MHz CBW - For information: - RAN4 discussed the proposal to further specify 3MHz requirements for other power classes other than PC3, but cannot reach agreement. - RAN4 recognizes the following: - Current WID explicitly mention less than 5MHz objectives should focus on PC3 - Some companies believe 3MHz channel bandwidth is demanded by the industry - Introducing 3MHz for other power classes will take significant time and work - UE cannot indicate that it supports a higher power class in a band for only some channel bandwidths

According to current WID in [2], it is explicitly captured that the objective of NTN less than 5MHz focus on PC3 for Tx RF requirements.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In addition, RAN4 is expected to take significant time and work load to introduce the UE RF requirements for other power classes, e.g. A-MPR requirements, the simulation evaluation is expected for companies to derive the A-MPR values. Considering this is the last meeting for this WI. It is proposed that RAN4 focus on PC3 for UE RF requirement for objective of NTN less than 5MHz in Rel-19.

Proposal 1: RAN4 focus on PC3 for UE RF requirements for NTN less than 5MHz in Rel-19.

Conclusion

In this contribution, we discussed the HPUE for NTN less than 5MHz and provides our proposals:

Proposal 1: RAN4 focus on PC3 for UE RF requirements for NTN less than 5MHz in Rel-19.

- Reference

- R4-2508765	WF for [115][310] NR_IoT_NTN_less_than_5MHz_UERF		Xiaomi

- RP-240857	New WID: Enhanced requirements and test methodology for NR and IoT NTN