3GPP TSG-RAN4 Meeting #116 	R4-2509861

# Bengaluru, India, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2914 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_unlic-Core) CR to correct the frequency offset of the interferer used in clause 7.5F.1 - TS38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A change is needed: - The frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured) is currently 20/-20 MHz for all channel bandwidths specified (20, 40, 60, 80, 100MHz) which causes frequency overlapping between the wanted signal and interferer signal. - The frequency offset in MHz should be different for each channel bandwidth. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.5F.1-2: - Change the frequency offset of the interferer from “20 / -20” MHz to “(BWChannel + BWinterferer)/2 / -(BWChannel + BWinterferer)/2”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is not applicable and testing of the requirement is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5F.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Resubmission of R4-2505648 (RAN4#115) as it did not respect the 3GPP guidelines for it (and associated CRs) to be agreed. This CR is a mirror to the Cat-F Rel-17 CR (R4-2509859). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

## 7.5F	Adjacent channel selectivity for shared spectrum channel access

### 7.5F.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Instead of the general ACS requirements specified in clause 7.5, the UE shall fulfil the minimum requirements specified in Table 7.5F.1-1. These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.5F.1-2, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5F.1-1: ACS for shared spectrum channel access bands

| RX parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 20, 40, 60, 80, 100 MHz |
| ACS | dB | 24 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |

Table 7.5F.1-2: Test parameters for shared spectrum channel access bands

| RX parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 20, 40, 60, 80, 100 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 36.5 dB – 10log10(BWChannel /20) dB |
| BWinterferer | MHz | 20 |
| Finterferer (offset) | MHz | (BWChannel + BWinterferer)/2 / -(BWChannel + BWinterferer)/2 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

### 7.5F.1A	Adjacent channel selectivity for shared spectrum channel access CA

#### 7.5F.1A.1	Intra-band contiguous shared spectrum channel access CA

ACS for intra-band contiguous shared access CA requirements are specified in Table 7.5F.1A.1-1.  These requirements apply for any SCS specified for the channel bandwidth of the wanted signal.  For the test parameters specified in Table 7.5F.1A.1-2, the throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5F.1A.1-1: ACS for intra-band contiguous shared access CA

|  |  | NR-U CA bandwidth class |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | I | M | N | O |
| ACS | dB | 24 – 10log10(BWChannel_CA/20) |  |  |  |  |  |  |  |
| NOTE 1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |  |  |  |  |  |

Table 7.5F.1A.1-2: Test parameters for intra-band contiguous NR-U CA

| Rx Parameter | Units | NR-U CA bandwidth class |
| --- | --- | --- |
|  |  | B, C, D, E, M, N, O |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 22.5 – 10log10(BWChannel_CA/20) dB |
| BWInterferer | MHz | 20 |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3F.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

### 7.5F.2	Void

<<Unchanged parts of the section skipped>>

<<End of change>>