3GPP TSG-RAN WG4 Meeting #116	R4-2509921

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	7.17.4

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#115 in the scope of Rel-19 WI NR_AIML_air ‎[1]. Agreements achieved during RAN4#115 meeting are summarized in the WF ‎[2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion. In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- AI/ML BM Failure Recovery

- Conditions for known TCI state

Discussion

Measurement delay requirements

The following agreement was reached in RAN4#115 meeting:

Agreement: Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) Case 2: postpone discussion until RAN1 concludes

The below table shows the measurement period for L1-RSRP reporting in legacy.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max( TReport, ceilM*P*N*TCSI-RS ) |
| DRX cycle ≤ 320ms | max( TReport, ceil 1.5*M*P*N)*max⁡( TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceilM*P*N* TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. Treport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

For non-DRX configuration, the measurement period (TL1-RSRP_Measurement_Period_CSI-RS) in legacy is defined in the above table, where the parameters M, N and P are defined as follows:

Parameter M:

- For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise.

- For aperiodic CSI-RS resources M=1

Parameter N:

- For periodic CSI-RS and semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1.

- For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam/ N_res_per_set), where N_res_per_set is number of resources in the resource set.

- For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1.

- For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1

Parameter P:

- P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

- P= 11 - TCSI-RSTSMTC period , when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion ( TCSI-RS < TSMTC period ).

In our view, the measurement period for L1-RSRP measurement in legacy could be applied for BM-Case1. A single time instance of measurement reporting needs to be reported for BM-Case1, therefore the parameter M=1 could be the same as in legacy. Similarly, the parameters N and P will be the same as considered in legacy. Following RAN1 discussion, the N, M, and P parameters would also follow legacy for BM-Case1.

For BM-Case2, multiple time instances of measurement may need to be measured during observation window. Therefore, the current measurement period for L1-RSRP measurement in legacy could not be applied for AI/ML beam prediction for BM-Case2. Since the current measurement period in Clause 9.5.4.2 does not include when there are multiple time instances measurements corresponding to observation window. However, it might depend on UE to buffer any past measurements. RAN4 should still further discuss on the delay related to measurements especially with respect to measurement instances in observation window.

The measurement period for L1-RSRP measurement (TL1-RSRP_Measurement_Period_CSI-RS) will need to be enhanced, where the parameter to define the latest measurements (parameter M) in  Clause 9.5.4.2 will need to be redefined. However, parameters N and P could remain the same as in legacy.

- RAN4 should consider the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1.

- RAN4 should consider the enhancement of measurement period for L1-RSRP measurement for BM-Case2, where the values of M higher than 3 should be considered depending upon the observation window size.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

- RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

LCM and performance monitoring related requirements

The following agreement was reached in RAN4#115 meeting in general aspects agenda item:

Agreement from RAN4#115 in general aspects: Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

The LCM operations for AI/ML BM, e.g., activation, deactivation could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. By reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

- RAN4 should consider Table 1 as a baseline for further discussion on LCM operations related RRM core requirements.

Table 1: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Deactivation |
| --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay from re-configuration (RRC Reconfiguration) Delay from UE processing related to capability report (Option 1) Delay due to CSI-RS measurement period (Option 2) Delay due to UE’s acknowledgement upon reconfiguration (RRCReconfigComplete) |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay from re-configuration (RRC Reconfiguration) |
| Aperiodic CSI reporting | Delay related activating aperiodic CSI resource set, Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay related activating aperiodic CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

In RAN4#115 meeting, RAN4 sent an LS to RAN2 to clarify if the applicable functionality is already activated before reporting applicable functionalities via RRCReconfigurationComplete in step 4 or UE starts to activate the applicable functionality upon reporting applicable functionalities via RRCReconfigurationComplete in step 4?

As per our understanding, some processing delay may be needed to activate the applicable functionality upon reporting applicable functionalities via RRCReconfigurationComplete in step 4.

RAN4 to consider RRCReconfigurationComplete message containing applicable functionality report as the starting point to define activation delay requirement for periodic CSI reporting for AI/ML BM use case.

RAN4 to consider first inference report as the end point to define activation delay requirements for periodic/aperiodic/semi-persistent CSI reporting for AI/ML BM use cases.

    - Performance monitoring related requirements

The following agreement was reached in RAN4#115 meeting:

Agreement from RAN4#115: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has agreed on the definition of report quantity of beam prediction accuracy, RS-PAI, which can be defined as the total count of accurate reference signal prediction instance(s), Np, that meets the conditions and Np is calculated over N valid performance monitoring instances. The need to determine the N latest transmission occasion(s) of monitoring resources is considered.

- Accordingly, RAN4 will need to define new delay requirements on the reporting of beam prediction accuracy, RS-PAI, corresponding to number of N latest transmission occasion(s) of monitoring resources. When the UE calculates and reports RS-PAI, NW would interpret those reports and, if necessary, makes a decision and indicates to UE about functionality (CSI-ReportConfig) selection/activation/ deactivation/switching/ fallback operation.

From the RAN1 definition of the report quantity of the beam prediction accuracy report for monitoring, we can see that this performance monitoring metric is different from the beam prediction accuracy metrics being discussed in RAN4 as part of the performance requirements. Hence, RAN4 can define accuracy requirements for this metric, which can be tested at least in static conditions.

RAN4 to define accuracy requirements for UE-assisted monitoring where the UE reports the monitoring metric (i.e., monitoring Type 1 Option 2) in BM-Case1 and BM-Case2. The accuracy of the performance monitoring metric should be tested at least in static radio conditions.

Similar to accuracy requirements, RAN4 can also define delay requirements for RS-PAI reporting in BM-Case1 and BM-Case2, where the starting point of RS-PAI reporting delay is when UE receives the monitoring RS resources, while the ending point of this delay can be when UE reports the performance monitoring metric, RS-PAI.

RAN4 to define delay requirements for UE-assisted monitoring where the UE reports the performance monitoring metric, RS-PAI, in BM-Case1 and BM-Case2. The starting point of this delay can be when UE sends RRCReconfigurationComplete message in response to the configuration of monitoring RS resources via RRCReconfiguration, and the ending point can be when UE reports the first performance monitoring metric, RS-PAI.

    - AI/ML BM Pre-Activation Monitoring

According to RAN2 discussion, it is not agreed yet the exact procedure for the activation of the AI/ML BM functionality, while applicability of inference related parameters are checked by the UE and/or by the NW before the AI/ML BM functionality is enabled/activated. The applicability of the parameters still may not guarantee that accurate prediction results can be provided by the UE. For example, it may happen that AI/ML functionality at the UE is compatible with BS parameters, e.g., beam codebook, antenna configurations, but the prediction quality may be poor due to other conditions that are not part of the applicability parameters. However, when performance evaluation is a part of the AI/ML functionality activation, the performance metric could be calculated over multiple instances and compared to the performance evaluation threshold before it can be concluded that the performance is sufficiently high, and the functionality can be fully activated. The time that UE needs for such performance evaluation to verify the performance before AI/ML functionality is activated would be required. In such scenario, activation and evaluation periods can be defined either by configured pre-defined numbers or as a scaling factor of periodicity or interval in between the RS used for measurements, in the case of semi-persistent or periodic configuration.

- RAN4 to further discuss the delay requirement for AI/ML BM functionality pre-activation when performance monitoring is considered when verifying AI/ML BM functionality to be activated

AI/ML BM Failure Recovery

The applicability of the parameters still may not guarantee that accurate inference results, i.e., which may lead to performance degradation and may consequently lead to AI/ML functionality (CSI-ReportConfig) failure. However, the performance degradation and AI/ML functionality failure may be due to temporary circumstances, such as a temporary blockage obstructing signals to the UE, therefore, in this case the AI/ML functionality should not be definitely disabled/de-activated, since the blockage can move away and the signals to the UE may be resumed. In this case, there must be a recovery procedure for AI/ML functionality to recover once the conditions are changed again. On the other hand, the circumstances/conditions may have changed for indefinite period in other cases. The AI/ML functionality should be rather definitely disabled/de-activated until some other event is triggered such as, UE switch off or site change etc. In such case, the AI/ML functionality (CSI-ReportConfig) is not active but still can be supported by the UE, where the recovery mode can be triggered, e.g., after the functionality has failed. When the UE is enabled in the recovery mode, the time for recovery starts and the UE will compare the predicted beams with the measurements from the monitoring RS resources. If the prediction accuracy is higher than the predefined threshold for the recovery evaluation period, the AI/ML BM functionality (CSI-ReportConfig) will be considered recovered and hence become applicable again. Therefore, the delay requirement regarding AI/ML functionality (CSI-ReportConfig) recovery success should be further discussed in RAN4.

- RAN4 to further discuss the delay requirement for AI/ML BM functionality failure recovery success when AI/ML BM functionality failure happens and UE is enabled for recovery mode.

Conditions for known TCI state

The following agreement was reached in RAN4#115 meeting:

Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

Following RAN4#115 agreement, if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. This condition also aligns with RAN1 view, including when Set B is a set of wide beams and Set A is a set of narrow beams, where the UE could determine the QCL reference (source RS included in the TCI state) associated with CSI-RS resources. In such case, the UE knows Rx beam when receiving predicted Tx beam.

Given that Rx beam is known at the UE for receiving the predicted Tx beam, if the predicted beam(s) is not measured by the UE for long enough period in both BM-Case1 and BM-Case2, the predicted beam(s) will still be corresponding to unknown TCI state. The delay for switching the beam to an unknown TCI state is higher than switching to a known TCI state. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the known conditions for TCI state, specified in Clause 8.10.2 of TS 38.133:

In legacy, Clause 8.10.2: The TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

The condition of known TCI state could be relaxed such that the predicted beam could be measured, e.g., option 1: with a delay longer than 1280 ms or option 2: the beam prediction should be added in TCI state conditions. For option 1,it needs to be further studied how much longer than 1280ms will be needed. However, for option 2, the known TCI state conditions in spec would need to be updated as follows “TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement or prediction”.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for both BM-Case1 and BM-Case2. The TCI state switch command is received within a delay X*1280 ms or the prediction should be added in the TCI state condition.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- AI/ML BM Failure Recovery

- Conditions for known TCI state

In the paper, the following Proposals were made:

Proposal 1: RAN4 should consider the legacy L1-RSRP measurements with the same values of M, N, and P for measurement delay requirements for BM-Case1.

Proposal 2: RAN4 should consider the enhancement of measurement period for L1-RSRP measurement for BM-Case2, where the values of M higher than 3 should be considered depending upon the observation window size.

Proposal 3: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Proposal 4: RAN4 should consider the Table below as a baseline for further discussion on LCM operations related RRM core requirements.

Table : Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Deactivation |
| --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay from re-configuration (RRC Reconfiguration) Delay from UE processing related to capability report (Option 1) Delay due to CSI-RS measurement period (Option 2) Delay due to UE’s acknowledgement upon reconfiguration (RRCReconfigComplete) |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay from re-configuration (RRC Reconfiguration) |
| Aperiodic CSI reporting | Delay related activating aperiodic CSI resource set, Delay from UE processing related to capability report Delay due to CSI-RS measurement period | Delay related activating aperiodic CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

Proposal 5: RAN4 to consider RRCReconfigurationComplete message containing applicable functionality report as the starting point to define activation delay requirement for periodic CSI reporting for AI/ML BM use case.

Proposal 6: RAN4 to consider first inference report as the end point to define activation delay requirements for periodic/aperiodic/semi-persistent CSI reporting for AI/ML BM use cases.

Proposal 7: RAN4 to define accuracy requirements for UE-assisted monitoring where the UE reports the monitoring metric (i.e., monitoring Type 1 Option 2) in BM-Case1 and BM-Case2. The accuracy of the performance monitoring metric should be tested at least in static radio conditions.

Proposal 8: RAN4 to define delay requirements for UE-assisted monitoring where the UE reports the performance monitoring metric, RS-PAI, in BM-Case1 and BM-Case2. The starting point of this delay can be when UE sends RRCReconfigurationComplete message in response to the configuration of monitoring RS resources via RRCReconfiguration, and the ending point can be when UE reports the first performance monitoring metric, RS-PAI.

Proposal 9: RAN4 to further discuss the delay requirement for AI/ML BM functionality pre-activation when performance monitoring is considered when verifying AI/ML BM functionality to be activated

Proposal 10: RAN4 to further discuss the delay requirement for AI/ML BM functionality failure recovery success when AI/ML BM functionality failure happens and UE is enabled for recovery mode.

Proposal 11: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for both BM-Case1 and BM-Case2. The TCI state switch command is received within a delay X*1280 ms or the prediction should be added in the TCI state condition.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2508080, “WF on NR_AIML_air_part1”, Qualcomm Incorporated, RAN4#115 meeting, May 19th – 23rd 2025, Malta, Malta.