3GPP TSG-RAN WG4 Meeting #116	R4-2509956

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on RRM performance requirements for NR_NTN_Ph3

Agenda item:	7.27.6

Document for:	Discussion

Introduction

RAN plenary has revised the WI for NR NTN enhancements in [1], which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_Ph3).

In this document we provide our views on the 1st objective, namely the support of downlink coverage enhancement, and on the 5th objective, namely the support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems, related to RRM performance requirements considering the open issues identified in the latest WFs from RAN4 #115 [2] [3] and further detailed in the Topic summaries [4] [5].

Discussion

  - Support of (e)RedCap UEs

In the present document we discuss the open issues presented in the WF [2] and the Topic summary [4], as listed below:

- Issue 2-1-1: Measurement accuracy requirement

- Issue 2-1-2: Test configurations and parameters for (e)RedCap UE with FR1-NTN

- Issue 2-1-3: How and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN

- Issue 2-1-4-1: Separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN

- Issue 2-1-4-2: General test case scope

RAN4 has discussed general aspects for RRM performance requirements for (e)RedCap UE with FR1-NTN.

Measurement accuracy

At RAN4#115, the following way forward was provided by Moderator [2].

Issue 2-1-1: Measurement accuracy requirement <Way Forward>: - Proposal 1 (CMCC): - For 2Rx RedCap UE, reuse the measurement accuracy requirement of NTN. - For 1Rx RedCap UE, define new measurement accuracy requirement on NTN operating bands with reusing the RSRP accuracy and side conditions from legacy 1Rx RedCap UE. - Proposal 2 (vivo): For the performance requirement in NTN for NR Phase 3, measurement accuracy requirements need to be defined for (e)Redcap UEs with FR1-NTN. - Proposal 3 (HW): RAN4 to define accuracy requirements for following measurements for RedCap in NTN. - Intra- and inter-frequency RSRP - Intra- and inter-frequency RSRQ - Intra- and inter-frequency SINR - L1-RSRP - Rx-Tx for NW verified location

In our view, the following accuracy requirements for RedCap UE apply, as mentioned in Proposal 3 above.

RAN4 to define accuracy requirements for following measurements for RedCap UE with FR1-NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- SSB based / CSI-RS based L1-RSRP

- UE Rx-Tx time difference (if multi-RTT is supported)

Accuracy requirements for 2Rx RedCap with FR1-NTN and 1Rx RedCap with FR1-NTN can fully reuse accuracy requirements for RedCap UE in TN, as specified in clause 10.1A for RSRP, RSRQ, SINR and L1-RSRP and in clause 10.1.25C for UE Rx-Tx time difference based on reduced number of samples introduced for TN UE in Rel-17 and NTN UE in Rel-19.

RAN4 to define accuracy requirements for 1Rx and 2Rx RedCap UE with FR1-NTN reusing accuracy requirements in clause 10.1A for RSRP, RSRQ, SINR and L1-RSRP and in clause 10.1.25C for UE Rx-Tx time difference.

Test configurations and parameters

At RAN4#115, the following way forward was provided by Moderator [2].

#### Issue 2-1-2: Test configuration and parameters for (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1 (CATT): - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured in test cases. - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN. - Proposal 1a (ZTE): RAN4 shall consider the following general test configurations as follows: Configuration; Description. 1; GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 2; NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 3; GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. 4; NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz. NOTE: The UE is only required to be tested in one of the supported test configurations. - Proposal 2 (HW): Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 3 (CMCC): Take the NTN test setups as the reference, and use the ‘refer to’ method when drafting the spec to avoid duplicated contents. - Proposal 4 (ZTE): RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

In [4], the Moderator gave following recommendation.

- Recommended WF - Recommend to agree on: - Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN. - 15 kHz SCS and 10MHz bandwidth can be configured in test cases. - Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN. - When drafting the spec, RAN4 to use the ‘refer to’ method for procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

We agree with the above recommended WF. In our view, NTN configurations specified in TS 38.133 used in RRC_CONNECTED should serve as baseline here. For instance, for the SAN handover from FR1 to FR1 test case in clause A.14.2.1.1, the supported test configurations are as listed below.

Table A.14.2.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Accounting for different RedCap device types in FR1, FDD and HD-FDD configurations need to be introduced. Moreover, no different test configurations are needed for 1Rx and 2Rx UE’s, they can be accommodated in the same. This yields to following proposal for supported test configurations for (e)RedCap with FR1-NTN.

RAN4 to use following supported test configurations for (e)RedCap with FR1-NTN.

Table A.14.5.x.y.z: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

How and where to specify test cases for (e)RedCap with FR1-NTN

At RAN4#115, the following way forward was provided by Moderator [2].

#### Issue 2-1-3: How and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN, <Way Forward>: - Proposal 1(ZTE): The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario. - Proposal 1a (HW): Re-use existing test cases for NTN in A.14 for RedCap in NTN - Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values - Proposal 2 (ZTE): For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

We agree with the Proposals 1 and 2. The test cases for 1Rx and 2Rx (e)RedCap UE with FR1-NTN should be included in Annex A.14. New test cases are needed for HD-FDD for 1Rx and 2 Rx (e)RedCap UE and for FDD for 1Rx (e)RedCap UE, whilst 2Rx FDD (e)RedCap UE is expected to reuse the legacy NTN RRM test cases in FR1.

RAN4 to include test cases for (e)RedCap UE with FR1-NTN in Annex A.14, referring to legacy NTN test cases where appropriate.

Separate test cases for 1Rx and 2Rx (e)RedCap UE with FR1-NTN

At RAN4#115, the following way forward was provided by Moderator [2].

##### Issue 2-1-4-1: Whether to define separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN, <Way Forward>: - Proposal 1 (CATT, CMCC): RAN4 to define the separate test cases for 1Rx and 2Rx (e)RedCap UE with NTN. - Proposal 2 (HW): Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs - Proposal 3 (ZTE): Discuss case by case.

In our view, Proposal 1 can serve as baseline. For RedCap UE test cases, specified in Annex A.16, test cases are distinguished between 1Rx UE and 2 Rx UE: for cell reselection, for CG-SDT, for handover from FR1 to FR1, for handover to E-UTRAN, for RRC Re-establishment, for Tx timing, TA adjustment, RLM, BFD, active BWP switch, UE specific CBW change and for RRM measurement types SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP. Thus, similar structuring of the test cases should be done for (e)RedCap UE with FR1-NTN.

RAN4 to adopt separate test cases for 1Rx and 2Rx (e)RedCap with FR1-NTN as baseline and evaluate thereafter, if test cases can be merged to reduce the extent of changes to spec.

General test scope

At RAN4#115, the following way forward was provided by Moderator [2].

##### Issue 2-1-4-2: General test scope <Way Forward>: The following Table is based on the input from companies, and it can be a baseline. Test category; Company comments. RRC_IDLE state mobility; Cell reselection; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. RRC_IDLE state mobility; Configured Grant based Small Data Transmissions (CG-SDT); Support: CATT, Ericsson, vivo. RRC_CONNECTED state mobility; Handover - RACH-based - RACH-less; Support: CATT, vivo, ZTE, CMCC, HW. CHO - time-based - location-based; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. satellite switching with re-synchronization; Support: CATT, vivo, ZTE, CMCC, HW. RRC Connection Mobility Control; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Timing; UE transmit timing; Support: CATT, Ericsson, vivo, CMCC, HW. UE timer accuracy; Support: vivo. Timing Advance; Support: CATT, Ericsson, vivo, CMCC, HW. Signalling characteristics; Radio Link Monitoring; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Beam Failure Detection and Link recovery; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. Active BWP switch delay; Support: CATT, Ericsson, vivo, CMCC, HW. Active TCI state switching delay; Support: vivo. UE-specific CBW change; Support: CATT, Ericsson, vivo, CMCC, HW. Pathloss reference signal switching delay; Support: CATT, Ericsson, vivo, HW. Measurement procedure; NR intra-frequency measurements; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. NR inter-frequency measurements; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. L1-RSRP measurements for Reporting; Support: CATT, Ericsson, vivo, ZTE, CMCC, HW. NR measurements for positioning; Support: CATT, Ericsson. Measurement Performance; SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance; Support: Ericsson, CMCC, HW. L1-RSRP measurement Performance for beam reporting; Support: Ericsson, CMCC, HW. - RAN4 to further discuss the specific test case list with configurations.

First, we agree with the Moderator, that the above list can serve as a baseline for the test case specification work for (e)RedCap with FR1-NTN and we support all of the test cases. Second, with regard to NR measurements for positioning, further discussion is needed. For normal UE in FR1-NTN, the support of UE Rx-Tx time difference has been introduced into Rel-19 for support in RRC_CONNECTED used for Multi-RTT to support NW verified location. This is based on Rel-17 reduced samples performance without FH. In our view, the test case NR measurements for positioning should refer to UE Rx-Tx time difference to align with normal UE in NTN.

RAN4 to adopt the test case scope as listed in the Way Forward under issue 2-1-4-2.

RAN4 to discuss the scope of the test case on supported NR measurements for positioning for (e)RedCap UE and preferably align it to normal UE in FR1-NTN.

Support of DL coverage enhancement

As the core part for DL coverage enhancement is not yet complete [3], in our view it is too early to discuss the performance part.

Wait for the completion of the core part for DL coverage enhancement before investigating respective performance requirements and test cases.

Conclusion

This paper has analysed the open issues related to performance requirements for (e)RedCap UE with FR1-NTN and for DL coverage enhancement, provided in the latest WF for this WI [2], and the following set of proposals are made.

- RAN4 to define accuracy requirements for following measurements for RedCap UE with FR1-NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- SSB based / CSI-RS based L1-RSRP

- UE Rx-Tx time difference (if multi-RTT is supported)

RAN4 to define accuracy requirements for 1Rx and 2Rx RedCap UE with FR1-NTN reusing accuracy requirements in clause 10.1A for RSRP, RSRQ, SINR and L1-RSRP and in clause 10.1.25C for UE Rx-Tx time difference.

RAN4 to use following supported test configurations for (e)RedCap with FR1-NTN.

Table A.14.5.x.y.z: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15 kHz SSB SCS, 10 MHz BW |
| 3 | GSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| 4 | NGSO, NR HD-FDD, 15 kHz SSB SCS, 10 MHz BW |
| NOTE1: If (e)RedCap UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. NOTE2: If (e)RedCap UE supports both FDD and HD-FDD operation, the UE is only required to be tested in one of both. |  |

RAN4 to include test cases for (e)RedCap UE with FR1-NTN in Annex A.14, referring to legacy NTN test cases where appropriate.

RAN4 to adopt separate test cases for 1Rx and 2Rx (e)RedCap with FR1-NTN as baseline and evaluate thereafter, if test cases can be merged to reduce the extent of changes to spec.

RAN4 to adopt the test case scope as listed in the Way Forward under issue 2-1-4-2.

RAN4 to discuss the scope of the test case on supported NR measurements for positioning and preferably align it to normal UE in FR1-NTN.

Wait for the completion of the core part for DL coverage enhancement before investigating respective performance requirements and test cases.

- References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #115

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated), RAN4 #115

- R4-2505576, Topic summary for [115][227] NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #115

- R4-2505577, Topic summary for [115][228] NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated), RAN4 #115