3GPP TSG-RAN WG4 Meeting #116	R4-2509961

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM performance requirements

Agenda item:	7.26.4

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on open issues after RAN4 #114bis on RRM requirements for XR enhancements [1] and provide respective observations and proposals. Note, agreements reached in RAN4#114bis are contained in [2].

Discussion

Discussion Highlights

In particular, the following aspects are treated in this contribution and respective observations and proposals are made.

- Extended list of potential test cases

- Prioritization among test cases

- Proposals regarding measurement gap cancelation model

List of test cases

Below we present an extended list of potential test cases for XR measurement gap cancelation. This list is presented here for discussion. Since the XR enhancements WID includes limited number of TUs, it is proposed that RAN4 focuses on the most important test cases. Therefore, we include one column that states Nokia’s opinion on which test cases should be developed in order to provide enough test coverage for XR measurement gap cancelation.

RAN4 to discuss which test cases to develop based on the listed test cases in the following table:

Table 1 List of potential RRM test cases for measurement cancelation

| Gap type | Intra/inter/interRAT | Frequency range | DRX | Priority |
| --- | --- | --- | --- | --- |
| Type 1 gap per FR | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 1 gap per UE | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |
|  | Inter-frequency | FR1 | ON | High |
|  |  |  | OFF | High |
|  |  | FR2 | ON | High |
|  |  |  | OFF | High |
|  | Inter-RAT | FR1 (serving) | ON | Medium |
|  |  |  | OFF | High |
|  |  | FR2 (serving) | ON | Drop |
|  |  |  | OFF |  |
| Type 2 gaps per FR | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per UE | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |
|  | Inter-frequency | FR1 | ON | Drop |
|  |  |  | OFF | High |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | High |
|  | Inter-RAT | FR1 (serving) | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 (serving) | ON | Drop |
|  |  |  | OFF | Drop |
| LTM measurements with gaps | Inter-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |

The first thing to notice on that list is that we propose prioritization of per-UE test cases. Since per-FR depends on optional UE capability, if we define test cases for per-FR, those are not applicable for UEs not supporting per-FR.

RAN4 to focus on test cases for per-UE MGs.

Regarding DRX, RAN4 is defining core requirements for XR with and without DRX configured. Since DRX configuration is also increasing XR data latency, we believe that higher priority should be assigned to test cases without DRX.

Test cases without DRX should have higher priority than with DRX.

Test cases with DRX should be considered at least for Inter-frequency with Type 1 gaps. Inter-RAT with Type 1 per-UE MG can be considered as second priority.

Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG.

Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority.

When Type 2 gaps are considered, it is important to develop test cases which cover also the combination of concurrent gap dropping rules and XR measurement gap cancelation. However, the test coverage for Type 2 gaps does not need to be as broad as for Type 1 gaps. Therefore, we suggest focussing on non-DRX test cases for Type 2 gaps.

Focus on non-DRX test cases for Type 2 MGs.

For the test case of Type 2 MG, RAN4 needs to discuss the gap configuration. For Type 2 gaps, the main different aspect to be tested is the combination of concurrent gaps dropping rules together with measurement gap cancelation. For example, if the test case is considering 2 non-overlapping gap patterns, the expected behaviour would be similar to skipping of type 1 gap occasions. Therefore, we propose that 2 partially overlapping MG patterns are used for testing Type 2 gaps. One example can be to reuse the configuration of A.6.6.18.2.1 for the partially overlapping scenario, i.e. MGRP=40ms, offset=39ms for the MG with low priority, and MGRP=80ms and offset = 4ms for the MG with high priority.

For Type 2 MGs test cases, include 2 gaps with partial overlap.

For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e.

  - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

  - High priority MG pattern #2 with MGRP=80ms, offset=4ms

Measurement cancelation model

As part of the Rel-16 NR-U performance work, a model for CCA failure was developed for the test cases. This model was developed because NR-U core requirements included limitation on the number of samples during a measurement cycle over which the requirements don’t apply.

The discussion on the conditions for measurement gap cancelation is still open for the RRM core requirements. In case the limitation on the maximum number of skipped occasions is not specified by RAN4, then the measurement cancelation model can be very simplified in comparison to the NR-U model.

NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

Furthermore, since 2 gap patterns can be configured with Type 2 MGs, and the measurement delay is different for each MG pattern, it is proposed that the cancelation model can apply a different cancelation ratio for each gap pattern. In that manner, we can verify that the cancelation of one gap pattern is not influencing the measurement delay of the MOs related to the other gap pattern.

Measurement gap cancelation ratio is configured independently for each configured gap pattern.

For Type 2 MGs test cases, configure cancelation ratio as:

  - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

  - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

Tested metrics

XR measurement gap cancelation shares some aspects of NR_unlicensed test cases and Pre-MG test cases. Therefore, we can identify the following points to be tested.

Firstly, the ability of the UE to adapt the measurement delay in accordance with the number of canceled measurement gap occasions is tested. Secondly, it is tested if the UE can react to the DCI with skip indication within the reported Toffset capability (i.e. 5ms or 3ms). Thirdly, it is tested, if the UE can transmit or receive data on canceled gap occasions.

The following aspects should be evaluated in XR test cases:

  - Measurement delay extension due to skipping

  - Capability of canceling measurement gap within Toffset of 5ms or 3ms depending on UE capability

  - Reception of PDCCH/PDSCH during canceled measurement gap occasions

  - Transmission of PUCCH/PUSCH during canceled measurement gap occasions

Conclusion

The following Observations and Proposals were made:

- RAN4 to discuss which test cases to develop based on the listed test cases in the following table:

Table 1 List of potential RRM test cases for measurement cancelation

| Gap type | Intra/inter/interRAT | Frequency range | DRX | Priority |
| --- | --- | --- | --- | --- |
| Type 1 gap per FR | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 1 gap per UE | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |
|  | Inter-frequency | FR1 | ON | High |
|  |  |  | OFF | High |
|  |  | FR2 | ON | High |
|  |  |  | OFF | High |
|  | Inter-RAT | FR1 (serving) | ON | Medium |
|  |  |  | OFF | High |
|  |  | FR2 (serving) | ON | Drop |
|  |  |  | OFF |  |
| Type 2 gaps per FR | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per UE | Intra-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |
|  | Inter-frequency | FR1 | ON | Drop |
|  |  |  | OFF | High |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | High |
|  | Inter-RAT | FR1 (serving) | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 (serving) | ON | Drop |
|  |  |  | OFF | Drop |
| LTM measurements with gaps | Inter-frequency | FR1 | ON | Drop |
|  |  |  | OFF | Medium |
|  |  | FR2 | ON | Drop |
|  |  |  | OFF | Medium |

RAN4 to focus on test cases for per-UE MGs.

Test cases without DRX should have higher priority than with DRX.

Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG.

Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority.

Focus on non-DRX test cases for Type 2 MGs.

For Type 2 MGs test cases, include 2 gaps with partial overlap.

For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e.

  - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

  - High priority MG pattern #2 with MGRP=80ms, offset=4ms

- NR-U CCA model for RRM test cases includes limitation on the maximum number of samples with non-successful CCA.

Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

Measurement gap cancelation ratio is configured independently for each configured gap pattern.

For Type 2 MGs test cases, configure cancelation ratio as:

  - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

  - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

The following aspects should be evaluated in XR test cases:

  - Measurement delay extension due to skipping

  - Capability of canceling measurement gap within Toffset of 5ms or 3ms depending on UE capability

  - Reception of PDCCH/PDSCH during canceled measurement gap occasions

  - Transmission of PUCCH/PUSCH during canceled measurement gap occasions

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2508285, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2508312, LS on UE assistance information, Nokia

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2504972, Reply LS on UE assistance information, Nokia

- R4-2502700, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2500542, Topic summary for [113][228] NR_XR_Ph3, Moderator (Nokia)

- R4-2502241, General aspects for XR enhancements, Nokia

- R4-2502229, RRM requirements for XR enhancements, Qualcomm Incorporated

- R4-2502150, Discussion on XR ph3 RRM requirements, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, Nokia

- R4-2504508, Discussion on XR ph3 RRM requirements, Nokia

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, INC.

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, To: RAN1, cc: RAN2, Source: Qualcomm

- R4-2504972, Reply LS on UE assistance information, Nokia